3GPP TSG-RAN WG4 Meeting #90-bis                                               R4-19xxxxx

Xi’an, China, 8th to 12th Apr. 2019

Agenda Item:	9.3.2

Source: 	MediaTek Inc.

Title: 	TP for TR37.716-11-11 for DC_13A_n66A

Document for:	Approval

- 1. Introduction

The EN DC 13A_n66 has been requested in Release-16 basket WI in RAN4 meetings. The contribution is to provide TP for TR37.716-11-11 for the combination.

Revision 1 corrects UE co-existence error in section 6.1.x.4 highlighted below

- 2. Discussion

TP for TR37.716-11-11

- 6.1.x	DC_13A_n66

6.1.x.1	Operating bands for DC

Table 6.1.x.1-1: DC band combination of 1 LTE band + 1 NR band

| EN-DC band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_13_n66 | 13 | n66 |  |

#### 6.1.x.2	Configuration for DC

Table 6.1.x.2-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_13A_n66A | DC_13A_n66A | 13A | n66A |

#### 6.1.x.3	Maximum output power for DC

Table 6.1.x.3-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_13A_n66A | 23 | +2/-3 |

#### 6.1.x.4	Spurious emission band UE co-existence for DC

Table 6.1.x.4-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_13_n66 | Bands 2, 4, 5, 7, 10, 12, 13, 17, 25, 26, 27, 29, 41, 48, 49, 50, 51, 53, 66, 70, 71, 74, 85 and | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  |  | 769 | - | 775 | -35 | 0.00625 | 5 |
|  |  | 799 | - | 805 | -35 | 0.00625 | X, 5 |
|  | Bands 24, 30, 46, | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th or 5th harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, 5 for the 2nd, 3rd, 4th or 5th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 5: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE X: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD |  |  |  |  |  |  |  |

# 6.1.x.5	MSD analysis for DC

Table 6.1.x.5-1 lists Band 13 +Band n66 2UL DC up to 7th order harmonics and 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.1.x.5-1: Band 13 and Band n66 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 777 | 787 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits (MHz) | 1554 | 1574 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2331 | 2361 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 3108 | 3148 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3885 | 3935 | 8550 | 8900 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6* fy_low | 6* fy_high |
| 6th harmonics frequency limits (MHz) | 4662 | 4722 | 10260 | 10680 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 5439 | 5509 | 11970 | 12460 |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1003 | 923 | 2487 | 2567 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 226 | 136 | 2633 | 797 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3264 | 3354 | 4197 | 4347 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 2006 | 1846 | 4974 | 5134 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 551 | 651 | 4343 | 4563 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 4041 | 4141 | 5907 | 6127 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 6343 | 6053 | 1438 | 1328 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7617 | 7907 | 4818 | 4928 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 3786 | 3556 | 1059 | 1229 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6684 | 6914 | 5751 | 5921 |

Based on Table 6.1.x.5-1;

-	2nd order harmonic may fall into Rx frequencies bands 24, 42, n77 and n78

-	3rd order harmonic may fall into Rx frequencies of bands 30, 40, 46, 52

-	5th order harmonic may fall into Rx frequency of n77.

-	6th order harmonic may fall into Rx frequency of n79

-	7th order harmonic may fall into Rx frequency of band 46

-	2nd order IMD may also fall into Rx frequencies of bands 7, 41

-	3rd order IMD may also fall into Rx frequencies of bands 7, 41, n77, n78 and n79

-	4th order IMD may also fall into Rx frequencies of bands 2, 3, 9, 25, 33, 35, 36, 37, 39, 46, 47, 70, 71, n77 and n79

-	5th order IMD may also fall into Rx frequencies of bands 42, 43, 46, 47, 48, 49, 50, 51, ,75, 76, n77, n78, n79

When 2UL inter-band DC UE is operating with other systems such as WiFi, Bluetooth and GNSS system, the harmonics and intermodulation products can have impact on these systems. Table 6.1.x.5-2 lists if up to 7th order harmonics and IMD up to 5th order falls into one of these receiving bands.

Table 6.1.x.5-2: 2UL Band 13 +Band n66 harmonic and IMD for ISM and GNSS bands

| Victim Systems | Frequency range [MHz] |  |  | Impact | Regions | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| COMPASS (Beidou) | 1559 | - | 1591 | Yes |  | Harmonic2 |
| Galileo | 1559 | - | 1591 | Yes |  | Harmonic2 |
| GLONASS | 1591 | - | 1610 | No |  |  |
| GPS | 1563 | - | 1587 | Yes |  | Harmonic2 |
| ISM band (2.4GHz) | 2400 | - | 2483.5 | No | US/Europe |  |
|  | 2400 | - | 2494 | Yes | Asia | IMD2 |
| ISM band (5GHz) | 5150 | - | 5925 | Yes | US | Harmonic3, 7 IMD4, 5 |
|  | 5150 | - | 5350 | Yes | Europe | Harmonic3 |
|  | 5470 | - | 5725 | Yes |  | Harmonic7 |
|  | 5150 | - | 5825 | Yes | Asia | Harmonic3, 7 IMD5 |

### 6.1.x.6	∆TIB and ∆RIB values

For DC_13_n66, the TIB,c and RIB,c values are reused from the LTE combination CA_13-66, and are given in the tables below.

# Table 6.1.x.6-1: ΔTIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13_n66 | 13 | 0.3 |
|  | n66 | 0.3 |

# Table 6.1.x.6-2: ΔRIB,c

| E-UTRA and NR DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_13_n66 | 13 | 0 |
|  | n66 | 0 |