3GPP TSG-RAN4 Meeting #92	R4-1808043

# Ljubliana, Slovenia, 26th – 30th August 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.105 | CR | 0153 | rev | - | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RX spurious emissions applicability range for SR E-UTRA BS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Core |  |  |  |  |  | Date: |  |  | Rel-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | 2019-08-26 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | It was previously agreed that for AAS basestations, the exclusion around the operating band for spurious emissions should be extended for bands >100MHz. This is implemented correctly for NR type 1-H and 1-O and is implemented correctly for MSR AAS, but is not implemented for single RAT E-UTRA AAS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For the conducted requirement, the exclusion frequency range is clarified. For the OTA requirement, the exclusion is changed from 10MHz to ΔfOBUE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Single RAT E-UTRA AAS spec not consistent with MSR AAS or NR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.4, 10.7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR 37.145-1, 37-145-2 CR 0174, 0145 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 7.6.4	Minimum requirement for single RAT E-UTRA operation

The single RAT E-UTRA wide area, medium range area and local area RX spurious emissions basic limits are the same as those specified in 3GPP TS 36.104 [8], subclause 7.7.1 with the exception that the frequency range around the band edge within which the requirement is not applicable is extended from 10MHz to ΔfOBUE.

The RX spurious emissions requirements for a single RAT E-UTRA AAS BS are that for each applicable basic limit specified in 3GPP TS 36.104 [4] for each TAB connector RX min cell group, the power sum of emissions at the TAB connectors of the TAB connector RX min cell group shall not exceed an AAS limit specified as the basic limit + X, where X = 10log10(NRXU,countedpercell), unless stated differently in regional regulation.

NOTE:	Conformance to the AAS BS receiver spurious emissions requirement can be demonstrated by meeting at least one of the following criteria as determined by the manufacturer:

1)	The sum of the emissions power measured on each TAB connector in the TAB connector RX min cell group shall be less than or equal to the AAS BS limit as defined above for the respective frequency span.

Or

2)	The spurious emission power at each TAB connector shall be less than or equal to the AAS BS limit as defined above for the respective frequency span, scaled by -10log10(n), where n is the number of TAB connectors in the TAB connector RX min cell group.

### 10.7.4	Minimum requirement for single RAT E-UTRA operation

The TRP of any spurious emission shall not exceed the levels in table 10.7.4-1:

Table 10.7.4-1: General spurious emission minimum requirement

| Frequency range | Maximum level (Note 2) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 30MHz - 1 GHz | -57 + X dBm | 100 kHz |  |
| 1 GHz - 12.75 GHz | -47 + X dBm | 1 MHz |  |
| 12.75 GHz - 5th harmonic of the upper frequency edge of the UL operating band in GHz | -47 + X dBm | 1 MHz | Applies only for Bands 22, 42, 43, 48. |
| NOTE 1: The frequency range between 2.5 * channel bandwidth below the first carrier frequency and 2.5 * channel bandwidth above the last carrier frequency transmitted by the AAS BS may be excluded from the requirement. However, frequencies that are more than ΔfOBUE below the lowest frequency of any of the AAS BS supported downlink operating band or more than ΔfOBUE above the highest frequency of any of the AAS BS supported downlink operating band shall not be excluded from the requirement. For a multiband RIB, the exclusion applies for all supported operating bands. NOTE 2: X = 9 dB, unless stated differently in regional regulation. |  |  |  |

In addition to the requirements in table 10.7.4-1, the power of any spurious emission shall not exceed the levels specified for Protection of the E-UTRA FDD BS receiver of own or different BS in subclause 9.7.6.4.2 and for Co-existence with other systems in the same geographical area in subclause 9.7.6.4.3. In addition, the co-existence requirements for co-located base stations specified in subclause 9.7.6.4.4 may also be applied.