3GPP TSG-RAN Working Group 4 (Radio) meeting #93	R4-1915148

# Reno, USA, 18th – 22nd Nov, 2019

Source:	LG Electronics

Title:	On NRU UE Output power

Agenda item:	9.1.2.1	Transmitter characteristics, [NR_unlic-Core]

Document for:	Discussion

# Introduction

RAN4 has been discussing on UE output power and transmitter linearity requirements for NRU. The discussion in the previous meeting is captured into the AdHoc minutes in [1] and in way forward in [2]. In this document we present Output power and ACLR measurements results for different NR-U waveforms using a 5GHz WiFi PA.

# Discussion

In order to understand the performance of WiFi PAs currently used in mobile handset one such PA was selected and measured with NRU waveforms. The used waveforms were

NRU 15kHz SCS 106 PRBs

- CP-OFDMA modulated

- DFTS-OFDMA modulated

NRU 15kHz SCS 11 PRBs interlaced [ 0, 10, 20, …100 ]

- CP-OFDMA modulated

- DFTS-OFDMA modulated

The output power is measured from the output of the PA and no post-pa losses have been taken into account.

## NRU 15kHz SCS 106 PRBs

The measurement results for CP-OFDMA modulated 106 PRBs wide signal is shown in Figure 1 below.

Observation #1: +23dBm output power FROM PA can be achieved with ACLR of 27dB using CP-OFDMA modulated 106PRBs wide signal. Power difference between low and typical Vcc level is about 1dBm.

Figure 1. CP-OFDMA 106 PRBs

The measurement results for DFTS-OFDMA modulated 106 PRBs wide signal is shown in Figure 2 below.

Observation #2: +25dBm output power FROM PA can be achieved with ACLR of 27dB using DFTS-OFDMA modulated 106PRBs wide signal. Power difference between low and typical Vcc level is about 1dBm.

Figure 2. DFTS-OFDMA 106 PRBs

## NRU 15kHz SCS 11 PRBs interlaced

The measurement results for CP-OFDMA modulated 11 PRBs interlaced waveform is shown in Figure 3 below.

Observation #3: +23dBm output power FROM PA can be achieved with ACLR of 27dB using CP-OFDMA modulated 11PRBs interlaced waveform. Power difference between low and typical Vcc level is about 1dBm.

Figure 3. CP-OFDMA 11 PRBs interlaced waveform

The measurement results for DFTS-OFDMA modulated 11 PRBs interlaced waveform is shown in Figure 4 below.

Observation #4: +24.5dBm output power FROM PA can be achieved with ACLR of 27dB using DFTS-OFDMA modulated 11PRBs interlaced waveform. Power difference between low and typical Vcc level is about 1dBm.

Figure 4. DFTS-OFDMA 11 PRBs interlaced waveform

# Conclusion

In this contribution measurement results for typical WiFi PA are shown with different NRU waveforms. The used waveforms were.

NRU 15kHz SCS 106 PRBs

- CP-OFDMA modulated

- DFTS-OFDMA modulated

NRU 15kHz SCS 11 PRBs interlaced [ 0, 10, 20, …100 ]

- CP-OFDMA modulated

- DFTS-OFDMA modulated

The output power was measured FROM OUTPUT OF THE PA and no post-PA losses were taken into account in the measurements.

Based on the measurements done four observations were made that are collected into the table below:

Table 1. Achievable output power with ACLR of 27dB for different waveforms

|  | CP-OFDMA 106PRB | CP-OFDMA 11PRBs interlaced | DFTS-OFDMA 106PRBs | DFTS-OFDMA 11PRBs interlaced |
| --- | --- | --- | --- | --- |
| Pout @ ACLR= 27dB | 23dBm | 23dBm | 25dBm | 24.5dBm |
| Room for post-PA-losses for PC5 | 3dB | 3dB | 5dB | 4.5dB |

When the measured typical output power levels are compared to nominal output power level of +20dBm /PC5 it can be concluded that there is 3 or 4.5-5dB remaining for post-PA-losses depending on the used waveform. In some cases 3dB post-PA-losses may not be enough for real implementations, while 4.5-5dB should be enough cover wide-range of different implementations.

Conclusion #1: Based on this measurements done we can conclude that it seems possible to achieve +20dBm/PC5 output power with typical WiFi PA at least with DFTS-OFDMA modulated waveforms. It must also be noted that testing was carried out with one  PA model and one randomly selected sample only.

# References

[1]		R4-1913058	“Meeting minutes for NR-U Ad-hoc”, Qualcomm Incorporated, RAN4 #92bis, October 2019

[2]		R4-1913059	“WF on the punctured channel emission requirements”, Charter, RAN4 #92bis, October 2019