3GPP TSG-RAN WG4 #94-e	R4-2000077

Online, 24 Feb. – 6 Mar., 2020

Source: 		Qualcomm Incorporated

Title: 			FR2 Beam Correspondence using SSB only

Agenda Item:	8.14.1.2

Work Item:		NR_RF_FR2_req_enh

Release:		16

Document for:	Approval

# 1.	Introduction

In [1] agreed in the last RAN4 #93 meeting, open issues for SSB-only based BC requirements are captured in the table as below.

- Whether or not to configure P3 CSI-RS

- Minimum SNR for TRS

- Minimum SNR for SSB

In this contribution, we propose specific values for the three open issues.

# 2. 	Discussion

- Whether or not to configure P3 CSI-RS

For Rel-16 eBC requirements, we will define two different eBC modes based on a source of spatial reference signal, namely;

- SSB-based eBC (the UE should be able to rely on SSB alone for Beam Refinement)

- CSI-RS-based eBC (the UE should be able to take advantage of configured CSI-RS for Beam Refinement)

Based on agreement [1], P3 CSI-RS is going to be used as a source of spatial reference signal for CSI-RS-based eBC. Therefore, it is obvious that SSB-based eBC requirement should not rely on P3 CSI-RS.

Proposal 1: For SSB-based eBC, P3 CSI-RS is not configured.

- Minimum SNR for TRS

Note that TRS is an essential CSI-RS for time/frequency tracking and channel estimation. And the purpose of configuring TRS in SSB-based eBC is to provide PDCCH/PDSCH DMRS with qcl-typeA and qcl-typeD sources so that the UE will not encounter any potential issue about PDCCH/PDSCH detection performance which is not of interest to SSB-based eBC test. Hence, TRS SNR does not need to be unnecessarily lowered to less than SSB SNR.

Proposal 2: For SSB-based eBC, minimum TR SNR is equal to or greater than minimum SSB SNR.

- Minimum SNR for SSB

There are two alternatives, 6dB and 13dB, for SSB SNR. According to [2], 13dB was proposed because UE may have to use wide beam when it receives SSB to perform cell/SSB search and/or measurement, which results in 7dB SNR loss with respect to narrow beam. Given that the BC performance measurement will be conducted after a sufficient amount of time so that beam refinement can be settled, we do believe the UE should be able to perform both tasks based on SSB alternately. It should be also noted that TS38.133 provides further confirmation of insensitivity to RS type in this context, by informing that RRM/RLM (e.g. L1-RSRP requirements from 10.1.20.1 to 10.1.20.2 and RLM requirements from A.7.5.1.1 to A.7.5.1.8) requirements and side conditions are identical for both, SSB and CSI-RS.

Proposal 3: PSD of reference signal (RS) used by the UE to achieve beam correspondence shall be the same, regardless of RS type (SSB or CSI-RS)

# 3.	Conclusion

There is both, motivation and feasibility to use SSB as the sole reference signal for the UE to achieve BC. Using rel-15 agreements on BC test side-conditions as a starting point, we propose:

Proposal 1: For SSB-based eBC, P3 CSI-RS is not configured.

Proposal 2: For SSB-based eBC, minimum TR SNR is equal to or greater than minimum SSB SNR.

Proposal 3: PSD of reference signal (RS) used by the UE to achieve beam correspondence shall be the same, regardless of RS type (SSB or CSI-RS)

# 4. 	References

[1]	R4-1916172, “WF on side conditions of SSB based and CSI-RS based beam correspondence tests,” Apple Inc., RAN4#93, Reno, USA, Nov. 2019

[2]	R4-1915371, “On beam correspondence for Rel-16,” Huawei, HiSilicon RAN4#93, Reno, USA, Nov. 2019