3GPP TSG-RAN WG4 Meeting #94-e		R4-2000359

E-meeting, 24 February – 6 March, 2020

Agenda item:	8.18.1.1

Source:	Intel Corporation

Title:	Discussion on NR CA UE demodulation requirements

Document for:	Discussion

# Introduction

In RAN4 #93 WF on PDSCH CA normal demodulation requirements was approved [1]. In this paper we provide view on remaining open issues for NR CA PDSCH normal demodulation requirements.

# Discussion

FRC for FR2 tests

In the previous RAN4 meeting the following agreements were reached on FRC for CA requirements definition:

- Rank and MCS for FR2 - Option 1: rank 2 and MCS 10 - Option 2: rank 1 and MCS 13 - Down-select to one option

In RAN4 #93 meeting, the following information on maximum testable SNR for CA scenarios was provided by TE vendors

Table 1. Feasible FR2 SNR levels from [2]

| Operating band | SNR (dB) / Aggregated Channel bandwidth |  |  |
| --- | --- | --- | --- |
|  | 200 MHz | 400 MHz | 800 MHz |
| n257 | 21.8 | 18.8 | 15.8 |
| n258 | 21.8 | 18.8 | 15.8 |
| n260 | 16 | 13 | 10 |
| n261 | 21.8 | 18.8 | 15.8 |

In companion paper [2] we provided simulation results for Rank 1 MCS 13 and Rank 2 MCS 10 for different CBW. From this results we can observe that SNR operating point for alignment results is ~4.8 dB for Rank 1 MCS 13 and ~8.0 dB for Rank 2 MCS10. Taking into account RX impairments and requirement margin 0.5 dB, we get the following approximate SNR for requirement definition: 7.4 dB for Rank 1 MCS 13 and 10.5 dB for Rank 2 MCS10.

Observation #1: Based on link level analysis

- SNR operating point for Rank 1 MCS13 is equal to 7.4 dB.

- SNR operating point for Rank 2 MCS10 is equal to 10.5 dB.

From these results we can conclude that:

- Rank 1 MCS13 can be tested for scenarios with aggregated channel bandwidth up to ~5500 MHz for bands n257,258 and 261 and up to ~1400 MHz for band n260.

- Rank 2 MCS10 can be tested for scenarios with aggregated channel bandwidth up to ~2700 MHz for bands n257,258 and 261 and up to ~700 MHz for band n260.

Proposal 1:	Use Rank 1 MCS13 for FR2 NR Normal CA requirements.

Requirements for TDD-FDD CA and TDD-TDD CA with different SCSs

In the previous RAN4 meeting the following agreements were reached on definition of requirement for TDD-FDD CA and TDD-TDD CA with different SCSs:

- FDD 15 kHz + TDD 30 kHz CA and FDD 15 kHz + TDD 15 kHz CA - FFS whether requirements will be defined for FDD PCell and TDD Pcell - If Pcell in both carriers are supported, the performance for FDD Pcell and FDD Scell is similar and the performance for TDD Pcell and TDD Scell is similar, FFS which one is used for the tests. - HARQ process number for FDD 15 kHz + TDD 30 kHz CA - Pcell: 4 for FDD and [8 or 10] for TDD - Scell: FFS - HARQ process number for FDD 15 kHz + TDD 15 kHz CA: FFS - TDD 15 kHz + TDD 30 kHz CA - FFS whether requirements will be defined for 15 kHz PCell and 30 kHz Pcell - If Pcell in both carriers are supported, the performance for 15kHz Pcell and 15 kHz Scell is similar and the performance for 30 kHz Pcell and 30 kHz Scell is similar, FFS which one is used for the tests. - HARQ process number: FFS - Performance requirements: - Further discuss whether single carrier requirements from FDD and TDD CA with the same SCS can be reused. - Companies are encouraged to bring performance analysis in the next meeting

As for HARQ process number for each scenario, we suggest to reuse single carrier assumptions for PCell. Detailed analysis of HARQ timing for SCell in case A/N feedback is applied on PCell is provided in Annex A. Based on this analysis we can observe that the minimum required number of HARQ processes to meet requirement on UE PDSCH processing time is equal to:

- PCell FDD 15kHz + SCell TDD 15kHz: 4

- PCell TDD 15kHz + SCell FDD 15kHz: 8

- PCell FDD 15kHz + SCell TDD 30kHz: 8

- PCell TDD 30kHz + SCell FDD 15kHz: 8

- PCell TDD 15kHz + SCell TDD 30kHz: 12

- PCell TDD 30kHz + SCell TDD 15kHz: 6

From our analysis, we can observe that in case minimum required HARQ process value will be configured for test then for some scenarios initial transmission and retransmission will be scheduled on different type of TDD slots. Therefore, we suggest to increase the number of HARQ process (in comparison to minimum required value) for requirement definition for SCell configuration for the following scenarios:

- PCell TDD 15kHz + SCell TDD 30kHz: 16

- PCell TDD 30kHz + SCell TDD 15kHz: 8

In Table 3 we summarize our proposals of HARQ process configuration for TDD-FDD CA and TDD-TDD CA with different SCSs.

Table 3. Summary of views on HARQ process configuration

| Scenario | HARQ process for PCell | HARQ process for SCell |
| --- | --- | --- |
| PCell FDD 15kHz + SCell TDD 15kHz | 4 | 4 |
| PCell TDD 15kHz + SCell FDD 15kHz | 8 | 8 |
| PCell FDD 15kHz + SCell TDD 30kHz | 4 | 8 |
| PCell TDD 30kHz + SCell FDD 15kHz | 8 | 8 |
| PCell TDD 15kHz + SCell TDD 30kHz | 8 | 16 |
| PCell TDD 30kHz + SCell TDD 15kHz | 8 | 8 |

Proposal 2:	Consider the following HARQ process configuration for TDD-FDD CA and TDD-TDD CA with different SCSs:

- PCell FDD 15kHz + SCell TDD 15kHz: PCell – 4, SCell – 4

- PCell TDD 15kHz + SCell FDD 15kHz: PCell – 8, SCell – 8

- PCell FDD 15kHz + SCell TDD 30kHz: PCell – 4, SCell – 8

- PCell TDD 30kHz + SCell FDD 15kHz: PCell – 8, SCell – 8

- PCell TDD 15kHz + SCell TDD 30kHz: PCell – 8, SCell – 16

- PCell TDD 30kHz + SCell TDD 15kHz: PCell – 8, SCell – 8

From Table 3 we can observe that FDD CC with 15 kHz can be configured with 4 or 8 HARQ processes, TDD CC with 15 kHz can be configured with 4 or 8 HARQ processes and TDD CC with 30 kHz can be configured with 8 or 16 HARQ processes. Therefore, the next question is whether same requirements can be used for scenarios with different number of HARQ processes. In Figure 2 we provide the performance analysis of impact of HARQ processes number on PDSCH performance for selected scenarios.

| FDD 15 kHz, 10 MHz, 2Rx | FDD 15 kHz, 10 MHz, 4Rx |
| --- | --- |
| TDD 15 kHz, 10 MHz, 2Rx | TDD 15 kHz, 10 MHz, 4Rx |
| TDD 30 kHz, 40 MHz, 2Rx | TDD 30 kHz, 40 MHz, 4Rx |
| Figure 2. Impact of HARQ processes number on PDSCH performance. |  |

From these results we can observe that performance is rather same for scenarios with different number of HARQ processes. Therefore, we can reuse single carrier FDD and TDD requirements for FDD-TDD CA and TDD CA with different SCSs. Also, we can conclude that requirements will be same in case PCell is configured on FDD CC or TDD CC for FDD-TDD CA or in case PCell is configured on 15 kHz CC on 30 kHz CC for TDD-TDD CA with different SCSs. Therefore, requirements can be defined only for one scenarios PCell FDD or PCell TDD for FDD-TDD CA and PCell 15 kHz or PCell 30 kHz for TDD-TDD CA with different SCSs. We suggest to define the requirements for scenarios with less RTT: PCell FDD 15kHz + SCell TDD 15kHz, PCell FDD 15kHz + SCell TDD 30kHz and PCell TDD 30kHz + SCell TDD 15kHz

Proposal 3:	Reuse single carrier FDD and TDD requirements for FDD-TDD CA and TDD CA with different SCSs and define requirements for the following scenarios: PCell FDD 15kHz + SCell TDD 15kHz, PCell FDD 15kHz + SCell TDD 30kHz and PCell TDD 30kHz + SCell TDD 15kHz.

Test applicability rule

In the previous RAN4 meeting the following agreements were reached on NR CA applicability rules:

- Test applicability for different CA duplex mode - Test all the supported CA duplex mode - Categorizing of CA capabilities - Option 1: Define different capabilities for intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands. - Option 2: Define different capabilities for intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA - Other options are not precluded. - Companies to bring proposals on the demod spec structure for CA, with the motivation to minimize future maintenance. - Test of different CA capabilities - Option 1: Test intra-band contiguous CA, Intra-band non-contiguous CA and Inter-band CA with the largest number of bands - Option 2: Test intra-band contiguous CA, Intra-band non-contiguous CA and Inter-band CA with the largest aggregated CBW - Option 3: Test all the supported CA capabilities, including intra-band contiguous CA, intra-band non-contiguous CA and inter-band CA with different numbers of bands - Option 4: Any one of the supported CA capabilities - Other options are not precluded. - Selection of CA configuration(s) and CBW combination - Further discuss by taking into account: - The supportedSubCarrierSpacingDL, maxNumberMIMO-LayersPDSCH and supportedModulationOrderDL are reported for each CC and scalingFactor are reported per band for FR1 and FR2. - The testable SNR for FR2.

- Categorizing of CA capabilities

At current stage, the following CA capabilities are defined for LTE Normal CA requirements for scenarios with different number of CCs: Intra-band contiguous CA, Intra-band non-contiguous CA and Inter-band CA with different number of bands. Such categorizing of CA capabilities is aligned with RF definition. We suggest to consider same approach as for LTE and align categorizing of NR CA capabilities with RF specification. Same time, we suggest to consider different from LTE approach for CA capability definition in TS, to avoid regular maintenance of these requirements once new CA configuration are defined in RF specifications (38.101-1 and 38.101-2). One of the possible solution is to add references to sections 5.2A.1 and 5.5A.1 for FR1 intra-band continues CA capability, which contain all required information about supported CA scenarios. Same approach can be used for other CA capabilities.

- Testing of different CA capabilities

Depending on scenarios, LTE UE is tested for each or any one supported CA capability. We think that it is rather important to defined CA requirements with testing on all CA scenarios, i.e. contiguous Intra-band, non-contiguous Intra-band and Inter-band. Same time, we think that it is redundant to test UE for multiple Inter-band CA scenarios with different number of bands and it can be sufficient to test only for Inter-band CA scenarios with the highest number of bands. Such approach will allow to test UE which support CA with X band and X CCs for full inter-band CA scenario. Same time, testing for CA scenario with Y band and X CCs (where Y < X) leads to testing to mixed CA scenarios (intra-band + inter-band). In this case, dedicated testing of intra-band looks rather redundant. Therefore, we suggest to test contiguous Intra-band, non-contiguous Intra-band and Inter-band with maximum number of bands to have rather good balance between test coverage and number of test.

- Selection of CA configuration for testing

In the previous RAN4 meeting [1] it was agreed that CA configuration(s) and CBW for testing should be selected taking into account UE capability signalling on supported SCS, maximum supported number of MIMO layers, maximum supported modulation order, scaling factor and testable SNR limitations in FR2. Therefore, as the first step, we suggest to select CA configuration with maximum number of CCs where UE supports SCS used for requirements definition.

In the next steps we need to take into account that requirements are defined for fixed FRC. First, we need to ensure that data rate during the test does not exceed maximum supported data rate (calculated based on equation from TS 38.306 [3] Section 4.1.2). Second, it is important to select CA configuration which contains CCs with support of number of MIMO layer specified for requirements.

After selection of CA configurations where UE supports required SCS, Number of MIMO layers and Data Rate, further downselection of CA configurations can be done based data rate or aggregated channel bandwidth calculations.

Based on suggestions above, we propose the following methodology for selection of CA configuration for testing for each CA capability:

  - Step 1: Select CA configurations with maximum number of CCs, on which UE capability field supportedSubCarrierSpacingDL is equal to SCSreq (SCS used for requirements definition), among all supported CA configurations

  - Step 2: Select CA configurations with maximum number of CCs, on which UE capability field maxNumberMIMO-LayersPDSCH is higher or equal to νLayersreq (number of MIMO layers for requirements definition), among all the selected CA configurations from Step 1

  - Step 3: Select any one of CA configurations, which contain CBW combinations with the largest data rate not exceeding DataRatereq, among all the selected CA configurations from Step 2.

or

For FR2 CA testing, in the first step it is rather beneficial to select CA configurations which contain CBW combinations with maximum achievable SNR (SNRTEmax) higher or equal to SNR used for requirements definition (SNRreq). Such approach allows to avoid selection of CA configuration and CBW combination which can not be tested due to limitation at TE side.

Proposal 4:	Align categorizing of CA capabilities for NR Normal CA requirements with RF specifications. Use references to sections with CA configurations descriptions in RF specifications (for example, 5.2A and 5.5A) for definition of CA capabilities to avoid regular maintenance of TS 38.101-4.

Proposal 5:	Consider the following CA capabilities for NR Normal CA testing: Intra-band contiguous CA, Intra-band non-contiguous CA and Inter-band CA with the largest number of bands

Proposal 6:	Use the following approach for selection of CA configuration for NR FR1 Normal CA testing:

- Step 1: Select CA configurations with maximum number of CCs, on which UE capability field supportedSubCarrierSpacingDL is equal to SCSreq, among all supported CA configurations

- Step 2: Select CA configurations with maximum number of CCs, on which UE capability field maxNumberMIMO-LayersPDSCH is higher or equal to νLayersreq, among all the selected CA configurations from Step 1

- Step 3: Select any one of CA configurations, which contain CBW combination with the largest data rate not exceeding DataRatereq, among all the selected CA configurations from Step 2.

Proposal 7:	Use the following approach for selection of CA configuration for NR FR2 Normal CA testing:

- Step 1: Select CA configurations, which contain CBW combinations with SNRTEmax higher or equal to SNRreq, among all supported CA configurations

- Step 2: Select CA configurations with maximum number of CCs, on which UE capability field supportedSubCarrierSpacingDL is equal to SCSreq, among all the selected CA configurations from Step 1

- Step 3: Select CA configurations with maximum number of CCs, on which UE capability field maxNumberMIMO-LayersPDSCH is higher or equal to νLayersreq, among all the selected CA configurations from Step 2

- Step 4: Select any one of CA configurations, which contain CBW combination with the largest data rate not exceeding DataRatereq and aggregated bandwidth with SNRTEmax higher or equal to SNRreq, among all the selected CA configurations from Step 3.

# Conclusion

In this paper we provided view on test methodology and simulation assumptions for NR CA PDSCH normal demodulation requirements and made the following proposals:

Proposal 1:	Use Rank 1 MCS13 for FR2 NR Normal CA requirements.

Proposal 2:	Consider the following HARQ process configuration for TDD-FDD CA and TDD-TDD CA with different SCSs:

- PCell FDD 15kHz + SCell TDD 15kHz: PCell – 4, SCell – 4

- PCell TDD 15kHz + SCell FDD 15kHz: PCell – 8, SCell – 8

- PCell FDD 15kHz + SCell TDD 30kHz: PCell – 4, SCell – 8

- PCell TDD 30kHz + SCell FDD 15kHz: PCell – 8, SCell – 8

- PCell TDD 15kHz + SCell TDD 30kHz: PCell – 8, SCell – 16

- PCell TDD 30kHz + SCell TDD 15kHz: PCell – 8, SCell – 8

Proposal 3:	Reuse single carrier FDD and TDD requirements for FDD-TDD CA and TDD CA with different SCSs and define requirements for the following scenarios: PCell FDD 15kHz + SCell TDD 15kHz, PCell FDD 15kHz + SCell TDD 30kHz and PCell TDD 30kHz + SCell TDD 15kHz.

Proposal 4:	Align categorizing of CA capabilities for NR Normal CA requirements with RF specifications. Use references to sections with CA configurations descriptions in RF specifications (for example, 5.2A and 5.5A) for definition of CA capabilities to avoid regular maintenance of TS 38.101-4.

Proposal 5:	Consider the following CA capabilities for NR Normal CA testing: Intra-band contiguous CA, Intra-band non-contiguous CA and Inter-band CA with the largest number of bands

Proposal 6:	Use the following approach for selection of CA configuration for NR FR1 Normal CA testing:

- Step 1: Select CA configurations with maximum number of CCs, on which UE capability field supportedSubCarrierSpacingDL is equal to SCSreq, among all supported CA configurations

- Step 2: Select CA configurations with maximum number of CCs, on which UE capability field maxNumberMIMO-LayersPDSCH is higher or equal to νLayersreq, among all the selected CA configurations from Step 1

- Step 3: Select any one of CA configurations, which contain CBW combination with the largest data rate not exceeding DataRatereq, among all the selected CA configurations from Step 2.

Proposal 7:	Use the following approach for selection of CA configuration for NR FR2 Normal CA testing:

- Step 1: Select CA configurations, which contain CBW combinations with SNRTEmax higher or equal to SNRreq, among all supported CA configurations

- Step 2: Select CA configurations with maximum number of CCs, on which UE capability field supportedSubCarrierSpacingDL is equal to SCSreq, among all the selected CA configurations from Step 1

- Step 3: Select CA configurations with maximum number of CCs, on which UE capability field maxNumberMIMO-LayersPDSCH is higher or equal to νLayersreq, among all the selected CA configurations from Step 2

- Step 4: Select any one of CA configurations, which contain CBW combination with the largest data rate not exceeding DataRatereq and aggregated bandwidth with SNRTEmax higher or equal to SNRreq, among all the selected CA configurations from Step 3.

# References

- R4-1915861 “Way forward on PDSCH CA normal demodulation requirements”, Intel, RAN4 #93, November 2019.

- R4-2000360 “Simulation results for Normal CA requirements”, Intel, RAN4 #94, February 2020

- TS 38.101-4 “User Equipment (UE) radio transmission and reception; Part 4: Performance requirements”

- TS 38.306 “User Equipment (UE) radio access capabilities”.

# Annex A

In this section we provide illustrations of SCell HARQ timing for the following scenarios:

- PCell FDD 15kHz + SCell TDD 15kHz (Figure 2)

- PCell TDD 15kHz + SCell FDD 15kHz (Figure 3)

- PCell FDD 15kHz + SCell TDD 30kHz (Figure 4)

- PCell TDD 30kHz + SCell FDD 15kHz (Figure 5)

- PCell TDD 15kHz + SCell TDD 30kHz (Figure 6)

- PCell TDD 30kHz + SCell TDD 15kHz (Figure 7)

Figure 3. SCell HARQ timing for PCell FDD 15kHz + SCell TDD 15kHz

Figure 4. SCell HARQ timing for PCell TDD 15kHz + SCell FDD 15kHz

Figure 5. SCell HARQ timing for PCell FDD 15kHz + SCell TDD 30kHz

Figure 6. SCell HARQ timing for PCell TDD 30kHz + SCell FDD 15kHz

Figure 7. SCell HARQ timing for PCell TDD 15kHz + SCell TDD 30kHz

Figure 8. SCell HARQ timing for PCell TDD 30kHz + SCell TDD 15kHz