3GPP TSG-RAN WG4 Meeting #94-e                                   R4-2000889

Online, 24h Feb. – 6th March, 2020

Agenda item:			8.5.5.4

Source:			Samsung

Title:	Discussion on RLM requirement for IAB-MT

Document for:		Discussion

1	Introduction

As part of the WI on Integrated Access and Backhaul for NR, the specification of RF and RRM core requirements for IAB nodes is already captured in the IAB WID:

- Specification of RF and RRM requirements [RAN4-led]: - … - Define RRM core requirements for both backhaul and access links of IAB node. - …

In the last few meetings, many have heatedly discussed on RRM requirement needs for IAB supported by RAN4. The downlink radio link quality is monitored by a UE for the purpose of indicating out-of-sync/in-sync status to higher layers. Likewise, the same capability should be possessed of IAB-MT and corresponding requirement be defined, which was also proposed by companies. In RP#86 meeting, a way-forward regarding to IAB-MT RRM requirement (RP-193199 [1]) was approved, which is captured as following:

- RAN4 will introduce RLM and BFD/BFR requirements for the MT targeting certain scenarios classes depending on RAN4 definition - Requirements will be defined for scenarios not targeting macro type of deployments. No requirements will be defined for scenarios targeting macro type of deployments: - If multiple MT classes (e.g. macro type, pico type deployments) are defined, the requirements should be defined only for the MT classes not targeting macro type of deployment. - If a single MT class is defined, the requirements should be defined for the DU class not targeting DU macro type of deployment. - Rel.15 UE requirement framework for RLM and BFD/BFR will be taken as baseline

In this contribution, we would like to discuss on how to define the radio link monitoring (RLM) requirement for IAB-MT on the basis of UE requirement.

- RLM requirement for IAB

IAB serves as a novel technique to support for wireless backhaul and relay links enabling flexible and is deployed densely in NR cell without the need for densifying the wired transport network proportionately. Unlike FR1 radio link, radio on higher frequency (i.e. mmWave) can hardly propagate by cluster reflection. Therefore, FR2 radio link would be better beneficial from IAB technique since most of time the radio power will concentrate on the line-of-sight (LoS) path on FR2. It can be imaged that FR2 is the main use case for IAB network and these wireless relay will be most likely used for FR2 first in commercialization. Further, considering more complex situation in FR2 than in FR1, in Rel-16, we can first discussion on the IAB RLM requirement on FR2 to guarantee its applicability then extend it to FR1.

Proposal 1: For IAB radio link monitoring, the requirement for FR2 could be first discussed and then extended to FR1.

On the other hand, the existing RLM requirement for UE in TS 38.133 [2] requires the UE to monitor the quality of multiple RLM-RS periodically to detect IS and OOS detection for the sake of re-establishment of the connection upon RLF. Considering the fixed IAB in Rel-16, it indeed implies simpler case for MT radio link monitoring, and the requirement correspondingly, as the radio propagation environment would not be changed too frequently as UE in spite of more sensitive because of higher frequency of FR2. Thus we can expect a much simplified case regarding to UE RLM procedure for IAB-MT. Therefore, some unnecessary parts can be removed from UE for IAB.

Proposal 2: Consider non-mobility IAB in Rel-16, RLM procedure for IAB-MT could be simpler case compared to UE’s procedure. Unnecessary parts may be removed from UE’s requirement for IAB’s requirement.

- Requirement Analysis

As shown in previous contributions, though in Rel-16 the IAB is assumed fixed in location and the connection between MT and DU would be more stable than normal UE, MT still could experience radio link deterioration due to unexpected blockage. Considering this deterioration may be probably caused by external environment change, here the channel between the MT and DU could be modeled as a very slow fading channel in time scale.

In addition, the radio propagation on mmWave band usually has less radio paths and rays than carriers of lower frequency. Normally it is assumed that in mmWave band most power will concentrate on the LoS path; it is however in practice that the IAB will be deployed along a street or stick to a building, in which case 1 or 2 alternative paths may be generated by the reflector close to MT.

Fig. Possible blockage on LoS path where link can be recovered by beam switching

Therefore, channel between MT and DU would be a very slow changing channel, with very small delay spread as well as very small doppler spread, but not merely AWGN. Consequently, the coherent time of the channel between MT and DU will be increased. Even if unexpected blockage occurs, radio link can be probably recovered by beam failure recovery procedure, switching to another beam to the alternative path. This still can secure no radio link failure occurs.

Observation 1: If unexpected blockage occurs, radio link can be probably recovered by beam failure recovery procedure, switching to another beam to the alternative path.

To summary, the channel between MT and DU can be hardly experience radio link failure. In light of this, previous RLM requirement for rel-15 UE can be much relaxed.

Proposal 3: Apply relaxed RLM requirement of NR UE to the requirement of IAB-MT.

The time of Evaluation period for RLM requirement for UE in TS 38.133 [2] is captured below (for clarity only SSB based tables are listed):

Table 8.1.2.2-2: Evaluation period TEvaluate_out_SSB and TEvaluate_in_SSB for FR2 Configuration; TEvaluate_out_SSB (ms); TEvaluate_in_SSB (ms). no DRX; Max(200, Ceil(10 P N) TSSB); Max(100, Ceil(5 P N) TSSB). DRX cycle≤320ms; Max(200, Ceil(15 P N) Max(TDRX,TSSB)); Max(100, Ceil(7.5 P N) Max(TDRX,TSSB)). DRX cycle>320ms; Ceil(10 P N) TDRX; Ceil(5 P N) TDRX. NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length..

From the table, it is found that the UE RLM evaluation period is highly dependent on DRX. However, unlike UE, an IAB node is not expected to operate in DRX in most cases as the more data package transmission by an IAB node than a UE. If an IAB is often working in DRX, possibly the IAB topology and routing need to reconsider. Further, IAB as a wired transmission node, it does not have the power limitation as UE does. Hence, DRX cycle should not be taken into consideration for the purpose of defining IAB RLM requirement.

Proposal 4: Requirement of Evaluation Period of IAB RLM should be defined which is applicable for non-DRX only.

As mentioned above, the channel state for MT is more stable than for UE. The existing evaluation period in the table is no longer suitable for MT since it is defined for UE which may move quickly or self-rotate, leading to quick channel state changing. Now that IAB-MT channel experience a comparatively long coherence time where the LoS propagation or less path can be expected, it is necessary to have RLM requirement for the scenario. To allow more samples for evaluation period is reasonable in the case, i.e. largely improve the number of samples (10 and 5 in above table) in UE requirement. Also, the lower boundary (200 and 100 in above table) of Evaluation Period should be increased accordingly for both cases of SSB and CSI-RS as RLM-RS resource.

Proposal 5: For IAB RLM requirement, largely increase the number of samples and the lower boundary of the Evaluation Period of original requirement of UE in both SSB and CSI-RS cases.

Another objective for defining a relaxed RLM requirement for IAB is to save the monitoring overhead since IAB node is expected to be busier, or in other words, higher throughput than a normal UE. The same logic should be used for other parameters of the RLM requirement, for example, the Qin and Qout level. When UE is not configured with signaling rlmInSyncOutOfSyncThreshold from the network, UE determines out-of-sync and in-sync block error rates from Config#0 as 10% and 2%, as shown in the table below.

Table 8.1.1-1: Out-of-sync and in-sync block error rates Configuration; BLERout; BLERin. 0; 10%; 2%.

Nevertheless, whether the default BLER level is still suitable for IAB-MT is an unresolved problem. A more strict Qout BLER level may lead to higher sensitivity of channel deterioration and help UE quickly re-establish the connection upon radio link failure, increasing the IAB throughput. Therefore more discussions are needed for the Qin and Qout BLER level. The way could be defining more configurations, adjusting the BLER level directly or directly informed by higher layers.

Proposal 6: Consider the out-of-sync and in-sync block error rates whether it is feasible for IAB RLM requirement and discuss on how we can modify it.

In UE RLM requirement, there is significant factor to calculate the time of evaluation period, i.e. the sharing factor P in TS 38.133, for both SSB case and CSI-RS case. UE has many types of reference signals to do RLM, measurement, beam management and the like, thus the sharing factor is quite important for defining UE RRM requirement. Yet compared to a UE which is always short of reference signal in NR, IAB do not need to do overburdened works as UE does. On the other hand, UE indeed cannot support to long evaluation period considering mobility, but IAB can do that as the aforementioned. If the evaluation period for IAB RLM is largely increased from that of UE, the sharing factor P is no longer important for IAB since the conflict of reference signal for different purposes will be relieved to a large extent. In this case, the sharing factor P is not necessary and can be omitted in IAB RLM requirement. As no requirement of measurement for MT now, MT may not have too many reference signal conflict as UE. Besides, the sharing factor P brought a great complexity to current Evaluation Period computation of UE BFD and CBD requirement, and remove the P could lead to a simple and clear specification.

Proposal 7: Remove the sharing factor P in Evaluation Period calculation for IAB RLM requirement if the number of samples and the lower boundary is largely increased.

In the current UE requirement, the evaluation period for CSI-RS based RLM is defined for Density=3 case. For CSI-RS of Density=1 case, it is raised a concern that it cannot provide enough accuracy under propagation channel with long delay spread. This is reasonable concern for UE requirement, but for IAB the case will be different. As analyzed above, the delay spread for MT and will much longer than the time for UE, let alone the increased number of samples for MT RLM requirement. If Density=1 is not defined, it is in essence means that the network cannot configure D=1 for RLM CSI-RS even it is more proper for the network. Meanwhile, D=1 leads to a smaller overhead compared to D=3 and network can decide which density to use based on current scenario. Especially for IAB case where LOS propagation or small number of path can be expected, more flexibility would be brought if network can choose to use Density=1.

Proposal 8: For IAB RLM requirement, the case where Evaluation Period for CSI-RS of RLM with Density=1 should be taken into consideration.

- Conclusion

In this paper, we analyze the radio link monitoring requirement for IAB-MT, clearly showing that the IAB RLM requirement can be defined based on the UE RLM requirement and how could it be modified from existing UE requirement to fit IAB scenario. We have made the following proposals for helping define the IAB RLM requirements.

Proposal 1: For IAB radio link monitoring, the requirement for FR2 could be first discussed and then extended to FR1.

Proposal 2: Consider non-mobility IAB in Rel-16, RLM procedure for IAB-MT could be simpler case compared to UE’s procedure. Unnecessary parts may be removed from UE’s requirement for IAB’s requirement.

Observation 1: If unexpected blockage occurs, radio link can be probably recovered by beam failure recovery procedure, switching to another beam to the alternative path.

Proposal 3: Apply relaxed RLM requirement of NR UE to the requirement of IAB-MT.

Proposal 4: Requirement of Evaluation Period of IAB RLM should be defined which is applicable for non-DRX only..

Proposal 5: For IAB RLM requirement, largely increase the number of samples and the lower boundary of the Evaluation Period of original requirement of UE in both SSB and CSI-RS cases.

Proposal 6: Consider the out-of-sync and in-sync block error rates whether it is feasible for IAB RLM requirement and discuss on how we can modify it.

Proposal 7: Remove the sharing factor P in Evaluation Period calculation for IAB RLM requirement if the number of samples and the lower boundary is largely increased.

Proposal 8: For IAB RLM requirement, the case where Evaluation Period for CSI-RS of RLM with Density=1 should be taken into consideration.

- Reference

[1] RP-193199, “Proposed Way Forward on RRM Requirements for the IAB Project”, Qualcomm Inc, AT&T, KDDI, Samsung, Telstra, ·Ericsson, RP#86.

[2] TS 38.133 V16.2.0, Dec. 2019.