3GPP TSG-RAN WG4 Meeting #94-e                                                               R4-2001060

Electronic Meeting, 24th February – 6th March, 2020

Source:	Huawei, HiSilicon, BT plc

Title:	Updated TP to TR 38.716.02-00 for CA_n1A-n78(2A)

Agenda item:	9.2.2

Document for:	Approval

Background

This contribution provides text proposal on the NR CA band combination CA_n1A-n78(2A) as defined in Revised WID on NR inter-band Carrier Aggregation/Dual connectivity for 2 bands DL with x bands UL (x=1, 2) RP-192615 [1].

Text Proposal

---Start of changes---

6.1	CA_n1-n78

6.1.1		Common for 1 band UL and 2 bands UL

6.1.1.1		Operating bands for CA

Table 6.1.1.1-1: CA band combination of band n1+n78

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |

6.1.1.2		Channel bandwidths per operating band for CA

Table 6.1.1.2-1: Supported bandwidths per CA band combination of band n1+n78

| CA operating / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Bandwidth combination set |
| CA_n1A-n78A | CA_n1A-n78A | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes | Yes | Yes | Yes |  |
| CA_n1A-n78(2A) | CA_n1A-n78A | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78(2A) Bandwidth Combination Set 0 in Table 5.5A.2-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n1A-n78C | CA_n1A-n78A | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |
|  |  | n78 | See CA_n78C Bandwidth Combination Set 0 in Table 5.5A.1-1 in 38.101-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |

6.1.1.3		UE co-existence studies

Table 6.1.1.3-1/2 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_n1-n78.

Table 6.1.1.3-1: Impact of UL/DL Harmonic

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | nth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 3840 | 3960 | 5760 | 5940 |  |  |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 |  |  |

Based on above table, there is no harmonic issue for the band combination of n1 and n78.

Table 6.1.1.3-2: Impact of UL/DL Harmonic mixing

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | mth Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| n1 | 1920 | 1980 | 2110 | 2170 | 4220 | 4340 | 6330 | 6510 |  |  |
| n78 | 3300 | 3800 | 3300 | 3800 | 6600 | 7600 | 9900 | 11400 |  |  |

Based on above table, there is no harmonic mixing issue for the band combination of n1 and n78.

6.1.1.4		∆TIB and ∆RIB values

For CA_n1-n78 , the TIB,c and RIB,c values are given in the tables below.

Table 6.1.1.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n78 | n1 | 0.3 |
|  | n78 | 0.8 |

Table 6.1.1.4-2: ΔRIB,c

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n78 | n1 | 0 |
|  | n78 | 0.5 |

6.1.1.5		REFSENS requirements

There are no specific REFSENS requirements

6.1.2		Specific for 2 bands UL CA

6.1.2.1		UE co-existence studies

Table 6.1.2.1-1 lists Band n1 +Band n78 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 6.1.2.1-1: Band n1 and Band n78 UL harmonics and IMD products

| UE UL carriers | fx_low | fx_high |  | fy_low | fy_high |  |
| --- | --- | --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1920 | 1980 |  | 3300 | 3800 |  |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ |  | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 1320 – 1880 |  |  | 5220 – 5780 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ |  | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |  |
| IMD frequency limits (MHz) | 40 – 660 |  |  | 4620 – 5680 |  |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ |  | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |  |
| IMD frequency limits (MHz) | 7140 – 7760 |  |  | 8520 – 9580 |  |  |
| Two-tone 3rd order IMD products | (fx_low – max BW fy) |  | (fx_high + max BW fy) | (fy_low – max BW fx) |  | (fy_high + max BW fx) |
| IMD frequency limits (MHz) | 1820 – 2080 |  |  | 3280 – 3820 |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ |  | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ |  | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1960 – 2640 |  |  | 7920 – 9480 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ |  | ¦2*fx_high –2* fy_low¦ |  |  |  |
| IMD frequency limits (MHz) | 2640 – 3760 |  |  |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ |  | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ |  | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 9060 – 9740 |  |  | 11820 – 13380 |  |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ |  | ¦2*fx_high +2* fy_high¦ |  |  |  |
| IMD frequency limits (MHz) | 10440 – 11560 |  |  |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ |  | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ |  | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 11220 – 13280 |  |  | 3880 – 4620 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ |  | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ |  | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 5940 – 7560 |  |  | 660 – 1840 |  |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ |  | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ |  | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 15120 – 17180 |  |  | 10980 – 11720 |  |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ |  | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ |  | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13740 – 15360 |  |  | 12360 – 13540 |  |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. |  |  |  |  |  |  |

Based on above table ,  4th order IMD may fall into Rx frequencies of band n1.

Table 6.1.2.1-2 lists the protected bands required for the 2UL bands CA configuration.

Table 6.1.2.1-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n1-n78 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 26, 28, 34, 40, 41, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1880 | - | 1895 | -40 | 1 | RB restriction |
|  | Frequency range | 1895 | - | 1915 | -15.5 | 5 | RB restriction |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | RB restriction |
| NOTE 1: To simplify Table, E-UTRA band numbers are listed for bands which are specified only for E-UTRA operation or both E-UTRA and NR operation. NR band numbers are listed for bands which are specified only for NR operation. |  |  |  |  |  |  |  |

6.1.2.2		REFSENS requirements

Table 6.1.2.2-1 lists the MSD required due to 4th  IMD for the dual uplink configuration.

Table 6.1.2.2-1: MSD due to IMD issue

| Operating band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | Operating band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n1-n78 | n1 | 1950 | 5 | 25 | 2140 | 8.0 | FDD | IMD4 |
|  |  |  |  |  |  | 10.7 2 |  |  |
|  | n78 | 3710 | 10 | 50 | 3710 | N/A | TDD | N/A |
| NOTE 1: RBSTART = 0 NOTE 2: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 3: 15 kHz SCS is assumed. |  |  |  |  |  |  |  |  |

---End of changes---

Reference

[1]	RP-192615, “Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2)”, ZTE Corporation