3GPP TSG-RAN WG4 Meeting #94-e	R4-2001457

# Electronic Meeting, 24th Feb - 6th Mar, 2020

Title: 	Discussion on the requirement definition for NR UE HST single tap channel

Source: 	Huawei, HiSilicon

Agenda item:	8.17.2.1.5

Document for:	Discussion

Background

During RAN4#93 meeting, way forward [1] for NR Rel-16 UE demodulation was approved. The issue about single tap requirements definition is shown below:

- Single tap requirements definition - Option A: Define requirements based on worst case and UE performs multi-shot TRS-based time/freq tracking - Option B: Define requirements under assumption UE is aware on HST single tap conditions and can adjust time/freq tracking algorithms - Further study how UE can become aware on conditions - Option 1: UE detects the conditions - Option 2: Rely on agreed NR HST RRM enhancement network assistance signaling - Option 3: Additional network assistance is provided - Further study UE demodulation performance under HST single tap conditions for the case of single-shot and multi-shot TRS-based tracking

In this contribution, we share our views about the requirement definition for NR Rel-16 UE HST single tap channel.

Discussion

In contribution [2] and [3], n-shot problem was proposed. This sub-section is to study whether there is any performance loss using n-shot filter based frequency tracking.

Figure 2.1-1: residual frequency error for n shots

Figure 2.1-1 shows residual frequency error for 1, 2, 4 and 8 shots for maximum Doppler shift 1667Hz. As we can see, the maximum residual frequency error is about 1500Hz, 2000Hz, 2500Hz, 2700Hz corresponded to 1, 2, 4 and 8 shots, respectively. The time UE compensating residual frequency error is about 40ms, 100ms, 200ms and 500ms corresponded to 1, 2, 4 and 8 shots, respectively.

Observation 1: With the number of shots increases, the maximum residual frequency error and the time UE compensating residual frequency error also increases.

Further, we provide our simulations for different number of shots filtering as per the simulation assumptions in [1] for HST single-tap for TDD as following:

Table 3-1: Ideal Simulation results for NR PDSCH under HST single-tap

| Case Number | CHBW/ SCS | Max Doppler shift fd (Hz) | MIMO | MCS | filter | SNR@70% Max TP |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 40MHz/30kHz | 1667 | 2x2 | 13 | 1-shots | 4.48 |
| 2 | 40MHz/30kHz | 1667 | 2x4 | 13 | 1-shots | 2.40 |
| 3 | 40MHz/30kHz | 1667 | 2x2 | 13 | 2-shots | 4.48 |
| 4 | 40MHz/30kHz | 1667 | 2x4 | 13 | 2-shots | 2.40 |
| 5 | 40MHz/30kHz | 1667 | 2x2 | 13 | 4-shots | 4.48 |
| 6 | 40MHz/30kHz | 1667 | 2x4 | 13 | 4-shots | 2.40 |
| 7 | 40MHz/30kHz | 1667 | 2x2 | 13 | 8-shots | 4.49 |
| 8 | 40MHz/30kHz | 1667 | 2x4 | 13 | 8-shots | 2.41 |

Figure 3-1: Ideal Simulation results for NR PDSCH under HST single-tap

Observation 2: There is almost no influence in testing metric for different number of shots, although with the different residual frequency error.

Network assistance signaling is used for improving performance, e.g. receiving signaling in SFN scenario to change the corresponding receiver algorithm. However, we do not find a glaring difference between using multi-shot or not as per our simulation results. So we think there is no need to define additional network assistance for single-tap. Besides, the number of shots to use for filtering is an implement question and it is not reasonable to require all companies to use the same implementation.

Proposal 1: Define requirements based on worst case since there is almost no performance improvement for different number of shots.

Proposal 2: Whether to use single-shot or to use multi-shot depends on UE implement and should not be limited.

Proposal 3: No need to define additional network assistance for single-tap.

Proposals

In this contribution, we discuss and provide our simulation results on NR UE HST performance requirements under single-tap channel model. Our observations and proposals are:

Observation 1: With the number of shots increases, the maximum residual frequency error and the time UE compensating residual frequency error also increases.

Observation 2: There is almost no influence in testing metric for different number of shots, although with the different residual frequency error.

Proposal 1: Define requirements based on worst case since there is almost no performance improvement for different number of shots.

Proposal 2: Whether to use single-shot or to use multi-shot depends on UE implement and should not be limited.

Proposal 3: No need to define additional network assistance for single-tap.

Reference

- R4-1915926, WF on UE demodulation for NR HST, RAN4#93, CMCC

- R4-1911004, Views on the demodulation requirements for NR single tap HST scenario, RAN4#92bis, Intel

- R4-1913430, Views on NR UE demodulation requirements for HST single tap scenario, RAN4#93, Intel