3GPP TSG-RAN WG4 Meeting #94-e	R4-2001732

Electronic Meeting, February 24-March 6, 2020

Agenda Item:	8.1.1.2

Source:	Futurewei

Title:	NR-U Guard band analysis

Document for:	Approval

# Introduction

In RAN4#93, discussions about guard band for NR-U were captured in [1]. In particular, when a large bandwidth channel is divided into subchannels, how to define the subchannels in terms of RBs was proposed. This contribution analyzes the proposal and provides enhancements to account for the guard bands of various subchannel combinations.

# Discussion

## Background

In [1], the WF listed how 20 MHz subchannels can placed in channels of 40, 60, 80 MHz as a function of SCS. The following table from [1] captured the combinations. For 60 kHz SCS, the current release 15 standard sets the guard band for 20 MHz channels to 1330 kHz (alt. 1). A working agreement for release 16 is to reduce the guard band to 970 kHz (alt. 2).

Table 1. WF table

With this design, point A is fixed for the channel, and a BWP spans the entire bandwidth. Subchannels are allocated as part of the BWP grid with guard bands which are aligned to the grid. In general, the actual intra-carrier guard bands are larger than the minimum guard band for the subchannel.

Although the base station expects the UE to meet the RF requirements associated with the channel, in NR-U, the UE should also meet the RF requirements for the subchannel.

There are two perspectives to be consider:

- Base station: since the gNB schedules one or more UEs in the slot, the gNB must be aware of guard bands for each subchannel transmitted and received

- In general, each UE only receives scheduling information for itself. It needs to determine the correct guard band size for the allocation

## General procedure

The proposed table in Table 1 is applicable for all channels and subcarrier spacings when every subchannel is 20 MHz.

Observation 1: The proposed RB allocations for 20 MHz subchannels meet the minimum guard band sizes.

However, when subchannel combinations include either 40 or 60 MHz, there is some uncertainty whether the RB allocations in Table 1 are applicable given the guard band constraints. In Table 2, all combinations are highlighted with yellow, blue, and green indicating 2, 3, and 4 subchannels in channel, respectively.

Table 2. Combinations of subchannels as function of channel bandwidth and SCS

| SCS (kHz) | 20 MHz | 40 MHz | 60 MHz | 80 MHz |
| --- | --- | --- | --- | --- |
| 15 | {20} | {40}, {20,20} | N/A | N/A |
| 30 | {20} | {40}, {20,20} | {60}, {40,20}, {20,40}, {20,20,20} | {80}, {60,20}, {20,60}, {40,40}, {40,20,20}, {20,40,20}, {20,20,40}, {20,20,20,20} |
| 60 | {20} | {40}, {20,20} | {60}, {40,20}, {20,40}, {20,20,20} | {80}, {60,20}, {20,60}, {40,40}, {40,20,20}, {20,40,20}, {20,20,40}, {20,20,20,20} |

In the next section, analysis of Table 1 for different subchannel size combinations is presented.

## Analysis

To determine whether the RB allocations are adequate, a numerical analysis is performed for the SCS / channel bandwidth / subchannel combination. Using a 60 MHz channel with 30 kHz SCS as an example:

- With allocation listed in Table 1 for a subchannel configuration {40 MHz, 20 MHz}:

  - Determine the number of RBs for the 40 MHz subchannel (50+6+50)

  - Determine the number of RBs for the intra-carrier guard band between the 40 MHz and 20 MHz subchannels (6)

  - Determine the number of RBs for 20 MHz (50)

- Determine the guard band size for

  - Channel 60 MHz (825 kHz)

  - Determine the guard band for 40 MHz subchannel (905 kHz)

  - Determine the guard band for 20 MHz subchannel (805 kHz)

- Compare guard band sizes needed according to subchannel location

  - Case 1: guard band of the first subchannel compared to inter-channel guard band when the first subchannel is placed on RB#0 of the channel

Example: RB#0 starts at 825 kHz. With the guard band for 40 MHz at 905 kHz, if the first RB of the subchannel is placed on RB#0, there would be insufficient guard band for the 40 MHz subchannel

  - Case 2: guard band of the subchannel compared to the frequency difference between the starting location of the RBs containing the intra-carrier guard band and the bandwidth of the subchannel

Example: RE#0 of RB#106 starts at 825 kHz + 106 RB×12 RE/RB×30 kHz/RE = 38985 kHz. There is 1015 kHz of guard band (40-38.985 MHz). Minimum needed is 905 kHz.

  - Case 3: guard band of the subchannel compared to the frequency difference between the starting location of the first RB after the intra-carrier guard

Example: RB#112 starts at 825 kHz + 112 RB×12 RE/RB×30 kHz/RE = 41145 kHz. There is 1145 kHz of guard band (41.145-40 MHz). Minimum needed is 805 kHz

  - Case 4: guard band of the last subchannel compared to the inter-channel guard band

Example: RB#162 begins at 825 kHz + 162 RB×12 RE/RB×30 kHz/RE = 59145 kHz. There is 855 kHz of guard band (60-59.145 MHz). Minimum needed is 825 kHz.

The analysis is applied to the allocations as suggested by Table 1 to the subchannel combinations listed in Table 2. Table 3 captures the potential issues for the allocations as suggested by Table 1. In the table, the colors: red correspond to note 1 while green corresponds to note 2. Purple refers to note 3.

Table 3. Comparison of allocations.

| BW, MHz | SCS, kHz | Subchan, MHz | Proposed by Table 1 | Computed sizes of [guard band, subchannel, guard band] in kHz | Comments (x: y) (bandwidth MHz, Guard kHz) |
| --- | --- | --- | --- | --- | --- |
| 40 | 15 | {20,20} | 105-6-105 | 552.5 18900 547.5 532.5 18900 567.5 | (40: 552.5) (20: 452.5) |
|  | 30 | {20,20} | 50-6-50 | 905 18000 1095 1065 18000 935 | (40: 905) (20: 805) |
|  | 60 Alt1 | {20,20} | 23-5-23 | 1610 16560 1830 1770 16560 1670 | (40: 1610) (20: 1330) |
|  | 60 Alt2 | {20,20} | 24-3-24 | 1610 17280 1110 1050 17280 1670 | (40: 1610) (20: 970) |
| 60 | 30 | {20,20,20} | 50-6-50-6-50 | 825 18000 1175 985 18000 1015 1145 18000 855 | (60: 825) (20: 805) |
|  |  | {40,20} | 106-6-50 | 825 38160 1015 1145 18000 855 | Note 1 (should be ≥905) |
|  |  | {20,40} | 50-6-106 | 825 18000 1175 985 38160 855 | Note 1 (should be ≥905) |
|  | 60 Alt1 | {20,20,20} | 23-5-23-5-23 | 1530 16560 1910 1690 16560 1750 1850 16560 1590 | (60: 1530) (20: 1330) |
|  |  | {40,20} | 51-5-23 | 1530 36720 1750 1850 16560 1590 | Note 1 (should be ≥1610) |
|  |  | {20,40} | 23-5-51 | 1530 16560 1910 1690 36720 1590 | Note 1 (should be ≥1610) |
|  | 60 Alt2 | {20,20,20} | 24-3-25-3-24 | 1530 17280 1190 970 18000 1030 1130 17280 1590 | (60: 1530) (20: 970) Note 3 (should be >970) |
|  |  | {40,20} | 52-3-24 | 1530 37440 1030 1130 17280 1590 | Note 1: (should be ≥1610) Note 2: (should be ≥1610) |
|  |  | {20,40} | 24-3-52 | 1530 17280 1190 970 37440 1590 | Note 1: (should be ≥1610) Note 2: (should be ≥1610) |
| 80 | 30 | {20,20,20,20} | 50-6-50-5-50-6-50 | 925 18000 1075 1085 18000 915 885 18000 1115 1045 18000 955 | (80: 925) (20: 805) |
|  |  | {40,40} | 106-5-106 | 925 38160 915 885 38160 955 | Note 2 (should be ≥905) Note 3 (should be >905 |
|  |  | {40,20,20} | 106-5-50-6-50 | 925 38160 915 885 18000 1115 1045 18000 955 | (80: 925) (20: 805) (40: 905) Note 3 (should be >905) |
|  |  | {20,40,20} | 50-6-105-6-50 | 925 18000 1075 1085 37800 1115 1045 18000 955 | (80: 925) (20: 805) (40: 905) |
|  |  | {20,20,40} | 50-6-50-5-106 | 925 18000 1075 1085 18000 915 885 38160 955 | Note 2 (should be ≥905) |
|  |  | {60,20} | 161-6-50 | 925 57960 1115 1045 18000 955 | (80: 925) (20: 805) (60: 825) |
|  |  | {20,60} | 50-6-161 | 925 18000 1075 1085 57960 955 | (80: 925) (20: 805) (60: 825) |
|  | 60 Alt1 | {20,20,20,20} | 23-5-23-5-23-5-23 | 1450 16560 1990 1610 16560 1830 1770 16560 1670 1930 16560 1510 | (80: 1450) (20: 1330) |
|  |  | {40,40} | 51-5-51 | 1450 36720 1830 1770 36720 1510 | Note 1 (should be ≥1610) |
|  |  | {40,20,20} | 51-5-23-5-23 | 1450 36720 1830 1770 16560 1670 1930 16560 1510 | Note 1 (should be ≥1610) |
|  |  | {20,40,20} | 23-5-51-5-23 | 1450 16560 1990 1610 36720 1670 1930 16560 1510 | (80: 1450) (20: 1330) (40: 1610) Note 3 (should be >1610) |
|  |  | {20,20,40} | 23-5-23-5-51 | 1450 16560 1990 1610 16560 1830 1770 36720 1510 | Note 1 (should be ≥1610) |
|  |  | {60,20} | 79-5-23 | 1450 56880 1670 1930 16560 1510 | Note 1 (should be ≥1530) |
|  |  | {20,60} | 23-5-79 | 1450 16560 1990 1610 56880 1510 | Note 1 (should be ≥1530) |
|  | 60 Alt2 | {20,20,20,20} | 24-4-24-3-24-4-24 | 1450 17280 1270 1610 17280 1110 1050 17280 1670 1210 17280 1510 | (80: 1450) (20: 970) |
|  |  | {40,40} | 52-3-52 | 1450 37440 1110 1050 37440 1510 | Note 1: (should be ≥1610) Note 2: (should be ≥1610) |
|  |  | {40,20,20} | 52-3-24-4-24 | 1450 37440 1110 1050 17280 1670 1210 17280 1510 | Note 1: (should be ≥1610) Note 2: (should be ≥1610) |
|  |  | {20,40,20} | 24-4-51-4-24 | 1450 17280 1270 1610 36720 1670 1210 17280 1510 | (80: 1450) (20: 970) (40: 1610) Note 3 (should be >1610) |
|  |  | {20,20,40} | 24-4-24-3-52 | 1450 17280 1270 1610 17280 1110 1050 37440 1510 | Note 1: (should be ≥1610) Note 2: (should be ≥1610) |
|  |  | {60,20} | 79-4-24 | 1450 56880 1670 1210 17280 1510 | Note 1 (should be ≥1530) |
|  |  | {20,60} | 24-4-79 | 1450 17280 1270 1610 56880 1510 | Note 1 (should be ≥1530) |

Notes:

- Issue arises when the guard band for subchannel larger than guard band for channel (inter-channel guard band).

- Issue arises when the actual guard band needed between subchannels is larger than the guard band proposed in Table 1.

- The RE position can vary by up SCS due to the channel roster. The guard band may not be adequate depending on the channel raster.

Observation 2: the proposed table does not provide guard band for some combinations with 40 MHz and some combinations with 60 MHz subchannels.

For subchannel combinations indicated by note 1, decreasing the number of RBs while increasing the guard band via allocation (scheduling) can resolve the issue. Another possible way is to modify the table to capture the condition of when the guard band of the subchannel is greater than the guard band of the channel, an adjusting the entries of the table is needed.

For subchannel combinations indicated by note 2, the issue happens when the subchannel is greater than 20 MHz and the middle guard band is smaller in RBs when compared to the other guard bands.

To correct issue 1 (red), one RB of the subchannel in question is allocated as the inter-channel guard band. To correct the second issue (green), one RB of the subchannel in question is used for the intra-channel guard band. Table 4 captures those corrections.

Table 4. After corrections

| BW, MHz | SCS, kHz | Subchan, MHz | Proposed by Table 1 and with modifications | Computed sizes of [guard band, subchannel, guard band] in kHz |
| --- | --- | --- | --- | --- |
| 40 | 15 | {20,20} | 105-6-105 |  |
|  | 30 | {20,20} | 50-6-50 |  |
|  | 60 Alt1 | {20,20} | 23-5-23 |  |
|  | 60 Alt2 | {20,20} | 24-3-24 |  |
| 60 | 30 | {20,20,20} | 50-6-50-6-50 |  |
|  |  | {40,20} | 105-6-50 | 1185 37800 1015 (now GB ≥905) 1145 18000 855 |
|  |  | {20,40} | 50-6-105 | 825 18000 1175 985 37800 1215 (now GB ≥905) |
|  | 60 Alt1 | {20,20,20} | 23-5-23-5-23 |  |
|  |  | {40,20} | 50-5-23 | 2250 36000 1750 (GB ≥1610) 1850 16560 1590 |
|  |  | {20,40} | 23-5-50 | 1530 16560 1910 1690 36000 2310 (GB ≥1610) |
|  | 60 Alt2 | {20,20,20} | 24-3-25-3-24 |  |
|  |  | {40,20} | 50-4-24 | 2250 36000 1750 (GB ≥1610) 1130 17280 1590 (GB ≥1610) |
|  |  | {20,40} | 24-4-50 | 1530 17280 1190 (GB ≥1610) 1690 36000 2310 (GB ≥1610) |
| 80 | 30 | {20,20,20,20} | 50-6-50-5-50-6-50 |  |
|  |  | {40,40} | 106-6-105 | 925 38160 915 1245 37800 955 (now GB ≥905) |
|  |  | {40,20,20} | 106-5-50-6-50 |  |
|  |  | {20,40,20} | 50-6-105-6-50 |  |
|  |  | {20,20,40} | 50-6-50-6-105 | 925 18000 1075 1085 18000 915 1245 37800 955 (now GB ≥905) |
|  |  | {60,20} | 161-6-50 |  |
|  |  | {20,60} | 50-6-161 |  |
|  | 60 Alt1 | {20,20,20,20} | 23-5-23-5-23-5-23 |  |
|  |  | {40,40} | 50-5-50 | 2170 36000 1830 (GB ≥1610) 1770 36000 2230 (GB ≥1610) |
|  |  | {40,20,20} | 50-5-23-5-23 | 2170 36000 1830 1770 16560 1670 1930 16560 1510 |
|  |  | {20,40,20} | 23-5-51-5-23 |  |
|  |  | {20,20,40} | 23-5-23-5-50 | 1450 16560 1990 1610 16560 1830 1770 36000 2230 (GB ≥1610) |
|  |  | {60,20} | 78-5-23 | 2170 56160 1670 (GB ≥1530) 1930 16560 1510 |
|  |  | {20,60} | 23-5-78 | 1450 16560 1990 1610 56160 2230 (GB ≥1530) |
|  | 60 Alt2 | {20,20,20,20} | 24-4-24-3-24-4-24 |  |
|  |  | {40,40} | 50-5-50 | 2170 36000 1830 (GB ≥1610) 1770 36000 2230 (GB ≥1610) |
|  |  | {40,20,20} | 50-4-24-4-24 | 2170 36000 1830 (GB ≥1610) 1050 17280 1670 (GB ≥1610) 1210 17280 1510 |
|  |  | {20,40,20} | 24-4-51-4-24 |  |
|  |  | {20,20,40} | 24-4-24-4-50 | 1450 17280 1270 1610 17280 1110 (GB ≥1610) 1770 36000 2230 (GB ≥1610) |
|  |  | {60,20} | 78-4-24 | 2170 56160 1670 (GB ≥1530) 1210 17280 1510 |
|  |  | {20,60} | 24-4-78 | 1450 17280 1270 1610 56160 2230 (GB ≥1530) |

Proposal 1: Table 4 provides sufficient minimum guard band for the various subchannels.

## Discussion

The table from the WF (Table 1) and the corrections listed in Table 4 provide the basis for capturing the RB allocations for the various subchannel combinations.

Table 5 and Table 6 summarize the allocations for the guard band and the subchannels for 60 and 80 MHz channels, respectively. The color coding indicates the differences between from Table 1. Some general rules are captured below.

Table 5. Summary of the allocations for 60 MHz channel. When the prefix / suffix is 1, one RB is used for guard).

| Subchannel, MHz | 30 kHz SCS | 60 kHz SCS Alt 1 | 60 kHz SCS Alt 2 |
| --- | --- | --- | --- |
| {60} | 162 | 79 | 79 |
| {20,20,20} | 0-50-6-50-6-50-0 | 0-23-5-23-5-23-0 | 0-24-3-25-3-24-0 |
| {40,20} | 1-105-6-50-0 | 1-50-5-23-0 | 1-50-4-24-1 |
| {20,40} | 0-50-6-105-1 | 0-23-5-50-1 | 0-24-4-50-1 |

Table 6. Summary of the allocations for 80 MHz channel. When the prefix / suffix is 1, one RB is used for guard).

| Subchannel, MHz | 30 kHz SCS | 60 kHz SCS Alt 1 | 60 kHz SCS Alt 2 |
| --- | --- | --- | --- |
| {80} | 217 | 107 | 107 |
| {20,20,20,20} | 0-50-6-50-5-50-6-50 | 0-23-5-23-5-23-5-23-0 | 0-24-4-24-3-24-4-24-0 |
| {40,40} | 0-106-6-105-0 | 1-50-5-50-1 | 1-50-5-50-1 |
| {40,20,20} | 0-106-5-50-6-50-0 | 1-50-5-23-5-23-0 | 1-50-4-24-4-24-0 |
| {20,40,20} | 0-50-6-105-6-50-0 | 0-23-5-51-5-23-0 | 0-24-4-51-4-24-0 |
| {20,20,40} | 0-50-6-50-6-105-0 | 0-23-5-23-5-50-1 | 0-24-4-24-4-50-1 |
| {60,20} | 0-161-6-50-0 | 1-78-5-23-0 | 1-78-4-24-0 |
| {20,60} | 0-50-6-161-0 | 0-23-5-78-1 | 0-24-4-78-1 |

Rule 1: When subchannel placed at channel edge and if guard band of channel < guard band of subchannel, an extra RB is needed for guard band. (If true, one RB for the subchannel is used for guard (as noted with the prefix / suffix of 1)

- For example: a channel of 60 MHz has a 1530 kHz guard band while a subchannel of 40 MHz needs a guard band of 1610 kHz.

Rule 2: For intra-band guard for subchannel A and subchannel B, the sum of the guard bands for subchannel A and subchannel B should exceed the RB allocations for the intra-band guard.

- For example: two 40 MHz subchannels require 2×1610= 3220 kHz. With 60 kHz SCS, nearly 5 RBs are needed.

Note that rule 2 depends on how the RB alignment of the BWP is near the intra-band guard.

However, because the base station provides resource allocations on a UE-specific basis, the UE can only determine the subchannel bandwidth from resource allocation in the DCI. Specifically, the UE must determine the bandwidth according to SCS and resource allocation.

When comparing the channel bandwidth, SCS, and resource block allocation listed in Table 5.3.2-1 of 38.104 to the used allocations in Table 4, as shown in Table 7, the used allocations are the same or one less than what is specified.

Table 7. Comparison of allocations.

|  | Table 5.3.2-1 of 38.104 |  |  | Allocations in Table 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 20 MHz | 40 MHz | 60 MHz | 20 MHz | 40 MHz | 60 MHz |
| SCS, kHz | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 106 | 216 | N/A | 105 |  |  |
| 30 | 51 | 106 | 162 | 50 | 105, 106 | 161 |
| 60 Alt 1 | 24 | 51 | 79 | 23 | 50 | 78 |
| 60 Alt 2 | 25 | 51 | 79 | 24, 25 | 50, 51 | 78 |

Observation 3: It may not be necessary to capture subchannel bandwidths in the standards since the allocations are already specified.

# Conclusions

This contribution examines the guard bands for various subchannel combinations for a channel bandwidth and SCS.

Observation 1: The proposed RB allocations for 20 MHz subchannels meet the minimum guard band sizes.

There are some inconsistencies in the allocation.

Observation 2: the proposed table does not provide guard band for some combinations with 40 MHz and some combinations with 60 MHz subchannels.

Analysis provided some corrections to the table.

Proposal 1: Table 4 provides sufficient minimum guard band for the various subchannels.

In general, a table is not needed because the base station provides the allocations.

Observation 3: It may not be necessary to capture subchannel bandwidths in the standards since the allocations are already specified.

# References

R4-1916160, “WF on Guardbands for NR-U”, RAN4#93, Nov. 18-22, 2019.