3GPP TSG RAN WG4 Meeting #94-e	R4-2001887

Electronic Meeting, February 24-March 06, 2020

Agenda Item:	8.5.1

Source: 	Ericsson

Title:	TP to TR 38.xxx: RF Requirements reference points

Document for:	Approval

- Introduction

After RAN4#92bis the IAB TR [1] is email approved. Section 2 provides text proposal on 4.2 RF Requirements reference points for the IAB TR 38.xxx (Integrated access and backhaul radio transmission and reception).

# Text Proposal

--------------------------------------------------Start of TP------------------------------------------------------

## 4.2	RF Requirements reference points

<Text will be added>

The requirement reference points for legacy BS is defined as below []:

Table 1: BS requirement sets

| BS type / Requirement set | BS Description | Additional information |
| --- | --- | --- |
| 1-C | A BS operating at FR1 with all requirements defined at individual antenna connectors. | Following the approach used in 3GPP TS 36.104 and 3GPP TS 37.104 [19] |
| 1-H | A BS operating at FR1 with a requirement set holding requirements defined at the TAB and OTA requirements defined at RIB. | The requirement set is like the one defined for Hybrid AAS BS. Following the approach used in 3GPP TS 37.105 [7] |
| 1-O | A BS operating at FR1 with a requirement set consisting only OTA requirements defined at the RIB. | Following the approach developed in eAAS and documented in 3GPP TR 37.843 [9]. |
| 2-O | A BS operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB. | This requirement set is relevant for AAS BS and does not require access to RF connectors. |

The test and reference point of RF requirement for IAB DU should be reused and thus the test spec will not be impacted. For wide area IAB class, the test and reference point of IAB DU can be reused for IAB MT, there is no need to define a new reference for IAB MT as the OTA test will be the same referenced at RIB for IAB MT and IAB DU. As the reference points are same for IAB DU and IAB MT, there is no need to further differentiate the type of IAB MT and IAB DU separately, hence only IAB type will be specified.

Table 2: IAB requirement sets

| IAB type / Requirement set | IAB Description | Additional information |
| --- | --- | --- |
| 1-H | A IAB operating at FR1 with a requirement set holding requirements defined at the TAB and OTA requirements defined at RIB | The requirement set for both IAB DU and IAB MT. |
| 1-O | A IAB operating at FR1 with a requirement set consisting only OTA requirements defined at the RIB. | The requirement set for both IAB DU and IAB MT. |
| 2-O | A IAB operating at FR2 with a requirement set consisting only of OTA requirements defined at the RIB. | The requirement set for both IAB DU and IAB MT. |

--------------------------------------------------End of TP------------------------------------------------------

- References

- R4-1913332, “Updated IAB TR”, Samsung, RAN4#93