3GPP TSG-RAN WG4 Meeting #94-e	R4-2002012

Online, 24th February – 6th March 2020

Source:	Ericsson, T-Mobile US, MediaTek

Title:	TP for TR 37.716-21-11: Including band combinations 2-46_n66

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

A text proposal for TR 37.716-21-11 to add DC_2A-46A_n66A, DC_2A-46C_n66A and DC_2A-46D_n66A.

# 2. Text Proposal

---Start of changes---

## 5.1.X	DC_2-46_n66

### 5.1.X.1	Operating bands for EN-DC

Table 5.1.X.1-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-46_n66 | CA_2-46 | n66 | No |

### 5.1.X.2	Configuration for DC

Table 5.1.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-46A_n66A | DC_2A_n66A | CA_2A-46A | n66 |
| DC_2A-46C_n66A | DC_2A_n66A | CA_2A-46C | n66 |
| DC_2A-46D_n66A | DC_2A_n66A | CA_2A-46D | n66 |

### 5.1.X.3		∆TIB and ∆RIB values

For DC_2-46_n66, the TIB,c and RIB,c values are reused from LTE combination CA_2-46-66, and are given in the tables below.

Table 5.1.X.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46_n66 | 2 | 0.5 |
|  | 46 | 0 |
|  | n66 | 0.5 |

Table 5.1.X.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-46_n66 | 2 | 0 |
|  | 46 | 0 |
|  | n66 | 0 |

### 5.1.x.4	REFSENS requirements

Co-existence analysis from DC_2_n66 shows that there is IMD3 and IMD5 impact from DC_2_n66 UL to Band 46 DL. MSD for DC_2-46_n66 is proposed to be defined like in table below, where Note x is derived from Note 8 in TS36.101 Table 7.3.1A-0eA.

Table 5.1.x.4-1: Reference sensitivity QPSK PREFSENS (EN-DC with n46)

| E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| DC_2A_n66A | 46 |  |  |  | 0 |  | 0 |  | 0 | 0 |  | 0 |
| NOTE x: When Band 46 have self-interference problems by dual uplink CA/EN-DC, then the requirements do not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. DC_2A-46A_n66A; DC_2A_n66A; 2*fc_2A + fc_n66A; 2*BW_2A + BW_n66A. DC_2A-46A_n66A; DC_2A_n66A; fc_2A + 2*fc_n66A; BW_2A + 2*BW_n66A. |

---End of changes---

# Reference

[1]		RP-192815, “Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)” Huawei, HiSilicon