3GPPRAN4#94	R4-2002065

Online Meeting, Feb 24th – Mar 6th  , 2020

Title: 	Discussion on UE specific channel BW change

Source: 	Qualcomm Incorporated

Agenda item:	8.15.1.7	UE-specific CBW change	[NR_RRM_Enh_Core]

Document for:	Discussion

# Introduction

The NR RRM enhancement WID [1] has a study item on UE specific channel BW change for both FR1 and FR2. It is also mentioned that the requirements should be power class independent for both FR1 and FR2.

# Discussion

In NR, there is a concept of gNodeB channel BW which is broadcast and a UE specific channel BW. The UE specific channel BW can be smaller or larger than the gNB broadcast BW. The UE will perform RACH using the gNodeB broadcast BW. Once the UE is in connected mode, the gNodB can switch it to a different UE specific BW. This may be done for load balancing purposes or to move the UE to a larger BW than the gNodeB has in broadcast.  In order to change the UE specific channel BW, the gNodeB will provide it with an RRC reconfiguration command.

Observation 1: UE specific channel BW change is done via an RRC command.

The starting point of this procedure will be when the ACK’s the PDSCH containing the RRC command and the end point would be the UE sending an RRC complete back to gNodeB on the active BWP in the new channel BW. In order to switch its channel BW, the UE will need to decode the RRC command and the do some SW processing. This requirements here should be very similar to other RRC based procedures, such as handover etc., that are defined by RAN4. Thus, the time for channel BW switch would consist of RRC processing time and UE processing time.

Proposal 1: The time to switch UE specific channel BW TChannelBWSwitch = TRRC Processing + TUE processing

Where TRRC Processing is the RRC processing time and TUE processing is the UE SW processing time after the decoding the RRC processing.

Like other RRC based procedure requirements the RRC processing time should be defined by RAN2 and the UE processing time should be defined in RAN4.

Proposal 2: RAN4 to define the UE processing time for UE specific channel BW switch.

# Conclusions

Observation 1: UE specific channel BW change is done via an RRC command.

Proposal 1: The time to switch UE specific channel BW TChannelBWSwitch = TRRC Processing + TUE processing

Proposal 2: RAN4 to define the UE processing time for UE specific channel BW switch.

# References

[1] RP-193211, NR RRM Enhancement WID.