3GPP TSG-RAN WG4 Meeting #94-e	R4-2001003

Electronic Meeting, 24th Feb. – 6th Mar. 2020

Title: 	TP on channel arrangement for NR V2X

Source: 	vivo

Agenda item:	8.4.3

Document for:	Approval

- Introduction

Based on [1] and [2], this document presents a TP for TR 38.886 v0.4.0 [3] updating the channel arrangement section for 5G V2X in Section 3 Annex.

- Reference

[1]	R4-2000567, Remaining issues on channel raster for band n47, vivo, RAN4#94-e.

[2]      R4-1913063, LS on sync raster for NR V2X, vivo, RAN4#92bis.

[3]	3GPP TR38.886 v0.4.0, V2X Services based on NR: User Equipment (UE) radio transmission and reception

- Annex: TP for channel arrangement in TR38.886

============================ Start of TP ============================

7.3	Channel arrangement

7.3.1	Channel arrangement in FR1

7.3.1.1	Channel raster

7.3.1.1.1	NR-ARFCN and channel raster

The NR-ARFCN and channel raster defined in subclause 5.4.2.1 in TS38.101-1 are applied for NR V2X.

For NR V2X UE, the reference frequency can be shifted by configuration.

FREF_V2X = FREF + Δshift + N * 5 kHz

where

Δshift = 0 kHz or 7.5 kHz indicated in IE (frequencyShift7p5khz), and

N can be set as one of following values {-1, 0, 1}, are signalled by the network in higher layer parameters or configured by pre-configuration parameters.

7.3.1.1.2	Channel raster to resource element mapping

Channel raster to resource element mapping defined in subclause 5.4.2.2 in TS38.101-1 are applied for NR V2X.

7.3.1.1.3	Channel raster entries for each operating band

The channel raster entries for each operating band defined in subclause 5.4.2.3 in TS38.101-1 are applied for NR V2X. The RF channel positions on the channel raster in each NR V2X operating band are given through the applicable NR-ARFCN in Table 7.3.1.1-1, using the channel raster to resource element mapping in subclause 7.3.1.1.2.

For NR V2X operating band n47, ΔFRaster = I × ΔFGlobal, where I ϵ {1}. Every Ith  NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in table 7.3.1.1-1 is given as <I>.

Table 7.3.1.1-1: Applicable NR-ARFCN for NR V2X operating band

| NR operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n47 | 15 | 790334 – <1> – 795000 | 790334 – <1> – 795000 |

7.3.1.2	Synchronization raster

There is no synchronization raster definition for NR V2X for both licensed bands and unlicensed bands.

7.3.2	Channel arrangement in FR2

============================ End of TP ============================