3GPP TSG RAN WG4 Meeting #94-e	R4-2002654

Online, 24th Feb. – 06th March 2020

Agenda item:	9.7.2

Source:	LG Electronics

Title:	TP on MSD results for new EN-DC band combinations in TR37.716-21-21

Document for:	Approval

1	Introduction

Then new EN-DC of x DL Band LTE (x=1, 2, 3, 4) and 2 DL Band NR were approved in RAN#86[1]. In this paper we propose the required MSD levels for the new EN-DC bands combinations base on summary paper [2].

2	MSD analysis

In rel-16 EN-DC of x DL Band LTE (x=1, 2, 3, 4) and 2 DL Band NR basket WI, RAN4 also consider shared antenna RF architectures for NSA UE in sub-6GHz as LTE system. So we consider shared antenna RF architecture for general NSA DC UE to derive MSD levels.

For the MSD analysis of these 3DL/2UL EN-DC NR UE, we assume the parameters and attenuation levels based on current UE RF FE components as shown in Table 1 and 2.

Table 1 show the RF component isolation parameters to derive MSD level at sub-6GHz.

Table 1: UE RF Front-end component parameters

| UE ref. architecture Component | Triplexer-Diplexer Architecture w/ single ant. |  |  |  | Cascaded Diplexer Architecture w/ single ant. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DC_3A_n8A-n78A, DC_7A_n8A-n78A DC_2A_n7A-n78A DC_13A_n7A-n78A |  |  |  | DC_13A_n48A-n66A, DC_66A_n5A-n48A DC_11A_n3A-n28A DC_1A_n8A-n40A DC_1A_n28A-n40A DC_3A_n1A-n40A DC_7A_n1A-n40A |  |  |  |
|  | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| Ant. Switch | 112 | 68 | 55 | 55 | 112 | 68 | 55 | 55 |
| Triplexer | 110 | 72 | 55 | 52 |  |  |  |  |
| Quadplexer |  |  |  |  | 112 | 72 | 55 | 52 |
| Diplexer | 115 | 87 | 55 | 55 | 115 | 87 | 55 | 55 |
| Duplexer | 100 | 75 | 55 | 53 | 100 | 75 | 55 | 53 |
| PA Forward | 28.0 | 32 | 30 | 28 | 28.0 | 32 | 30 | 28 |
| PA Reversed | 40 | 30.5 | 30 | 30 | 40 | 30.5 | 30 | 30 |
| LNA | 10 | 0 | 0 | -10 | 10 | 0 | 0 | -10 |

Table 2 show the isolation levels according to the RF component.

Table 2: UE RF Front-end component isolation parameters

| Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna to Antenna | 10 | Main antenna to diversity antenna |
| PA (out) to PA (in) | 60 | PCB isolation (PA forward mixing) |
| Triplexer | 20 | High/low band isolation |
| Diplexer | 25 | High/low band isolation |
| PA (out) to PA (out) | 60 | L-H/H-L cross-band |
| PA (out) to PA (out) | 50 | H-H cross-band |
| LNA (in) to PA (out) | 60 | L-H/H-L cross-band |
| LNA (in) to PA (out) | 50 | H-H cross-band |
| Duplexer | 50 | Tx band rejection at Rx band |

Based on these assumptions, we proposed the MSD levels as below.

Table 3: Proposed MSD test configuration and results by IMD problems

| DC bands | UL DC | IMD | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_13A_n48A-n66A | 13 | IMD3 ¦2*fB13 -fn48¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n48 |  | 3680 | 5 | 25 | 3680 | 5 |  |
|  | n66 |  | 1716 | 5 | 25 | 2116 | 5 | 14.4 |
|  | 13 | IMD5 ¦2*fB13 -3*fn66¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n66 |  | 1716 | 5 | 25 | 2116 | 5 |  |
|  | n48 |  | 3584 | 5 | 25 | 3584 | 5 | 2.8 |
| DC_66A_n5A-n48A | 66 | IMD5 ¦3*fB66 -2*fn5¦ | 1750 | 5 | 25 | 2150 | 5 | N/A |
|  | n5 |  | 834 | 5 | 25 | 879 | 5 |  |
|  | n48 |  | 3582 | 5 | 25 | 3582 | 5 | 3.3 |
| DC_11A_n3A-n28A | 11 | IMD5 ¦3*fB11 -2*fn3¦ | 1438 | 5 | 25 | 1486 | 5 | N/A |
|  | n3 |  | 1758 | 5 | 25 | 1853 | 5 |  |
|  | n28 |  | 743 | 5 | 25 | 798 | 5 | 2.5 |
| DC_1A_n8A-n40A | 1 | IMD4 ¦2*fB1 -2*fn40¦ | 1930 | 5 | 25 | 2120 | 5 | N/A |
|  | n40 |  | 2395 | 5 | 25 | 2395 | 5 |  |
|  | n8 |  | 885 | 5 | 25 | 930 | 5 | 9.2 |
| DC_1A_n28A-n40A | 1 | IMD4 ¦2*fB1 -2*fn28¦ | 1930 | 5 | 25 | 2120 | 5 | N/A |
|  | n28 |  | 743 | 5 | 25 | 798 | 5 |  |
|  | n40 |  | 2374 | 5 | 25 | 2374 | 5 | 10.1 |
|  | 1 | IMD4 ¦2*fB1 -2*fn40¦ | 1930 | 5 | 25 | 2120 | 5 | N/A |
|  | n40 |  | 2314 | 5 | 25 | 2314 | 5 |  |
|  | n28 |  | 713 | 5 | 25 | 768 | 5 | 8.6 |
| DC_3A_n1A-n40A | 3 | IMD5 ¦2*fB3 -3*fn1¦ | 1740 | 5 | 25 | 1835 | 5 | N/A |
|  | n1 |  | 1940 | 5 | 25 | 2130 | 5 |  |
|  | n40 |  | 2340 | 5 | 25 | 2340 | 5 | 3.7 |
| DC_3A_n8A-n78A | 3 | IMD3 ¦fB3 +2*fn8¦ | 1740 | 5 | 25 | 1835 | 5 | N/A |
|  | n8 |  | 900 | 5 | 25 | 945 | 5 |  |
|  | n78 |  | 3540 | 10 | 50 | 3540 | 10 | 16.3 |
| DC_7A_n1A-n40A | 7 | IMD3 ¦fB7 -2*fn40¦ | 2540 | 5 | 25 | 2660 | 5 | N/A |
|  | n40 |  | 2335 | 5 | 25 | 2335 | 5 |  |
|  | n1 |  | 1940 | 5 | 25 | 2130 | 5 | 15.2 |
| DC_7A_n8A-n78A | 7 | IMD2 ¦fB7 +fn8¦ | 2555 | 5 | 25 | 2675 | 5 | N/A |
|  | n8 |  | 900 | 5 | 25 | 945 | 5 |  |
|  | n78 |  | 3455 | 10 | 50 | 3455 | 10 | 28.5 |
|  | 7 | IMD2 ¦fB7 -fn78¦ | 2555 | 5 | 25 | 2675 | 5 | N/A |
|  | n78 |  | 3500 | 10 | 50 | 3500 | 10 |  |
|  | n8 |  | 900 | 5 | 25 | 945 | 5 | 29.7 |
| DC_2A_n7A-n78A | 2 | IMD5 ¦2*fB2 -3*fn7¦ | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n7 |  | 2525 | 5 | 25 | 2645 | 5 |  |
|  | n78 |  | 3775 | 10 | 50 | 3775 | 10 | 3.9 |
| DC_13A_n7A-n78A | 13 | IMD2 ¦fB13 +fn7¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n7 |  | 2530 | 5 | 25 | 2650 | 5 |  |
|  | n78 |  | 3312 | 10 | 50 | 3312 | 10 | 29.0 |
|  | 13 | IMD2 ¦fB13 -fn78¦ | 782 | 5 | 25 | 751 | 5 | N/A |
|  | n78 |  | 3432 | 10 | 50 | 3432 | 10 |  |
|  | n7 |  | 2530 | 5 | 25 | 2650 | 5 | 27.9 |

3	Proposed text for TR37.716-21-21:

[Unchanged Parts Skipped]

6.90	DC_13_n48-n66

6.90.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 produced by Band 13 and NR band n48 that impact the reference sensitivity of NR band n66. Also there is IMD5 produced by Band 13 and NR Band n66 that impact the reference sensitivity of NR band n48. These required MSD are shown in the following Table.

Table 6.90.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n48A-n66A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n48 | 3680 | 5 | 25 | 3680 | N/A | N/A |
|  | n66 | 1716 | 5 | 25 | 2116 | 14.4 | IMD3 |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n66 | 1716 | 5 | 25 | 2116 | N/A | N/A |
|  | n48 | 3584 | 5 | 25 | 3584 | 2.8 | IMD5 |

6.91	DC_66_n5-n48

6.91.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 produced by Band 66 and NR band n5 that impact the reference sensitivity of NR band n48. The required MSD is shown in the following Table.

Table 6.91.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_66A_n5A-n48A | 66 | 1750 | 5 | 25 | 2150 | N/A | N/A |
|  | n5 | 834 | 5 | 25 | 879 | N/A | N/A |
|  | n48 | 3582 | 5 | 25 | 3582 | 3.3 | IMD5 |

6.92	DC_11_n3-n28

6.92.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 produced by Band 11 and NR band n3 that impact the reference sensitivity of NR band n28. The required MSD is shown in the following Table.

Table 6.92.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_11A_n3A-n28A | 11 | 1438 | 5 | 25 | 1486 | N/A | N/A |
|  | n3 | 1758 | 5 | 25 | 1853 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | 2.5 | IMD5 |

6.93	DC_1_n8-n40

6.93.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 & IMD5 produced by Band 1 and NR band n40 that impact the reference sensitivity of NR band n8. The required MSD is shown in the following Table.

Table 6.93.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n8A-n40A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n40 | 2395 | 5 | 25 | 2395 | N/A | N/A |
|  | n8 | 885 | 5 | 25 | 930 | 9.2 | IMD4 |

6.94	DC_1_n28-n40

6.94.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 produced by Band 1 and NR band n28 that impact the reference sensitivity of NR band n40. Also there is IMD4 producted by Band 1 and NR band n40 that impact the reference sensitivity of NR band n28. These required MSD are shown in the following Table.

Table 6.94.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_1A_n28A-n40A | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n28 | 743 | 5 | 25 | 798 | N/A | N/A |
|  | n40 | 2374 | 5 | 25 | 2374 | 10.1 | IMD4 |
|  | 1 | 1930 | 5 | 25 | 2120 | N/A | N/A |
|  | n40 | 2314 | 5 | 25 | 2314 | N/A | N/A |
|  | n28 | 713 | 5 | 25 | 768 | 8.6 | IMD4 |

6.95	DC_3_n1-n40

6.95.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 produced by Band 3 and NR band n1 that impact the reference sensitivity of NR band n40. The required MSD is shown in the following Table.

Table 6.95.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n1A-n40A | 3 | 1740 | 5 | 25 | 1835 | N/A | N/A |
|  | n1 | 1940 | 5 | 25 | 2130 | N/A | N/A |
|  | n40 | 2340 | 5 | 25 | 2340 | 3.7 | IMD5 |

6.96	DC_3_n8-n78

6.96.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD3&IMD5 produced by Band 3 and NR band n8 that impact the reference sensitivity of NR band n78. The required MSD is shown in the following Table.

Table 6.96.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_3A_n8A-n78A | 3 | 1740 | 5 | 25 | 1835 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3540 | 10 | 50 | 3540 | 16.3 | IMD3 |

6.97	DC_7_n1-n40

6.97.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD3&IMD5 produced by Band 7 and NR band n40 that impact the reference sensitivity of NR band n1. The required MSD is shown in the following Table.

Table 6.97.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n1A-n40A | 7 | 2540 | 5 | 25 | 2660 | N/A | N/A |
|  | n1 | 2335 | 5 | 25 | 2335 | N/A | N/A |
|  | n40 | 1940 | 5 | 25 | 2130 | 15.2 | IMD3 |

6.98	DC_7_n8-n78

6.98.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2&IMD4 produced by Band 7 and NR band n8 that impact the reference sensitivity of NR band n78. Also ther are IMD2&IMD5 produced by Band 7 abd NR band n78 that impact the reference sensitivity of NR band n8. These required MSDs are shown in the following Table.

Table 6.98.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_7A_n8A-n78A | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |
|  | n8 | 900 | 5 | 25 | 945 | N/A | N/A |
|  | n78 | 3455 | 10 | 50 | 3455 | 28.5 | IMD2 |
|  | 7 | 2555 | 5 | 25 | 2675 | N/A | N/A |
|  | n8 | 3500 | 10 | 50 | 3500 | N/A | N/A |
|  | n78 | 900 | 5 | 25 | 945 | 29.7 | IMD2 |

6.99	DC_2_n7-n78

6.100	DC_13_n7-n78

6.100.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2&IMD4 produced by Band 13 and NR band n7 that impact the reference sensitivity of NR band n78. Also ther is IMD2 produced by Band 13 abd NR band n78 that impact the reference sensitivity of NR band n7. These required MSDs are shown in the following Table.

Table 6.100.5-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA / NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | IMD order |
| DC_13A_n7A-n78A | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n7 | 2530 | 5 | 25 | 2650 | N/A | N/A |
|  | n78 | 3312 | 10 | 50 | 3312 | 29.0 | IMD2 |
|  | 13 | 782 | 5 | 25 | 751 | N/A | N/A |
|  | n78 | 3432 | 10 | 50 | 3432 | N/A | N/A |
|  | n7 | 2530 | 5 | 25 | 2650 | 27.9 | IMD2 |

[End of changes]