3GPP TSG-RAN WG4 Meeting #94bis-e                                                          R4-2003118

Online, 20th April-30th April, 2020

Source:	Samsung, KDDI

Title:	TP for TR 36.716-03-02: CA_1A-18A-41A and CA_1A-18A-41C

Agenda item:			5.6.3

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 36.716-03-02 to include CA_1A-18A-41A and CA_1A-18A-41C according to the request in [1].

# 2. Reference

- RP-200510, Revised WID on Rel-16 LTE inter-band Carrier Aggregation for x bands DL (x= 3, 4, 5) with 2 bands UL.

# 3. Text Proposal

<Start of Text Proposal>

## 6.x	LTE-A inter-band CA: Band 1 and Band 18 and Band 41 DL with 2 bands UL

### 6.x.1	List of specific combination issues

#### 6.x.1.1	Channel bandwidth per operating band for CA

Table 6.x.1.1-1: CA configurations under study

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-18A-41A | CA_1A-18A CA_1A-41A CA_18A-41A | 1 |  |  | Yes | Yes | Yes | Yes | 55 | 0 |
|  |  | 18 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-18A-41C | CA_1A-18A CA_1A-41A CA_1A-41C CA_18A-41A CA_18A-41C | 1 |  |  | Yes | Yes | Yes | Yes | 75 | 0 |
|  |  | 18 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

### 6.x.2	Co-existence studies

Co-existence studies of CA_1-18-41 already covered in the constituent fall-back modes, the IMD issues for this band combination are as below list:

-  5th order IMDs generated by dual uplink of band 1 and band 41 maybe fall into the Rx of band 18, Since B18 only was used in Japan, considering Operator’s spectrum, there is no IMD impact to Band 18.

### 6.x.3	MSD

There is no MSD requirement considering the Operator’s spectrum.

### 6.x.4		ΔTIB,c and ΔRIB,c values

The TIB,c and RIB,c values of CA_1-18-41 are given in the tables below, based on the values of its consistent 2 Bands fallback mode.

Table 6.x.4-1: ΔTIB,c for 3DLs aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-18-41 | 1 | 0.5 |
|  | 18 | 0.3 |
|  | 41 | 0.5 |

Table 6.x.4-2: ΔRIB,c for 3DLs aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-18-41 | 1 | 0 |
|  | 18 | 0 |
|  | 41 | 0 |

<End of Text Proposal>