3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2003680

# Electronic Meeting, 20th Apr – 30th Apr, 2020

Title: 	Discussion on URLLC UE performance requirements for low latency for verifying pre-emption

Source: 	Huawei, HiSilicon

Agenda item:	6.9.1.2

Document for:	Discussion

Background

When considering the low latency of URLLC UE performance requirements, RAN4 agreed to verify the related features. According to the conclusion from RAN4 #94 e-meeting, three features were agreed to be verified and two of them (PDSCH mapping type B and PDSCH processing capability) were agreed to be verified together. Thus, two performance requirements will be defined for URLLC UE low latency.

- Define performance requirements for PDSCH mapping type B and PDSCH processing capability 2.

- Define performance requirements for preemption indication for eMBB UE.

In this paper, the preemption feature will be discussed. Regarding to the pre-emption feature, RAN 4 agreed that no performance requirements would be defined for URLLC UE [1].

Agreements:

- No URLLC PI performance requirements

For eMBB UE, some parameters have been agreed according to the WF [1]:

Agreements:

- Time frequency set: 14x1

- Number of symbols to be pre-empted: 2

- Starting symbol to be pre-empted: 3

- Test applicability for eMBB UE PI requirements: optional with UE capability signalling

- Antenna Configuration: 2x2 and 2x4, ULA low

There are still some open issues which will be discussed further:

Open Issues

- Pre-emption periodicity

    - Option 1: 10% probability with non-fixed scheduling in RAN4 spec (similar to CSI-RS trigger for PMI testing, also 10% probability with fixed scheduling in RAN5 spec) within 1 radio frame

    - Option 2: 1 slot

    - Option 3: 10% probability with fixed scheduling within 1 radio frame

- Channel Model

    - Option 1: TDLC300-100

    - Option 2: TDLA30-10

- FRC

    - Option 1: 16QAM with modified FRC

    - Option 2: QPSK

    - Option 3: New FRC with 64QAM

In this paper, based on the agreed parameters, the open issues are anlysed and our views are provided.

Discussion

  - Open issues

To verify the performance of UE eMBB service after being pre-emptied by the URLLC information, the BS notices the location of pre-emptied REs to eMBB UE by using Pre-emption Indication (PI).  After the eMBB UE receives the DCI, the pre-emptied REs will be flushed as there is no transmission. The eMBB UE will decode the received resources after flushing the pre-emptied REs. The eMBB signal will be retransmitted if an NACK is received.

Regarding to the pre-emption periodicity, companies have different opinions on whether to define fixed scheduling or non-fixed scheduling. From eMBB UE side, the performance will not be affected on whether the pre-empted REs are fixed scheduling or not. In order to simplify the test, we prefer to use fixed scheduling.

Proposal 1: We propose 10% probability with fixed scheduling within 1 radio frame for pre-emption periodicity.

When considering the propagation condition and coding rate. We can use a good propagation condition with a moderate coding rate for the test as we do not expect the eMBB UEs are always affected by URLLC pre-emption or there is no effect at all.

Proposal 2: We propose to use TDLA30-10 as the Channel-model and 16QAM as the coding rate.

RAN4 has agreed to use 14x1 as the time frequency set and the number of pre-empted symbol is 2, some key parameters related to this performance requirements are listed in table below:

Table 2.1-1 Key parameters for pre-emption test

| Parameters | Values |
| --- | --- |
| Time frequency set | 0 |
| TINT | 10% of the radio frame |
| μ | 0 |
| μINT | 0 |
| PI bitmap | ‘00011000000000’ |

Proposal 3: We propose the PI bitmap is ‘00011000000000’.

Proposals

In this contribution, we discussed the open issues for pre-emption. Our proposals are listed below:

Proposal 1: We propose 10% probability with fixed scheduling within 1 radio frame for pre-emption periodicity.

Proposal 2: We propose to use TDLA30-10 as the Channel-model and 16QAM as the coding rate.

Proposal 3: We propose the PI bitmap is ‘00011000000000’.

Reference

- R4-2002428, “Way forward for NR URLLC performance requirements”, Intel Corporation.