3GPP TSG-RAN WG4 Meeting #94-e-bis	R4-2003981

# Electronic Meeting, 20 – 30 Apr., 2020

Title:                    On remaining issues for inter-frequency measurement without MG

Source:                 OPPO

Agenda item:       6.15.1.5

Document for:     Discussion

Introduction

In RAN4#94-e meetings, RAN4 has approved a WF [1] on inter-frequency without MG. In this meeting, we will further discuss the following open issues, and share our opinions on those remaining issues.

Discussion

- UE capability

In last meeting, RAN4 agreed to define UE capability for UE supporting inter-frequency SSB based measurements under the condition that the SSB is completely contained in the active BWP of the UE. In our view, network can decide to always configure a measurement gap pattern for inter-frequency measurement, ignoring whether the Rel-16 UE supports the capability of inter-frequency measurement without gap or not. It is reasonable to be optional with UE capability signalling,

Proposal 1: Prefer optional with UE capability signalling.

- The UE behaviour and backward compatible issues

For Rel-15 UE, UE always uses the MG to perform inter-frequency measurement. For Rel-16 UE, UE can still use MG to perform inter-frequency measurement even if it meets the condition for measurement without gaps.

There are several scenarios for UE to consider backward compatible issue.

- Case 1: All MOs are inter-frequency with MG to UE

- Case 2: All MOs are inter-frequency w/o MG to UE

- Case 3: MOs are mixed with inter-freq w/o MG and inter-freq with MG

In our view, for Case 1, it is very clear for UE to conduct inter-frequency measurement when configured with MG.

For Case 2, if all the MOs are inter-frequency w/o MG to UE but NW configured MG, UE can use the MG to perform these inter-freq measurement if all or part of the SMTC occasions of these inter-freq MOs are overlapped by MGs.

For Case 3, if MOs are mixed with inter-freq w/o MG and inter-freq with MG, UE can measure MOs of inter-freq w/o MG outside MG, and measure MOs of inter-freq with MG inside MG. It somehow depends on UE’s implementation, which should not be specified.

Proposal 2: If all the MOs are inter-frequency w/o MG to UE but NW configured MG, UE can use the MG to perform these inter-freq measurement if all or part of the SMTC occasions of these inter-freq MOs are overlapped by MGs.

- CSSF and measurement requirements

As agreements, the definitions of CSSF within gap and outside gap are not changed. The number of searchers is up to 2 when discussing the requirement of CSSF outside gap.

- For CA capable UE, the number of searchers is 2

- For non-CA capable UE, the number of searchers is 1

For SMTC occasions of inter-frequency w/o MG partially overlapped and non-overlapped with MG, we support solution #1 to define the measurement requirements.

- If UE is non-CA capable UE, measurement in the MG will make sure PCC measurement delay not be extended. In this case, inter-frequency measurements should always happen in gaps, and define requirements based on the assumption that UE perform measurement within gaps.

- If UE is CA capable but only has PCC, measurement outside MG will not impact PCC and it can help to speed up the measurement since it doesn’t need to share measurement resource with other inter-freq within MG.  Thu, if UE is CA capable configured with 1 CC or multiple CC’s, the requirements can be defined based on the assumption that UE perform measurement outside gaps which is the same as intra-frequency measurement without MG.

Proposal 3: For non-CA capable UE, define requirements based on the assumption that UE perform measurement within gaps. For CA capable UE, define requirements based on the assumption that UE perform measurement outside gaps.

Conclusion

In this contribution, we provide our view on inter-frequency measurement without MGs.

Proposal 1: Prefer optional with UE capability signalling.

Proposal 2: If all the MOs are inter-frequency w/o MG to UE but NW configured MG, UE can use the MG to perform these inter-freq measurement if all or part of the SMTC occasions of these inter-freq MOs are overlapped by MGs.

Proposal 3: For non-CA capable UE, define requirements based on the assumption that UE perform measurement within gaps. For CA capable UE, define requirements based on the assumption that UE perform measurement outside gaps.

References

- R4-2002250, WF on inter-frequency without MG, CMCC

- R4-2000992, Further discussion on inter-frequency measurement requirement, OPPO