3GPP TSG-RAN WG4 Meeting #94bis-e	R4-2004606

Online, 20th April – 30th April 2020

Source:	Ericsson

Title:	TP for TR 38.716-02-00: CA_n78-n258

Agenda item:			8.2.3

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.716-02-00 to include CA_n78-n258 as defined in WID [1].

# 2. Text Proposal

# ---Start of changes---

## 8.6	CA_n78-n258

### 8.6.1	Common for 1 band UL and 2 bands UL CA

#### 8.6.1.1		Operating bands for CA

Table 8.6.1-1: CA band combination of band n78 + n258

| NR CA Band Combination | NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n78-n258 | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |

#### 8.6.1.2		Channel bandwidths per operating band for CA

Table 8.6.2-1: Supported bandwidths per CA band combination of band n78+n258

| NR CA configuration | Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90MHz | 100 MHz | 200 MHz | 400 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n78A-n258A | CA_n78A-n258A | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |
|  |  | n258 | 60 |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |
| CA_n78A-n258G | CA_n78A-n258A CA_n78A-n258G | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n258 | See CA_n258G Bandwidth Combination Set 0 in Table 5.5A.1-1 from 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n258H | CA_n78A-n258A CA_n78A-n258G CA_n78A-n258H | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n258 | See CA_n258H Bandwidth Combination Set 0 in Table 5.5A.1-1 from 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n258I | CA_n78A-n258A CA_n78A-n258G CA_n78A-n258H CA_n78A-n258I | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n258 | See CA_n258I Bandwidth Combination Set 0 in Table 5.5A.1-1 from 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n258J | CA_n78A-n258A CA_n78A-n258G CA_n78A-n258H CA_n78A-n258I CA_n78A-n258J | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n258 | See CA_n258J Bandwidth Combination Set 0 in Table 5.5A.1-1 from 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n258K | CA_n78A-n258A CA_n78A-n258G CA_n78A-n258H CA_n78A-n258I CA_n78A-n258J CA_n78A-n258K | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n258 | See CA_n258K Bandwidth Combination Set 0 in Table 5.5A.1-1 from 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n258L | CA_n78A-n258A CA_n78A-n258G CA_n78A-n258H CA_n78A-n258I CA_n78A-n258J CA_n78A-n258K CA_n78A-n258L | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n258 | See CA_n258L Bandwidth Combination Set 0 in Table 5.5A.1-1 from 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n78A-n258M | CA_n78A-n258A CA_n78A-n258G CA_n78A-n258H CA_n78A-n258I CA_n78A-n258J CA_n78A-n258K CA_n78A-n258L CA_n78A-n258M | n78 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n258 | See CA_n258M Bandwidth Combination Set 0 in Table 5.5A.1-1 from 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 8.6.1.3		Co-existence studies

Table 8.6.3-1 gives the UL 2nd, 3rd, 4th, 5th, 6th, 7th harmonic for CA_n78A-n258A. The 7th harmonic produced by UL band n78 may fall into Band n258 DL.

Table 8.6.1.3-1: Band n78 and Band n258 UL harmonics products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency(MHz) | 3300 | 3800 | 24250 | 27500 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2*fy_low | 2*fy_high |
| 2nd harmonics frequency limits(MHz) | 6600 | 7600 | 48500 | 55000 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3*fy_low | 3*fy_high |
| 3rd harmonics frequency limits(MHz) | 9900 | 11400 | 72750 | 82500 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits(MHz) | 13200 | 15200 | 97000 | 110000 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5*fy_low | 5*fy_high |
| 5th harmonics frequency limits(MHz) | 16500 | 19000 | 121250 | 137500 |
| 6th harmonics frequency limits | 6*fx_low | 6*fx_high | 6*fy_low | 6*fy_high |
| 6th harmonics frequency limits(MHz) | 19800 | 22800 | 145500 | 165000 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7*fy_low | 7*fy_high |
| 7th harmonics frequency limits(MHz) | 23100 | 26600 | 169750 | 192500 |

#### 8.6.1.4		∆TIB and ∆RIB values

For CA_n78A-n258A, the TIB,c and RIB values are given in the tables below.

Table 8.6.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n78-n258 | n78 | 0 |
|  | n258 | 0 |

Table 8.6.4-2: ΔRIB

| Inter-band CA Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n78-n258 | n78 | 0 |
|  | n258 | 0 |

#### 8.6.1.5		REFSENS requirements

There is no specific REFSENS requirement.

### 8.6.2	Specific for 2 bands UL CA

#### 8.6.2.1		UE co-existence studies

Table 8.6.2.1-1 lists Band n78 + Band n258 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

# Table 8.6.2.1-1: Band n78 and Band n258 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_high – fx_low¦ | ¦fy_low – fx_high¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 24200 | 20450 | 27550 | 31300 |
| 3rd order IMD products | ¦fy_high – 2*fx_low¦ | ¦fy_low – 2*fx_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 20900 | 16650 | 44700 | 51700 |
| 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 30850 | 35100 | 51800 | 58800 |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high – 2*fy_low¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 48400 | 40900 | 55100 | 62600 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 17600 | 12850 | 68950 | 79200 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high +1* fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 34150 | 38900 | 76050 | 86300 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 106700 | 93200 | 9050 | 14300 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 100300 | 113800 | 37450 | 42700 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 75900 | 65150 | 37100 | 45100 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 79350 | 90100 | 58400 | 66400 |

Based on Table 8.6.2.1-1 there are no IMD issues affecting own Rx frequencies.

#### 8.6.2.2		REFSENS requirements

Based on the co-existence studies in 8.6.2.1, there is no need to define MSD for CA_n78-n258.

---End of changes---

# Reference

[1] 		RP-200168, “Revised WID on Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2)”, ZTE Corporation