3GPP TSG-RAN WG4 #94bis-e	R4-2004725

April 20th ‒ 30th, 2020

Electronic Meeting

Agenda item:	6.1.2.1

Source: 	Qualcomm Incorporated

Title: 	NR-U MPR for PC5 single carrier and wideband

Document for:	Discussion

# Introduction

In [1] simulation results were presented to derive MPR for single carrier NR-U with maximum output power according to PC5.  This contribution is an update to [1] to also include results for wideband operation with one or more 20 MHz sub-bands. The MPR values proposed in this contribution for the NR-U interlaced waveform are based on a WiFi PA model for PC5.  The emission requirements are taken from recent agreements with the exception of in-band emissions which for which discussion began at the last meeting [2] with a proposal this meeting for further study [3].  EVM is also partitioned between transceiver and PA.  Finally, an MPR table is provided for consideration.  In this contribution, only interlaced waveforms are studied.  While RAN1 specifications permit an NR uplink waveform for regions where a spectral occupancy requirement is not applicable, its MPR has not been studied.

# Discussion

## PA model

A WiFi PA model has been used to generate the simulation results in this contribution.  The approach taken in this contribution is to characterize the maximum performance achievable by existing WiFi PA’s in order to enable their usage for NR-U applications, at least for power class 5.  This approach deviates slightly from the conventional approach of defining requirements starting from the power class for a purpose-built PA, but is motivated by the desire to enable reuse of existing PA’s that are already found on many phone designs today.

The method used to calibrate the PA against manufacturing process and temperature variation to extrapolate performance for worst case specifications is described in [4].  In short, the PA is calibrated against a WiFi requirement which is expected to bound its linearity with an assumption that a worst case PA would just meet this requirement at its maximum output power.  In the PA studied, it was found that 2 dB could be used to extrapolate the worst case behavior of this PA across process and temperature.  Moreover, it is assumed that the worst case front-end loss is 3.5 dB between the PA output and the antenna port.  This figure was obtained from studying various WiFi designs at 5 GHz and may be slightly underestimated since the loss was measured evaluated at the output of the front-end module rather than the PA itself.  Nevertheless, considering the 2 dB offset for process/temp and the 3.5 dB front-end loss, 5.5 dB is subtracted from the modeled PA output to obtain the power level at the antenna port.  Note that any absolute emissions (dBm) should also have at least the 3.5 dB front-end loss subtracted as well, but the emissions used in this study were relative emissions (dBr) where the front-end loss is normalized out.

## Emission requirements

The following emission requirements are included in the simulations for PC5

- Single carrier SEM in accordance with the figure in slide 3 of [5].  Measurement bandwidth in the 0 to 1 MHz transition from 0 dBr to -20 dBr was 100 kHz and compared against a requirement scaled to 100 kHz bandwidth.  The mask was evaluated in (near)-continuously swept increments.

- Wideband SEM in accordance with the figure in slide 8 of [5].  From previous agreement [8], only 20 MHz sub-bands that are contiguously spaced within the wideband channel are considered for Rel-16.  Therefore, the following sub-band configurations were simulated

| Channel bandwidth | 20 MHz sub-band configurations |
| --- | --- |
| 40 MHz | 01 |
| 60 MHz | 001, 011, [010] |
| 80 MHz | 0001, 0010, 0011, 0110, 0111 |

The notation in the table is a bitmap according to 20 MHz sub-bands where ‘1’ indicates that the sub-band is scheduled and allocated while ‘0’ indicates that it is not.  Symmetry about the center of the wideband channel is assumed in order to reduce the number of configurations for simulation.  In other words, it is assumed that ‘01’ is equivalent to ‘10’.  For the 60 MHz bandwidth, the configuration ‘010’ was overlooked in simulation but is not expected to change the final result due to its similarity with ‘001’ or ‘0010’.

- ACLR was simulated for 26 and 27 dB, with a tentative agreement of [27] dB documented in [2].

- Spurious emissions are the same as for NR; that is, -30 dBm/MHz beyond CBW+5 MHz from the channel edge

- EVM partitioned to the PA using an approach similar to [6] and [7] as follows.  EVM was not indirectly estimated by time domain or frequency domain spectral analysis, but was computed directly per tone after symbol decoding in the modulation domain.

| Modulation | Total transmitter (%) | Partitioned to PA (%) |
| --- | --- | --- |
| QPSK | 17.5 | 8 |
| 16QAM | 12.5 | 7.5 |
| 64QAM | 8 | 4 |
| 256QAM | 3.5 | 1.5* |
| *For 256QAM, the agreement was 1.8% partitioned to the PA but the simulations have not been rerun to reflect this yet. It is not expected to materially change the MPR results for 256QAM. |  |  |

- In-band emissions have not yet been agreed and are not included in these simulations.

## Baseline waveform

It has been suggested that a baseline waveform be proposed to be commonly used by all companies as a first step to align simulation results.  The suggested reference waveform is described as

QPSK CP-OFDM and DFT-S-OFDM, 20 MHz, 15 kHz SCS, fully allocated and single interlace

A 20 MHz channel holds up to 106 RB’s according to NR spectrum utilization, where these RB’s are located at the center of the channel.  For CP-OFDM, all 106 RB’s can be assigned but for DFT-S-OFDM, the length of the allocation is limited to 2^a*3^b*5^c, but mapping is still according to the 106 RB’s of CP-OFDM.  For fully allocated CP-OFDM waveforms, all 106 RB’s are allocated but for fully allocated DFT-S waveforms, only 100 RB’s are allocated.  These 100 RB’s are placed left justified in the 106 RB’s mapping so can be viewed as occupying RB’s 0 to 99 where RB’s 100 to 105 are empty.  The interlace-0 waveform is mapped as RB locations {0, 10, 20, 30, 40 … 100} with 11 active RB’s for CP-ODFM and RB locations {0, 10, 20, 30, 40 … 90} with 10 active RB’s for DFT-S-OFDM.

The simulation results for this waveform are shown below in Table 1.  The table shows the maximum output power referred to the antenna port while simultaneously meeting the emission requirement.  The maximum output power is reported separately for each emission requirement to provide detail.  The SEM is decomposed into Region 1 (0 to -20 dBr) and Region 2 (-20 dBr to -28 dBr).  However, the minimum of these output powers across all emission requirements is the maximum output power this PA is able to achieve while simultaneously meeting all emission requirements.

Table 1.  Maximum output power and MPR for baseline waveform

| ChBW | SCS | Modulation | DFT/CP | Allocation | Achievable maximum output power (dBm) |  |  |  | PC5 MPR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | EVM | ACLR (27 dB) | SEM Region 1 | SEM Region 2 |  |
| 20 | 15 | QPSK | CP | Interlace_0 | 16.3 | 16.7 | 20.2 | 16.5 | 3.7 |
| 20 | 15 | QPSK | DFT-S | Interlace_0 | 17.9 | 18.0 | 20.5 | 18.0 | 2.1 |
| 20 | 15 | QPSK | CP | Full | 16.0 | 16.9 | 17.7 | 16.9 | 4.0 |
| 20 | 15 | QPSK | DFT-S | Full | 18.7 | 18.8 | 19.5 | 18.8 | 1.3 |

It can be seen that the maximum output power is always limited by EVM for these waveform from this PA.  However, if EVM is removed as a criterion, then for the first, second, and fourth waveforms, there is only about 0.1 dB difference in maximum output power as ACLR (27 dB) and SEM2 quickly become limiting.  For the third waveform without EVM, the difference is more significant at 0.9 dB.  It is also seen that even the most benign waveform (DFT-S-OFDM with full allocation) requires 1.3 dB power backoff which will be rounded to 1.5 dB MPR.  A fully allocated QPSK waveform for NR requires 1 dB MPR, so the 1.5 dB shown here is not out-of-line.  However, it does mean that there is no 0 dB MPR waveform available for PC5.

## Single carrier simulation result

A more exhaustive simulation suite was run to derive an MPR table.  In addition to the baseline parameters described above, the following aspects were also simulated.

All possible allocations for interlaced waveforms were simulated initially.  However, it was found that the full allocation and interlace-0 allocation waveforms could bound the spread of MPR values.  Moreover, the spread of MPR values was not found to be significant to warrant further dissecting the allocation to identify waveforms with mid-point MPR.  Therefore, simulations were only run for these two waveforms recognizing also that eLAA MPR was also derived based on two similar waveforms.

Modulations simulated were QPSK, 16QAM, 64 QAM, and 256 QAM.

Channel bandwidths simulated were 20 MHz, 40 MHz, 60 MHz, and 80 MHz.  Results for wideband operation with partial allocation of sub-bands are in the next section of this report.

Table 2.  Simulation scenarios for all CBW/SCS

| Scenario | Modulation | DFT/CP | Allocation |
| --- | --- | --- | --- |
| 1 | QPSK | CP | Interlace_0 |
| 2 | QPSK | DFT-S | Interlace_0 |
| 3 | QPSK | CP | Full |
| 4 | QPSK | DFT-S | Full |
| 5 | 16QAM | CP | Interlace_0 |
| 6 | 16QAM | DFT-S | Interlace_0 |
| 7 | 16QAM | CP | Full |
| 8 | 16QAM | DFT-S | Full |
| 9 | 64QAM | CP | Interlace_0 |
| 10 | 64QAM | DFT-S | Interlace_0 |
| 11 | 64QAM | CP | Full |
| 12 | 64QAM | DFT-S | Full |
| 13 | 256QAM | CP | Interlace_0 |
| 14 | 256QAM | DFT-S | Interlace_0 |
| 15 | 256QAM | CP | Full |
| 16 | 256QAM | DFT-S | Full |

Figure 1.  MPR simulation results and proposal

Simulation scenarios are listed in Table 2 with MPR simulation results shown in Figure 1.  In the figure, the MPR required to meet all emission requirements is shown for each simulation scenario.  Different channel bandwidths and SCS are overlaid on top of each other; from the overlaid curves it can be seen that the performance is nearly agnostic to channel bandwidth and SCS.  This is due to the observation that the MPR is mostly dominated by EVM which is insensitive to channel bandwidth and SCS.  Based on these results, an MPR table can be constructed.

The approach used in this contribution to building the MPR table is to define a baseline MPR with offsets for more challenging waveform attributes.  All values are specified with 0.5 dB resolution.

Table 3.  Proposed MPR table for single carrier PC5 NR-U

| Waveform | MPR and offset |
| --- | --- |
| Baseline (QPSK, DFT-S, full allocation) | 2 dB |
| CP-OFDM + full allocation | +2.5 dB |
| CP-OFDM + any other allocation | +2 dB |
| DFT-S + any other allocation | +0.5 dB |
| 16QAM | +0.5 dB* |
| 64QAM | +2 dB |
| 256QAM | +3.5 dB |
| *It was found that the MPR offset for 16QAM is required to be 1 dB in wideband operation described below. |  |

For example, a 16QAM waveform, CP-OFDM, partial interlace would receive 2 dB (baseline) + 2 dB (CP-OFDM + any other allocation) + 0.5 dB (16QAM) for a total of 4.5 dB MPR.  This MPR table is illustrated as the top curve in Figure 1.  The minimum MPR is 2 dB while the maximum MPR is 8 dB.

## Wideband carrier simulation result

For wideband, the same set of 16 scenarios was run but with the various sub-band configurations as described above.

Figure 2.  Wideband MPR simulation results and proposal

The results for wideband in Figure 2 are strikingly similar to those for single carrier shown above in Figure 1.  One reason is that the sub-bands are only contiguously allocated and the SEM requirement is largely the same as it is for single carrier.  Moreover, as described above, the MPR was found to be limited by EVM which of course is primarily a function of modulation rather than distinguished between single carrier and wideband.  As also observed for single carrier, even with EVM removed as a criterion for MPR, for lower order modulations, the ACLR of 27 dB or SEM region 2 (-20 dBr to -28 dBr) quickly becomes limiting.  For some of the 16 QAM scenarios, it was found that the single carrier MPR was failing.  Therefore, the 16QAM MPR is adjusted by an additional 0.5 dB to arrive at the proposal for MPR below, and as shown in the uppermost trace in Figure 2.

Table 4.  Proposed MPR table for single carrier and wideband PC5 NR-U

| Waveform | MPR and offset |
| --- | --- |
| Baseline (QPSK, DFT-S, full allocation) | 2 dB |
| CP-OFDM + full allocation | +2.5 dB |
| CP-OFDM + any other allocation | +2 dB |
| DFT-S + any other allocation | +0.5 dB |
| 16QAM | +1 dB |
| 64QAM | +2 dB |
| 256QAM | +3.5 dB |

# Conclusion

This contribution provides simulation results of MPR for NR-U.  The basis for the simulation results is described in terms of PA modeling and emission requirements where the approach is to enable the reuse of WiFi PA’s for the PC5 NR-U application.  Detailed results for a baseline waveform have been provided to align results with those from other companies.  Finally, the results of an exhaustive simulation campaign are presented along with an MPR table proposal.  It is to be noted that the results in this contribution do not include in-band emission requirements and are relevant to both single carrier NR-U waveforms (all sub-bands allocated) as well as wideband waveforms where partial 20 MHz sub-bands may be allocated.  These allocated sub-bands are assumed to be contiguous within the wideband channel.  Also, conventional NR waveforms not interlaced are also not included in these results.

# Reference

- R4-2002094, “NR-U MPR for PC5 single carrier,” Qualcomm Incorporated

- R4-2002753, “WF on remaining UE Tx requirements for NR-U,” Qualcomm Incorporated, Skyworks Solutions, Inc.

- R4-2004724, “NR-U In-band emission requirement,” Qualcomm Incorporated

- R4-1907961, “NR-U PA calibration,” Qualcomm Incorporated

- R4-1915979, “WF on NR-U spectrum emission mask,” Nokia, CableLabs, Charter

- R4-150975, “EVM and MPR requirements in support of 64QAM UL,” Qualcomm Incorporated

- R4-163852, “TX Challenges for 256 QAM,” Qualcomm Incorporated

- R4-1912866, “LS on UL wideband operation for NR-U,” LS from RAN4 to RAN1