3GPPRAN4#94-e-Bis	R4-2004785

Online Meeting, April 20th – 30th  , 2020

Title: 	Further discussion on inter-frequency measurements without gaps

Source: 	Qualcomm Incorporated

Agenda item:	6.15.1.5 	Inter-frequency measurement requirement without MG	[NR_RRM_Enh_Core]

Document for:	Discussion

# Introduction

In RAN4#94-e, a WF [1] was agreed on the topic of inter-frequency measurement without gaps. The open items as listed in the WF are

- UE capability

- UE requirements on when configured MG is partially overlapped with inter-frequency SMTC

  - Option 1: Define requirements based on assumption that UE does measurements outside gaps

  - Option 2: Define requirements based on assumption that UE does measurements within gaps

- UE assumes synchronization between partially and fully overlapping TDD carriers.

  - FFS on how UE can be made aware of the sync condition.

# Discussion

RAN4 assumption while defining measurement requirements has been that the UE has two searchers. This requirement is valid for UE’s that support CA. However, for a UE that does not support CA, the two-searcher requirement does not hold.

Observation 1: A UE that does not support CA will have only one searcher.

Proposal 1: For UE that does not support CA, inter-frequency measurements should always happen in gaps.

While we still have concerns on both SCell measurements being de-prioritized and UE power, in order to make progress on this topic, we can agree to a compromise that when SMTC is partially overlapped with MG the UE should perform these measurements outside gap. Note that the above holds only for CA capable UE’s. For a UE that does not support CA, proposal 1 still holds.

Proposal 2: For inter-frequency SMTC’s that partially overlap with gaps, a UE capable of NR CA should perform inter-frequency measurements that lie within active BWP outside gaps

In addition, we would like to clarify the language used in one of the agreements in the previous WF which states the following

“If all the MOs are inter-freq w/o MG to this UE but NW configured MG, UE will use the MG to perform these inter-freq measurement; “

Note that the measurement gaps should only be the case if the SMTC is completely overlapped with gaps. If the SMTC is not fully overlapped the UE can keep the Rel-16 behavior the measure outside gaps. There was a concern on backward compatibility, but the older base stations would in any case need to account for this new behavior for cases where UE is being asked to measure MO’s which have a mix of inter-frequency measurement with and without gap.

Proposal 3: Clarify the language of previous agreement to include

If all the MOs are inter-freq w/o MG to this UE but NW configured MG, UE will use the MG to perform these inter-freq measurement if the SMTC are completely overlapped with MG.

## UE capability

Since this is an enhancement feature, it should be left up to the UE to decide if it wants to support this feature. Hence the feature should be optional with capability signaling.

Proposal 4: The capability associated with this feature should be optional with capability signaling.

# Conclusions

Observation 1: A UE that does not support CA will have only one searcher.

Proposal 1: For UE that does not support CA, inter-frequency measurements should always happen in gaps.

Proposal 2: For inter-frequency SMTC’s that partially overlap with gaps, a UE capable of NR CA should perform inter-frequency measurements that lie within active BWP outside gaps

Proposal 3: Clarify the language of previous agreement to include

If all the MOs are inter-freq w/o MG to this UE but NW configured MG, UE will use the MG to perform these inter-freq measurement if the SMTC are completely overlapped with MG.

Proposal 4: The capability associated with this feature should be optional with capability signaling.

# References

[1] R4-2002250, WF on inter-frequency without MG.