3GPP TSG-RAN WG4 Meeting #94bis-e    	R4-2004920

Electronic Meeting, 20th April– 30th April, 2020

Agenda item:	6.18.2.1

Source:	Samsung

Title:	Updated simulation results for Rel-16 performance enhancement

Document for:	Discussion

# 1	Introduction.

During the last RAN4 meeting, the NR performance requirement enhancement for Rel-16 was further discussed. Test parameters for 30%TP requirement are agreed [1].

In this contribution, the simulation results are provided for requirements finalization.

# 2	Discussion and simulation results

In this section, the ideal and impairment simulation results are provided for requirements finalization as indicated in table 1 and table 2.

Table 1 ideal and impairment simulation results for 30%TP requirement in FR1

| SCS BW | Antenna configuration | MCS | Channel | DMRS configuration | Resource allocation | Ideal SNR (dB)(30%) | Impairment SNR (dB) (30%) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 5MHz | 1x2 | MCS16 | TDLC 300-100Hz | 1+1 (2,11) | Type A | 0.87 | 2.87 |
| 15KHz, 5MHz | 1x2 | MCS16 | TDLC 300-100Hz | 1+1(2,11) | Type B | 0.85 | 2.85 |
| 30KHz, 10MHz | 1x2 | MCS16 | TDLC 300-100Hz | 1+1(2,11) | Type A | 0.93 | 2.93 |
| 30KHz, 10MHz | 1x2 | MCS16 | TDLC 300-100Hz | 1+1(2,11) | Type B | 0.92 | 2.92 |

Table 2, Ideal and impairment simulation results for 30%TP requirement in FR2

| SCS BW | Antenna configuration | MCS | Channel | DMRS configuration | Resource allocation | PTRS | Ideal SNR (dB)(30%) | Impairment SNR (dB) (30%) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60KHz, 50MHz | 1x2 | MCS16 | TDLA 30-300Hz | 1(0) | Type B | KPTRS =2, LPTRS =1 | 1.57 | 4.07 |
| 60KHz, 50MHz | 1x2 | MCS16 | TDLA 30-300Hz | 1(0) | Type B | Without PTRS | 1.27 | 3.77 |
| 60KHz, 50MHz | 1x2 | MCS16 | TDLA 30-300Hz | 1+1(0, 8) | Type B | KPTRS =2, LPTRS =1 | 1.32 | 3.82 |
| 60KHz, 50MHz | 1x2 | MCS16 | TDLA 30-300Hz | 1+1(0,8) | Type B | Without PTRS | 0.97 | 3.47 |
| 120KHz, 50MHz | 1x2 | MCS16 | TDLA 30-300Hz | 1(0) | Type B | KPTRS =2, LPTRS =1 | 1.73 | 4.23 |
| 120KHz, 50MHz | 1x2 | MCS16 | TDLA 30-300Hz | 1(0) | Type B | Without PTRS | 1.30 | 3.80 |
| 120KHz, 50MHz | 1x2 | MCS16 | TDLA 30-300Hz | 1+1(0,8) | Type B | KPTRS =2, LPTRS =1 | 1.38 | 3.88 |
| 120KHz, 50MHz | 1x2 | MCS16 | TDLA 30-300Hz | 1+1(0,8) | Type B | Without PTRS | 0.98 | 3.48 |

# 3	Conclusion

In this contribution, the simulation results are provided for requirement finalization.

# References

RAN4_93_RD_chairman_report_12_EOM

R4-2002397 WF on PUSCH demodulation requirements for 30% Throughput, NTT DOCOMO