3GPP TSG-RAN4#94bis-e	R4-2004998

20th Apr – 30th Apr 2020

Source: 	Nokia

Title: 					Usage of n77 in US C-band

Agenda Item: 			4.4.1.1

Document for:	Approval

# 1		Introduction

FCC has recently decided to allocate more spectrum for 5G use on C-band, see Fig 1.

Figure 1: New 5G allocation in US

Accelerated clearing of the band could happen in two phases, see Fig 2.

Figure 2: Time schedule

As this C-band frequency range is totally within n77 we discuss in this paper is it possible to use n77 in US.

# 2	Discussion

Typically, US bands have dedicated spectral emission mask regulated by FCC. However, in the case of C-band FCC has worked with industry to create a SEM that is compatible with NR general emissions mask [1] to allow companies to benefit from economics of scale as by using n77 also in US the band becomes global. Furthermore, this means no A-MPR is necessary.

Even in the case that no new SEM or A-MPR is needed the UE to UE co-existence table needs modification because n77 needs to protect US bands and vice versa.

Proposal: RAN4 needs to decide in this meeting if n77 can be used in US with the necessary modification of UEtoUE co-ex table in-order to prepare necessary CR to next RAN4 meeting.

If RAN4 decides to use n77 for US we can prepare the CR for the May meeting.

# 3	Conclusion

In this contribution we have discussed what are the needed changes in 38.101-1 if n77 is taken into use in US. These changes are modification of UE to UE co-existence table. And we have made following proposal.

Proposal: RAN4 needs to decide in this meeting if n77 can be used in US with the necessary modification of UEtoUE co-ex table in-order to prepare necessary CR to next RAN4 meeting.

# 4	References

[1] https://docs.fcc.gov/public/attachments/FCC-20-22A1.pdf