3GPP TSG-RAN WG4 Meeting #94-e-bis	R4-2005515

# Electronic Meeting, 20 – 30 April, 2020

Title: 	Simulation assumptions for LTE based 5G terrestrial broadcast performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.14.2

Document for:	Approval

Background

In this contribution, we summarize the detail simulation assumptions for alignment for LTE-based 5G terrestrial broadcast demodulation requirements based on the way forward [1] and [2].

PMCH

Test parameters

SCS:

- Rooftop scenario: 0.37 kHz

- Mobility scenario: 2.5 kHz

Channel bandwidth: 10MHz

Non-MBMS configuration: Allocated in first subframe within 40ms

PMCH configuration:

- Rooftop scenario: 13 3ms-slots every 40 ms.

- Mobility scenario: 39 subframes every 40ms.

MBSFN RS type for rooftop scenario: Type 1 and Type 2.

FRC:

- Rooftop scenario: Rank 1 with MCS 21 (64QAM)

- Mobility scenario: Rank 1 with MCS 12 (16QAM)

Smallest allocation unit considered in RAN4 demod requirement:

- Rooftop scenario:

  - 4 symbols for MBSFN RS type 1

  - 2 symbols for MBSFN RS type 2

- Mobility scenario: 2 symbols

Timing/frequency tracking loops: use the smallest allocation unit.

Antenna configuration:

- Rooftop scenario: 1x1

- Mobility scenario: 1x2 low

BLER test point: 1% of BLER.

FRC

Rooftop scenario

Table A.3.8.1-x: Fixed Reference Channel for subcarrier spacing 0.37 kHz with 5G terrestrial broadcast MBMS dedicated cell

| Parameter | PMCH |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unit | Value |  |  |  |  |  |
| Reference channel |  |  | R.10x-1 FDD | R.10x-2 FDD |  |  |  |
| Channel bandwidth | MHz |  | 10 | 10 |  |  |  |
| Allocated resource blocks |  |  | 50 | 50 |  |  |  |
| Allocated slots per 40ms(Note1) |  |  | 13 | 13 |  |  |  |
| Modulation |  |  | 64QAM | 64QAM |  |  |  |
| Target Coding Rate |  |  | 0.48 | 0.52 |  |  |  |
| Information Bit Payload (Note 2) |  |  |  |  |  |  |  |
| For each slot | Bits |  | 63776 | 63776 |  |  |  |
| Number of Code Blocks per Sub-Frame (Note 3) |  |  | 11 | 11 |  |  |  |
| Binary Channel Bits Per Subframe |  |  |  |  |  |  |  |
| For each slot | Bits |  | 133650 | 121500 |  |  |  |
| MBMS UE Category |  |  | TBD | TBD |  |  |  |
| Note 1: First subframe of every 40ms is allocated for non-MBMS transmission. Note 2: Zero OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |  |  |

Mobility scenario

Table A.3.8.1-y: Fixed Reference Channel for subcarrier spacing 2.5 kHz with 5G terrestrial broadcast MBMS dedicated cell

| Parameter | PMCH |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unit | Value |  |  |  |  |  |
| Reference channel |  |  | R.10x FDD |  |  |  |  |
| Channel bandwidth | MHz |  | 10 |  |  |  |  |
| Allocated resource blocks |  |  | 50 |  |  |  |  |
| Allocated subframes per 40ms(Note1) |  |  | 39 |  |  |  |  |
| Modulation |  |  | 16QAM |  |  |  |  |
| Target Coding Rate |  |  | 0.46 |  |  |  |  |
| Information Bit Payload (Note 2) |  |  |  |  |  |  |  |
| For each subframes | Bits |  | 9912 |  |  |  |  |
| Number of Code Blocks per Sub-Frame (Note 3) |  |  | 2 |  |  |  |  |
| Binary Channel Bits Per Subframe |  |  |  |  |  |  |  |
| For each subframes | Bits |  | 21600 |  |  |  |  |
| MBMS UE Category |  |  | ≥ 2 |  |  |  |  |
| Note 1: First subframe of every 40ms is allocated for non-MBMS transmission. Note 2: Zero OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [4]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |  |  |  |  |

Propagation condition

Rooftop scenario

| Extended Delay Spread |  |
| --- | --- |
| Zero Doppler frequency |  |
| Relative Delay [μs] | Relative Mean Power [dB] |
| 0 | -11 |
| 130 | -10 |
| 220 | -4.5 |
| 240 | -3.5 |
| 400 | 0 |
| 520 | -13 |
| 650 | -20 |
| 800 | -25 |

Mobility scenario

| Extended Delay Spread |  |
| --- | --- |
| Maximum Doppler frequency [162Hz] |  |
| Relative Delay [ns] | Relative Mean Power [dB] |
| -310 | -3.6 |
| -280 | -1.5 |
| -160 | -1.4 |
| 0 | 0 |
| 60 | -0.6 |
| 780 | -7.0 |
| 49650 | -10 |
| 49680 | -11.5 |
| 49800 | -11.4 |
| 49960 | -13.6 |
| 50020 | -10.6 |
| 50740 | -17.0 |
| 109650 | -20 |
| 109680 | -21.5 |
| 109800 | -21.4 |
| 109960 | -23.6 |
| 110020 | -20.6 |
| 110740 | -27.0 |

PBCH repetition in CAS

Test parameters

Transmission bandwidth: 10MHz

PBCH Bandwidth: 1.4MHz

CAS configuration: Periodicity 160 ms, allocated in first subframe within 40ms, number of transmissions is 4.

FRC: QPSK, Target CR=40/4416

Propagation condition & Antenna configuration:

- AWGN for 1x1

- EVA162Hz for 1x2 low

Throughput test point: 1% Pm-bch

FRC

Table 3.2-1: Reference Channel FDD/TDD

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.106 |
| Number of transmitter antennas |  | 1 |
| Channel bandwidth | MHz | 1.4 |
| Modulation |  | QPSK |
| Target coding rate |  | 40/4416 |
| Payload (without CRC) | Bits | 24 |

PBCH repetition pattern

Table 6.6.4.1-1: Slot and symbol number pair for repetition of PBCH [TS 36.211].

| l | Slot and symbol number pair ns',l' |
| --- | --- |
|  | Normal cyclic prefix |
| 0 | (0, 4) |
| 1 | (1, 4) |
| 2 | (1, 5) |
| 3 | (0, 3), (1, 6) |

Reference

R4-2002423, WF for 5G broadcast demod requirement, RAN4#94e, Qualcomm

R4-2005516, WF for 5G broadcast demod requirement, RAN4#94e-bis, Qualcomm