3GPP TSG-RAN WG4 Meeting # 95-e 	R4-2006272

Electronic Meeting, 25 May – 5 June, 2020

Title: 	TP for TR 38.809: Transmit ON/OFF power

Source: 	CATT

Agenda item:	6.5.2.1.3

Document for:	Approval

- Introduction

This contribution provides TP for TR 38.809 based on the RAN4 discussion and agreements.

- Reference

[1] R4-2002495, “WF on IAB-MT Tx power requirements”

[2] R4-2005438, “Email discussion summary for [94e Bis][208] NR_IAB_RF_Part_2”

Annex: TP for IAB TR

<First part>

7.4	Transmit ON/OFF power

Transmit ON/OFF power requirements include two parts: transmitter off power and transmitter transient period. For IAB-DU, it’s straightforward to reuse BS’s both requirements because IAB-DU behaviour is the same with BS.

For IAB-MT, there were proposals to reuse BS or UE requirements for both OFF power and transient period. Comparing BS OFF power and UE OFF power requirements, BS FR1 requirements are more stringent than UE but FR2 visa verse. After RAN4 discussion, the understanding is that there’s no problem if MT reuses BS OFF power requirements. Regarding transient period requirements, the general profiles for BS and UE are similar but FR2 BS transient period length is shorter than UE. UE has many specific profiles requirements to explicitly define the transient period location for different physical layers and different test cases. After discussion, the understanding is that IAB-MT can meet BS more stringent transient period length requirements and UE specific profiles are not needed for IAB-MT. IAB-MT should know UE behaviour when different physical channels are transmitted.

Based the above discussion and analysis, both IAB-DU and IAB-MT reuse BS ON/OFF power requirements including the OFF power and transient period.

<Next part>

9.5	OTA transmit ON/OFF power

According to the analysis in 7.4, both IAB-DU and IAB-MT reuse BS radiated ON/OFF power requirements including the OFF power and transient period.

<End of the TP>