3GPP TSG-RAN WG4 Meeting # 95-e 	R4-2007058

Electronic Meeting, 25 May – 5 June, 2020

Source:		Ericsson

Title:			Proposal for EMC reduction of test configurations

Agenda item:		4.8.3

Document for:	Discussion

# Introduction

Current testing of EMC requirements as specified in TS 37.113 [1] (MSR BS) and TS 37.114 [2] (AAS BS) is increasingly complex and considers many test configurations, due to various possible RAT combinations.

In this contribution we want to present some considerations on the need for reduction of test configurations, and present an initial proposal on how to achieve the reduction of the test configurations.

# Discussion

## 3GPP standards situation

## According to TS 37.113 [1], an MSR BS Base station is “characterized by the ability of its receiver and transmitter to process two or more carriers in common active RF components simultaneously in a declared RF bandwidth, where at least one carrier is of a different RAT than the other carrier(s).”

Observation 1: One characteristic of MSR BS is the ability of receiver and transmitter to process two or more carriers in common active RF components simultaneously.

Standards such as TS 37.113 [1] and TS 37.114 [2] define the BS test configurations that shall be used for demonstrating EMC conformance. These BS test configurations (TCs) are applied according to the declared RAT Capability Set (CS, CSA or RCSA) of the Base Station (MSR or AAS) and the Band Category of the declared operating band (i.e. BC1, BC2 or BC3). The TCs for MSR BS are defined in TS 37.141 [3], and include 19 different capability sets (including 3 of single mode condition: WCDMA, LTE and NB-IoT standalone), which cover the possible RAT combinations available in a MSR BS. This makes it necessary to repeat several times testing procedures in order to cover all the possible combinations.

As an example, CS19, which supports UTRA, E-UTRA and NR RATs requires for an EMC test to cover the following configurations:

| Capability Set supported by the BS | Supported RATs | Supported configurations | MR Configuration |
| --- | --- | --- | --- |
| CS19 | UTRA, E-UTRA3, NR4 | SR UTRA (SC, MC) SR E-UTRA3 (SC, MC, CA) SR NR4 (SC, MC, CA) MR UTRA + E-UTRA3 MR UTRA + NR4 MR E-UTRA3 + NR4 MR UTRA + E-UTRA3 + NR4 | WCDMA + LTE+ IoT (optional) WCDMA + NR+ IoT (optional) LTE + NR + IoT (optional) WCDMA + LTE+ NR+ IoT (optional) |

Table 1. Summary of Capability Set 19 for MSR BS

A rough calculation of the testing time required for each combination would say that it takes aprox. 2 working weeks to test just one combination. If we extrapolate this value to calculate the requirement to run EMC testing for CS19 would result on 18 working weeks to complete the 9 combinations.

Observation 2: Current testing of EMC requirements as specified in standards such as TS 37.113 [1] (MSR BS) is increasingly complex and considers many test configurations, due to various possible RAT combinations, which might lead to increasing testing costs.

## Analysis of EMC Testing

The goal of EMC testing is ensuring that any electronic or electrical device does not emit a large amount of electromagnetic interference (emissions) and that devices continue to function as intended in the presence of several electromagnetic phenomena (immunity).

## Emissions Testing

EMC emissions testing covers both conducted and radiated emissions.  EMC conducted emission testing is performed to assess the electromagnetic energy levels versus frequency conducted by product cables (power lines, network cables, etc). Test labs measure these emissions (usually from 150 kHz - 30 MHz and verify that they comply with specified limits. ETSI and 3GPP have agreed on following CISPR 32 [4] standard for this purpose. Other emissions testing include harmonic and voltage fluctuations and flicker testing.

Observation 3: Conducted emission test methods and levels defined by IEC/CISPR are independent of the product (BS) characteristics and features, including the operating frequency or the Radio Access Technology (RAT).

Regarding EMC radiated emissions test, this involves measuring the electromagnetic field strength of the emissions that are unintentionally generated by a product (a BS in this case). Emissions are inherent to the switching voltages and currents within any digital circuit. In this case, the radiated emission requirements as well as the test methods that apply are defined by IEC and CISPR subcommittee in the Standard CISPR 32 [4].

According to CISPR 32 [4] in section 6, the procedures to be used for measurement of emission levels depend upon several elements. These include but are not limited to the type of EUT,  the type of port, the types of cables used, the frequency range, and the mode of operation. Same standard defines mode of operation as the “set of operational states of all functions of an EUT during a test or measurement”. According to this definition the different RATs should be considered as different modes of operation.

CISPR 32 Annex B specifies the methods for exercising the EUT during emission measurements. Regarding MME (which typically have several different functions and numerous modes of operation associated with each function), CISPR indicates that “For each function, or group of functions selected to exercise the EUT, a number of representative modes of operation, including low power/standby mode, shall be considered for testing. The mode(s) that produce(s) the highest emissions shall be selected for the final measurements.”

Observation 4: EUT/BS typically have several different functions and numerous modes of operation associated with each function. The mode(s) that produce(s) the highest emissions could be selected for the final measurements.

## Immunity Testing

As well as EMC emissions testing, immunity testing covers both conducted and radiated immunity. Conducted immunity testing is used to simulate the normal voltage and current environment of external power and signal cables. Radiated immunity testing is intended to see how well a device performs when it is subjected to an electric field of a specified amplitude (measured in volts/meter) across a range of frequencies. ETSI EN 300 489-1 [5] defines that the test shall be in accordance with IEC 61000-4-3 [6].

The radiated immunity setup could be different according to the frequency operation; however, this principle does not apply for the RATs. In this case all RATs use the same setup in the chamber/shielding room. The different is the test bed which is set outside the chamber/shielding room for monitor different RATs performance.

Observation 5: Conducted immunity, ESD, EFT, Voltage dips, surges test methods and levels defined by IEC/CISPR are independent of the product (BS) characteristics and features, including the operating frequency or the Radio Access Technology (RAT). For Radiated Immunity testing the difference is in the test bed implementation outside of the chamber/shielding room used to monitor different RATs performance.

The object of IEC 61000-4-3 [6] is to establish a common reference for evaluating the immunity of electrical and electronic equipment when subjected to radiated, radio-frequency electromagnetic fields. IEC remarks that IEC 61000-4-3 [6] is a basic EMC publication for use by product committees of the IEC. IEC Guide 107 [7] also states that the IEC product committees are responsible for determining whether this immunity test standard should be applied or not, and if applied, they are responsible for determining the appropriate test levels and performance criteria.

Observation 6: IEC provides a common reference for evaluating the immunity of electrical and electronic equipment. This common reference can be adapted/adjusted in terms of levels and performance criteria .

## Proposal for EMC Test Configuration Reduction

In table 2 we present a summary of the different Capability Sets associated to the test configurations in TS 37.141 [3]. The table also shows a summary of the RAT combinations currently used for EMC testing.

Table 2. Summary of Capability sets and Supported RATs combinations for MSR BS

In view of the observations presented in the previous sections, we consider a simplification in the EMC testing can be achieved by reducing the number of RAT combinations defined in some Capability Sets. As a result of this reduction, it will be possible also to reduce the number of Test Configurations considered for EMC testing in an MSR environment.

The guiding principles of our proposal consider that to test MSR the best pick is to create the “worst” case (from an EMC testing perspective) for emission and immunity tests. Currently the Capability Sets for MSR standard add a new combination when a new RATs comes in. Even though, this principle looks for an understandable follow up of technical developments, it might not be the more efficient way from an EMC testing perspective.

The RATs technologies can be classified in two types: Narrow band and Wide band solutions. A simplification could consider this principle as a mechanism to achieve a test reduction. GSM is a RAT with a low ratio of immunity failure.  In addition, the test amplitude on receiver RX QUAL is -47 dBm (according to the standard). It can be stated that a very strong signal lead to very strong immunity, which does not represent the worst testing case. NB-IoT has narrow bandwidth but high amplitude (high power density) for emission. On the other hand, for the wideband category, WCDMA immunity is verified via RNC-sim measuring BER, BLER. This data is fault corrected in RNC-sim, which do not represent the worst-case scenario LTE is immunity verified with (the best) throughput UL/DL, which is sensitive to immunity and reacts very swift during immunity problems. It is very similar to NR.

Observation 7: By grouping GSM and NB-IoT in the narrow band category and WCDMA and LTE in the Wide band category, we have a starting point to develop a RAT-agnostic analysis based on the aforementioned categorization.

As an example of the possibilities of this proposal in the following figures, we shared a Radiated Spurious emission results comparison between LTE+IoT+GSM+WCDMA (Figure 1) and a scenario where only LTE+IoT (Figure 2) RATs are tested. Test is applied to a dual BS operating in Bands 1 and 3.

Figure 1. Radiated Spurious Emission result for dual band BS (LTE+IoT+GSM+WCDMA).

DL Downlink	B3: 1805 – 1880 MHz	B1: 2110 – 2170 MHz
Not defined ± 10 MHz on the downlink bands:	B3: 1795 – 1890 MHz	B1: 2100 – 2180 MHz

Figure 2. Radiated Spurious Emission result for dual band BS (LTE+IoT).

DL Downlink	B3: 1805 – 1880 MHz	B1: 2110 – 2170 MHz
Not defined ± 10 MHz on the downlink bands:	B3: 1795 – 1890 MHz	B1: 2100 – 2180 MHz

In the figures, it is possible to see that the spurious level is very similar. The final result for LTE+IOT+GSM+WCDMA LTE configuration is -60,19dBm while for LTE + IOT configuration is -59.92dBm. From a practical perspective there is no big difference in terms of the impact played by PA, baseband elements or DAC/ADC components. It is the RF chain the main contributor to EMC problems.

Considering both the testing results and the observations presented in this document, the EMC test simplification proposal is summarized in table 3.

| Capability Set | CS1 | CS2 | CS3 | CS4 | CS5 | CS6 | CS7 | CS8 | CS9 | CS10 | CS11 | CS12 | CS13 | CS14 | CS15 | CS17 | CS16 | CS18 | CS19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MR Configuration | SR | SR | WCDMA + LTE | GSM + WCDMA | GSM + LTE+Io (optional) | GSM + WCDMA + LTE GSM + WCDMA GSM + LTE WCDMA + LTE | GSM + WCDMA GSM + LTE+ IoT (optional) LTE+ WCDMA + IoT (optional) | SR | GSM + IoT | WCDMA + IoT | LTE + IoT | GSM + WCDMA + IoT GSM + IoT WCDMA + IoT | GSM + LTE + IoT GSM + IoT LTE + IoT | WCDMA + LTE + IoT WCDMA+ IoT LTE + IoT WCDMA + LTE | GSM + WCDMA +IoT GSM + LTE + IoT WCDMA + LTE + IoT GSM + IoT WCDMA + IoT LTE + IoT GSM + WCDMA GSM + LTE WCDMA + LTE | LTE+ NR+ IoT LTE + IoT NR + IoT LTE+ NR+ IoT+IoT | LTE+ NR+ IoT (optional) | GSM + LTE+ IoT (optional) GSM + NR+ IoT (optional) LTE + NR + IoT (optional) GSM + LTE+ NR+ IoT (optional) | WCDMA + LTE+ IoT (optional) WCDMA + NR+ IoT (optional) LTE + NR + IoT (optional) WCDMA + LTE+ NR+ IoT (optional) |
| Suggested RAT combinations | WCDMA | LTE | WCDMA + LTE | GSM + WCDMA | LTE + IoT-IN/IG | GSM + LTE | LTE + IoT-IN/IG | IoT-SA | GSM + IoT-SA | WCDMA + IoT-SA | LTE + IoT-SA | WCDMA + IoT-SA | LTE + IoT-SA | LTE + IoT-SA | LTE + IoT-SA | LTE+ NR+ IoT-SA If support IoT standalone | LTE+ NR+ IoT-IG/IN If support guard/inband IOT | LTE+ NR+ IoT-IG/IN | LTE+ NR+ IoT-IG/IN |
| Optimization suggestions for CS/TC | SR covered by MR WCDMA covered by LTE LTE tested in TC21/22 | SR covered by MR LTE tested in TC21/22 | WCDMA covered by LTE LTE tested in TC21/22 | GSM covered by IoT WCDMA covered by LTE IoT and LTE test in TC21/22 | GSM covered by IoT IoT and LTE tested in TC21/22 | GSM covered by IoT IoT-IN/IG and LTE test in TC21 | GSM covered by IoT IoT-IN/IG and LTE test in TC21 | SR covered by MR IoT-SA test in TC22 | GSM covered by IoT IoT-SA tested in TC22 | WCDMA covered by LTE IoT-SA and LTE tested in TC22 | LTE + IoT-SA test in TC22 | GSM covered by IoT WCDMA covered by LTE IoT-SA and LTE tested in TC22 | GSM covered by IoT LTE + IoT-SA test in TC22 | WCDMA covered by LTE IoT-SA and LTE tested in TC22 | GSM covered by IoT WCDMA covered by LTE IoT-SA and LTE tested in TC22 | TC22 | TC21 | GSM covered by IoT IoT-IG/IN and LTE tested in TC21 | WCDMA covered by LTE IoT-IG/IN and LTE tested in TC21 |
| Optimization suggestions | Covered by TC21/22 | Covered by TC21/22 | Covered by TC21/22 | Covered by TC21/22 | Covered by TC21/22 | Covered by TC21/22 | Covered by TC21 | Covered by TC22 | Covered by TC22 | Covered by TC22 | Covered by TC22 | Covered by TC22 | Covered by TC22 | Covered by TC22 | Covered by TC 22 | Test (Cover CS1,2,3,4,5,6,18,19) | Test (Cover CS1,2,3,4,5,6,7,8,9,10,11,12,13,14,15) | Covered by TC21 | Covered by TC21 |

Table 3. Proposal for EMC Test Configuration Reduction

Applying the simplification in the RAT combinations considered for EMC testing, it is possible to reduce the consider Capability sets to CS17 and CS18, which corresponds to use TC21 and TC 22 for EMC testing. It worth to mention that these TCs (21 and 22) are already defined in TS 37.141 [3]. Therefore, our goal is to reach a simplification by using existing TCs and not creating a new TC. In addition, our proposal covers at least one wideband and one narrowband active RF chain.

# Conclusion

In this contribution a proposal on EMC Test Configuration reduction is presented. Based on our observations, we propose:

Proposal 1: To agree in using only TC 21 and 22 defined in TS 37.141 [3] for EMC testing of MSR BS as presented in this contribution.

# References

3GPP TS 37.113. “NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) Electromagnetic Compatibility (EMC)”

3GPP TS 37.114. “Active Antenna System (AAS) Base Station (BS) Electromagnetic Compatibility (EMC)”

3GPP TS 37.141. NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) conformance testing.

CISPR 32. Electromagnetic compatibility of multimedia equipment - Emission requirements.

ETSI EN 301 489-1, ElectroMagnetic Compatibility (EMC) standard for radio equipment and services; Part 1: Common technical  requirements; Harmonised Standard covering the essential requirements of article 3.1(b) of Directive 2014/53/EU and the essential requirements of article 6 of Directive 2014/30/EU.

IEC 61000-4-3: "Electromagnetic compatibility (EMC) - Part 4: Testing and measurement techniques - Section 3: Radiated, radio-frequency electromagnetic field immunity test"

IEC GUIDE 107:2014. Electromagnetic compatibility - Guide to the drafting of electromagnetic compatibility publications