3GPP TSG-RAN WG4 # 95-e	R4-2007564

Electronic Meeting, 25 May – 5 June, 2020

Source:	Qualcomm Incorporated

Title:	Exclusion 100KHz for NB-IoT to meet FCC band-edge requirements

Agenda item: 	12.2

Document for:	Approval

# Introduction

In RAN4#94e-Bis meeting, the issues on NB-IoT band-edge performance against FCC regulation #22.917, #27.53, #24.238 were discussed in [1] and the WF was agreed in [2]. In this paper, we provide the analysis results and propose to exclude 100kHz for NB-IoT on Band 2, 3, 4, 5, 12, 17, 25, 66, 71, 85.

# Discussion

## FCC # 27.53 part (c)- Band 13

As per FCC specification # 27.53 part (c), the band-edge emission requirement is applicable outside 776 – 788MHz. As seen in Figure1, the allowed emissions at band-edge should be attenuated from the desired carrier by 43+10*log(P)dB. Also, the band-edge power must be measured using a resolution bandwidth of at least 30KHz. Since B13 spans from 777 -787 MHz, the devices need to meet the requirements at 1MHz from the edge of band.

Figure 1: FCC specification 27.53(c) excerpt

For simplicity in simulation, as shown in Figure 2, the band-edge is considered @ 9MHz (i.e. maps to 776MHz) and the first ARFCN center is @ 10.1MHz (i.e. maps to 777.1MHz corresponding to first ARFCN of B13).

Figure 2: Simulation for 1MHz from the edge of band

Observation 1: NB-IoT devices on Band 13 could meet the FCC # 27.53-part (c) requirements.

## FCC # 27.53 part (g)- Band 12, 17, 71 & 85

As per FCC specification # 27.53 part (g), highlighted in yellow, this spec requires to integrate the band-edge power with RBW of 30KHz.

Figure 3: FCC specification 27.53(g) excerpt

For simplicity in simulation, the band-edge is considered @ 10MHz and the first ARFCN center is @ 10.1MHz.

Figure 4: First and last ARFCN for NB1 with 12 tone with 30kHz RBW

Figure 5: First and last ARFCN for NB1 with single 3.75kHz tone

From the simulation in Figure 4, simulated Band-edge power is - 4.3dBm which could not meet the FCC Band-edge emission limit 20 – (43+10*log(0.1))=-13dBm.

As shown in Figure 5, a similar failure will be observed in the last ARFCN for 3.75KHz tone since simulated Band-edge power is 19.8dBm. Then we have the following observation:

Observation 2: With edge ARFCNs, NB-IoT devices fail band-edge limits for FCC # 27.53(g) since FCC mandates to use 30kHz RBW.

As per simulation in Figure 6 and 7,  we can observe that with exclusion to first and last ARFCN, NB-IoT devices can meet FCC band-edge requirement of -13dBm for both 12 tone and single 3.75KHz tone cases. Note that for simplicity in simulation, the band-edge is considered @ 10MHz, first ARFCN is excluded and the second ARFCN centered @ 10.2MHz is shown in blue.

Figure 6: Exclusion of first and last ARFCN for NB1 with 12 tone with 30kHz RWB

Figure 7: Exclusion of first and last ARFCN for NB1 with 12 tone with 30kHz RBW

Observation 3: NB-IoT devices on Band 12, 17, 71 & 85 can meet FCC band-edge requirement of -13dBm if we exclude the first and last ARFCN.

## FCC # 27.53 part (h)- Band 3, 4 & 66

As highlighted in yellow, this spec allows to integrate the band-edge power with RBW of 1% of emission BW (i.e. 2KHz for NB1/NB2 Waveforms).

Figure 8: FCC specification 27.53(h) excerpt

For simplicity in simulation, the band-edge is considered @ 10MHz and the first ARFCN center is @ 10.1MHz.

Figure 9: First and last ARFCN for NB1 with single 3.75KHz tone with 2kHz RBW

Observation 4: As shown in Figure 9, with 2kHz RBW at band-edge, NB-IoT devices are not able to achieve the required attenuation of 43+10*log(P).

Figure 10: Exclusion of first and last ARFCN for NB1 with single 3.75KHz tone with 2KHz RBW

Observation 5: With exclusion to first and last ARFCN on B3, 4 and 66, NB-IoT devices can meet FCC band-edge requirement of -13dBm.

## FCC # 24.238- Band 2 & 25

As per FCC# 24.238, the band-edge power with RBW of 1% of emission BW (i.e. 2kHz) should be used which is similar as FCC # 27.53 part (h).

Figure 11: FCC specification 24.238 excerpt

Observation 6: The exclusion of first and last ARFCN for B 2 and 25 is needed to meet FCC # 24.238 band-edge requirement.

## FCC # 22.917- Band 5

Figure 12: FCC specification 22.917 excerpt

As per FCC# 22.917, the band-edge power with RBW of 1% of emission BW (i.e. 2kHz) should be used which is similar as FCC # 27.53 part (h) and FCC # 24.238.

Observation 7: The exclusion of first and last ARFCN for B5 is needed to meet FCC # 22.917 band-edge requirement.

In summary, we propose to exclude the first and last ARFCNs for below bands:

- B12, B17, B71 & B85 to meet FCC # 27.53(g)

- B3, B4 & B66 to meet FCC # 27.53(h)

- B2 & B25 to meet FCC # 24.238

- B5 to meet FCC # 22.917

Proposal 1: Modify the NS_04 to exclude first and last ARFCN for Band 2, 3, 4, 5, 12, 17, 25, 66, 71, 85.

Table 6.2.4F-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | 6.6.2F.1 | Operating bands defined in 5.5F | N/A |
| NS_02 | 6.6.2F.2.1 | [1, 2, 3, 5, 8, 11, 12, 13, 17, 18, 19, 20, 21, 25, 26, 28, 66, 70 and 85] | [N/A] |
| NS_03 | 6.6.2F.2.2 | [1, 2, 3, 5, 8, 11, 12, 13, 17, 18, 19, 20, 21, 25, 26, 28, 66, 70 and 85] | [N/A] |
| NS_04 | 5.5F | 2, 3, 4, 5, 12, 17, 25, 26, 66, 71, 85 | N/A |

Regarding the options on which release shall apply the change, we propose to introduce this from the same release for Band 26 i.e. Release 14 [3]. Without this change, NB-IoT devices could not pass the FCC requirements. Moreover, this signalling is mainly up to eNB configuration on UE uplink channel, so it would not impact exiting NB-IoT devices which have already supported NS_04 for Band 26. With that, we have the following proposal:

Proposal 2: Introduce the change on network signalling NS_04 from Rel-14.

# Conclusion

In this paper, we provide the analysis results and views on NB-IoT to meet FCC band-edge requirements. We have the following observations and proposals:

Observation 1: NB-IoT devices on B13 could meet the FCC # 27.53-part (c) requirements.

Observation 2: With edge ARFCNs, NB-IoT devices fail band-edge limits for FCC # 27.53(g) since FCC mandates to use 30KHz RBW.

Observation 3: NB-IoT devices on B12, 17, 71 & 85 can meet FCC band-edge requirement of -13dBm if we exclude the first and last ARFCN.

Observation 4: As shown in Figure 9, with 2kHz RBW at band-edge, NB-IoT devices are not able to achieve the required attenuation of 43+10*log(P).

Observation 5: With exclusion of first and last ARFCN on B3, 4 and 66, NB-IoT devices can meet FCC band-edge requirement of -13dBm.

Observation 6: The exclusion of first and last ARFCN for B2 and 25 is needed to meet FCC # 24.238 band-edge requirement.

Observation 7: The exclusion of first and last ARFCN for B5 is needed to meet FCC # 22.917 band-edge requirement.

Proposal 1: Modify the NS_04 to exclude first and last ARFCN for Band 2, 3, 4, 5, 12, 17, 25, 66, 71, 85.

Table 6.2.4F-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | 6.6.2F.1 | Operating bands defined in 5.5F | N/A |
| NS_02 | 6.6.2F.2.1 | [1, 2, 3, 5, 8, 11, 12, 13, 17, 18, 19, 20, 21, 25, 26, 28, 66, 70 and 85] | [N/A] |
| NS_03 | 6.6.2F.2.2 | [1, 2, 3, 5, 8, 11, 12, 13, 17, 18, 19, 20, 21, 25, 26, 28, 66, 70 and 85] | [N/A] |
| NS_04 | 5.5F | 2, 3, 4, 5, 12, 17, 25, 26, 66, 71, 85 | N/A |

Proposal 2: Introduce the change on network signalling NS_04 from Rel-14.

# 4 	References

- R4-2003987 NBIOT guard band operation for FCC regulation considerations, RAN4#94bis-e

- R4-2005219 WF on NB-IoT band edge performance against FCC regulation, RAN4#94bis-e

- R4-1905011 CR to exclude 100kHz for NBIOT for B26 band edge operation due to PLMR emissions requirement., RAN4#90-bis

- https://www.ecfr.gov. FCC# 27.53, #24.238, # 22.917