3GPP TSG-RAN WG4 Meeting #96e															R4-2009758

Online, 17 – 28 August, 2020

Source:	Intel Corporation

Title:	On numerology and channel BW in 52.6 – 71 GHz

Agenda item:	13.2.2

Document for:	Approval

# Background

New spectrum in 60 GHz range, i.e. 52.6 – 71 GHz, will be introduced in 3GPP. During SI phase, numerology and channel BW will be studied and evaluated. In this paper, we present possible channel bandwidths, numerologies, and corresponding FFT sizes for discussion and decision

# Discussion

As indicated in the SID [1], existing DL/UL waveform is employed to support operation between 52.6 GHz and 71 GHz. Note that the selection of numerologies can follow design principle as defined in NR Rel-15. More specifically, subcarrier spacing at this frequency range can be equal to 2μ×15kHz, where μ is a positive integer number. Further, symbol- and subframe-level boundary alignment across different subcarrier spacings with same CP overhead is defined, which can help in achieving seamless coexistence when mixed numerologies are employed in the same frequency band.

Compared to FR2 where subcarrier spacings with 60 kHz and 120 kHz are specified for data and control channel, and 120 kHz and 240 kHz are specified for SSB transmission, a larger subcarrier spacing is envisioned for carrier frequency between 52.6 GHz and 71 GHz, to combat more severe phase noise. Table 1 summarizes the candidate numerologies for carrier frequency between 52.6 GHz and 71 GHz with normal CP. Note that in the table, slot duration is calculated for the slots other than first slot of 0.5 ms duration. For the first slot of 0.5 ms duration, additional 0.52 µs is needed to align the symbol- and slot boundary as for smaller subcarrier spacing in FR1 and FR2.

| µ | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- |
| Subcarrier spacing | 240 kHz | 480 kHz | 960 kHz | 1.92 MHz |
| Symbol duration | 4.17 µs | 2.08 µs | 1.04 µs | 0.52 µs |
| Slot duration | 62.43 µs | 31.22 µs | 15.60 µs | 7.80 µs |
| Normal CP length | 292.97 ns | 146.48 ns | 73.24 ns | 36.62 ns |

Table 1. Candidate numerologies and symbol/slot duration

As specified in NR, maximum channel bandwidth is 100 MHz and 400 MHz for FR1 and FR2, respectively. For carrier frequency between 52.6 GHz and 71 GHz, it is expected maximum channel bandwidth would be larger compared to that for FR2. As much wider channel bandwidths are typically available at this frequency range, utilizing relatively large subcarrier spacing can simplify implementation and reduce complexity at transceiver by avoiding an extremely large FFT size. Table 2 summarizes the possible channel bandwidth and corresponding FFT size with various candidate subcarrier spacings.

Table 2. Possible channel bandwidth, numerology and corresponding FFT size

Note that subcarrier spacing of 120 kHz which is currently supported by NR Rel-15 in FR2 for data transmission results in FFT/IFFT size greater than 4096, i.e., 8192, 16384 and 32768 for channel bandwidths larger than 800 MHz. These sizes may not be desirable for practical UE implementations from the perspective of implementation complexity and power consumption. Furthermore, limiting the FFT/IFFT size by 4096 while keeping the small SCS simultaneously will significantly reduce the supportable channel bandwidth. It is worth mentioning that other competing wireless systems, e.g., IEEE 802.11ad/ay, operating in the same unlicensed band around 60 GHz already support sufficiently large channel bandwidths, i.e., 2.16 GHz as a single channel bandwidth and the latest release specifies up to 8.64 GHz with channel aggregation, i.e., carrier aggregation in 3GPP. In this case, smaller channel bandwidth defined for NR in 52.6 – 71 GHz frequencies would make NR less competitive against the existing systems. In this regard, lack of supporting larger bandwidths would reduce a full potential of 52.6 – 71 GHz frequency range for truly NR-based massive broadband and make it less attractive for emerging applications like AR/VR.

While other wireless systems support 2.16 GHz as a single channel bandwidth, we think it is worthwhile to define 2 GHz as a maximum channel bandwidth in NR to maintain NR channel bandwidth principle yet. As the huge unlicensed spectrum is available, we do not see strong motivation having similar channel bandwidths specified in NR FR2, i.e., 50, 100, 200, and 400 MHz, which are too narrow considering huge available unlicensed spectrum, i.e., 18,400 MHz unlicensed spectrum. From this perspective, the minimum channel bandwidth is the one between 500 MHz and 800 MHz, in our view. Further down selection is required.

Observation #1: Competing wireless systems, i.e., 11ad/ay, sharing the same unlicensed spectrum already support 2.16 GHz as a single channel bandwidth. Latest release of 11ay specifies channel bandwidth up to 8.64 GHz with channel aggregation.

Observation #2: Supporting smaller channel bandwidth for NR in 52.6 – 71 GHz frequency range would make NR less competitive against to the existing other wireless systems.

Proposal #1: RAN4 agrees on the maximum FFT size of 4k.

Proposal #2: Supported channel bandwidth is up to 2000 MHz.

Proposal #3: Minimum channel bandwidth is one in range of 500 MHz – 800 MHz. Further down selection is required.

Regarding SCS selection, RAN4 might be able to narrow down once support channel BWs and maximum FFT size are decided.

# Summary

In this paper, we presented possible channel bandwidths, numerologies, and corresponding FFT size. The following observations and proposals were made.

Observation #1: Competing wireless systems, i.e., 11ad/ay, sharing the same unlicensed spectrum already support 2.16 GHz as a single channel bandwidth. Latest release of 11ay specifies channel bandwidth up to 8.64 GHz with channel aggregation.

Observation #2: Supporting smaller channel bandwidth for NR in 52.6 – 71 GHz frequency range would make NR less competitive against to the existing other wireless systems.

Proposal #1: RAN4 agrees on the maximum FFT size of 4k.

Proposal #2: Supported channel bandwidth is up to 2000 MHz.

Proposal #3: Minimum channel bandwidth is one in range of 500 MHz – 800 MHz. Further down selection is required.

# Reference

[1] RP-193259, “New SID Study on supporting NR above 52.6 – 71 GHz”, Intel Corporation, RAN#86, Dec. 2019