3GPP TSG-RAN WG4 Meeting #96-e            	R4-2010282

Electronic meeting, 17th Aug– 28th Aug, 2020

Agenda item:	7.8.1.2.3

Source:	Samsung

Title:	Discussion and simulation results for BS URLLC requirement

Document for:	Discussion

# 1	Introduction

During the last RAN4 meeting, the general issue and test setup for test feasibility and performance requirements for URLLC were further discussed and the related agreements were captured in WF [1] and WF [2], separately. Meanwhile, the simulation assumptions for related requirement were agreed to facilitate the requirement specification.

In this contribution, the view on the remaining issue of BS URLLC requirement are presented. Also, the initial results are provided for alignment purpose.

# 2	Discussion

2.1	Requirements for ultra-lower BLER

F2 requirement

In the last meeting, the URLLC requirement for FR1 was discussed. As for FR2, there is no consensus to define ultra-lower BLER requirement. From the receiver perspective, the performance shows no different based on current simulation assumption for FR1 and FR2. Only the testing time will be impacted.  From deployment perspective, both FR1 and FR2 are the targeting scenario for URLLC. RAN4 has already agreed to define requirement of  higher BLER and low latency with test applicability for FR2, in that sense, current test cases should be fulfilled the requirement of  test coverage for FR2.  FR2 is operated in the TDD mode, due to the number of unavailable UL slots, the testing time will be increased.

Therefore, we prefer to deprioritize the FR2 requirement for ultra-lower BLER considering the tradeoff between test coverage and testing time.

Proposal 1: Deprioritize FR2 URLLC requirement for ultra-lower BLER

BS SCS test applicability

Regarding to BS SCS test applicability, the following options are considered

- Option 1: ALL supported SCS tested - Option 2: only 15KHz SCS tested if both supported - Option 3: only 30KHz SCS tested if both supported

In previous meeting, it was agreed to introduce the ultra-lower BLER requirement with both 15 KHz SCS for FDD mode, and 30 KHz SCS for TDD mode. With AWGN as simulation assumption, the performance is not expected to show the difference. Thus, to reduce the number of test and test effort, we think there is no necessary to test all the supported SCS if BS can support both 15 KHz SCS and 30 KHz SCS. Similarly, to reduce the testing time, we prefer only 15 KHz SCS tested for ultra-lower BLER requirement if BS supports both 15 KHz SCS and 30 KHz SCS.

Proposal 2: Only 15KHz SCS tested for ultra-lower BLER requirement if BS supports both 15 KHz SCS and 30 KHz SCS

2.2	Requirements for higher BLER

HARQ operation

Both HARQ combination and slot aggregation are enable, when the URLLC requirement for higher BLER was specified. In NR Rel-15 PUSCH requirement, the number of maximum number of HARQ transmissions without slot aggregation is configured as 4 and the relevant RV sequence is configured as 0 2 3 1. Take the FDD structure with 8 HARQ process as example,

Figure 1. FDD structure with 8 HARQ process

With regarding the PUSCH aggregation level, the RV allocation for each transmission scheme is indicated as follows:

Table 1. RV allocation for each transmission with PUSCH aggregation level

| rvid indicated by the DCI scheduling the PUSCH | rvid to be applied to nth transmission occasion |  |  |  |
| --- | --- | --- | --- | --- |
|  | n mod 4 = 0 | n mod 4 = 1 | n mod 4 = 2 | n mod 4 = 3 |
| 0 | 0 | 2 | 3 | 1 |
| 2 | 2 | 3 | 1 | 0 |
| 3 | 3 | 1 | 0 | 2 |
| 1 | 1 | 0 | 2 | 3 |

Based on current assumption, considering the full buffer scenario, the structure with HARQ and slot aggregation n2 with configured {0 3 03} for RV allocation for each transmission/re-transmission can be explained as

Figure 2. FDD structure with 8 HARQ process with slot aggregation n2 for full buffer RTT

With non-full buffer scenario, the structure with HQRQ and slot aggregation n2 can be explained as

Figure 3. FDD structure with 8 HARQ process with slot aggregation n2 for non-full buffer HARQ RTT

Under this assumption, all the possible RV can be covered based on HARQ IR combination as least with 2 re-transmission.

Meanwhile, in order to align with non-slot aggregation, we prefer to use the full buffer mode.

Proposal 3: The RV sequence with 4 HARQ transmission is 0 3 0 3 under full buffer scenario

PUSCH aggregation level

Regarding the value of PUSCH aggregation level for 15 KHz SCS, there is no consensus yet. The following options are considered

- PUSCH aggregation factor for 15KHz - Option 1: No requirement for 15 KHz SCS - Option 2: Only configuration n2 for FDD and no requirement for TDD - Option 3: Configure n2 for FDD and n8 for TDD with note - Note: The intention for this configuration is to have two effective transmission of the transport block. To achieve this for the standard TDD pattern captured in this table, a value of n8 is necessary, while for FDD, a value of n2 is necessary

The motivation to configure n8 for TDD pattern DDDSU with 15 KHz is to guarantee two effective transmissions of the transport block. While the shortage for this configuration, the transmission delay will be increased in order to complete the transmission, due to the unavailable UL slots with TDD. Since the purpose of URLLS is designed to high reliability and low latency, in that sense, it seems that configured PUSCH aggregation factor with n8 is not a typical scenario to meet the URLLC requirement for 15 KHz SCS with TDD pattern DDDSU. Therefore, we prefer to no requirement for TDD with 15 KHz SCS.

Observation 1: Configured PUSCH aggregation factor with n8 is not a typical scenario to meet the URLLC requirement with high reliability and low latency for 15 KHz SCS with TDD patter DDDSU.

Proposal 4: No PUSCH aggregation requirement for TDD with 15 KHz SCS

Based on the RV allocation for each transmission, it can be calculated that the RV for effective UL transmission, for PUSCH aggregation level n8 with TDD 15 KHz, is the same as TDD 30 KHz SCS with PUSCH aggregation level n2, where RV value is calculated with mod (n, 4) operation for nth transmission.

Observation 2: The RV value for effective UL transmission with PUSCH aggregation level n8 is same as TDD 30 KHz with PUSCH aggregation level n2.

From the demodulation requirements perspective, the performance with PUSCH aggregation level n8 is not expected to show difference with PUSCH aggregation level n2 for FDD, considering the number of effective UL transmission is 2. Therefore, it is no need to define the URLLC requirement with PUSCH aggregation level n8 for TDD with 15 KHz. If needed, the requirement with PUSCH aggregation level n8 can be applied with FDD or TDD 30 KHz SCS with PUSCH aggregation level n2 with same requirement.

Based on above analysis, if the slot aggregation requirement with n8 for TDD with 15 KHz is introduced to guarantee the BS with supporting TDD 15 KHz for URLLC feature can be tested, we prefer to add the note as: The requirement with slot aggregation level n8 for TDD with 15 KHz SCS can be applied with FDD or TDD 30KHz SCS with slot aggregation level n2.

Proposal 5:  If the slot aggregation requirement with n8 for TDD with 15KHz SCS is introduced, adding a note as

- The requirement with PUSCH aggregation level n8 for TDD with 15 KHz SCS can be applied with FDD or TDD 30 KHz SCS with PUSCH aggregation level n2.

Test applicability

Regarding the test applicability rule for FR1 and FR2 if BS declares to support both FR1 and FR2, the following options are considered

- Option 1: Tests shall be done for either FR1 and FR2 - Option 2: Tests shall be done for both, and only for 1 SCS will be tested for each frequency band with test applicability - Option 3: Tests shall be done for both, all supported SCS will be tested for FR1 and only 1SCS need to be tested for FR2

Although both FR1 and FR2 are the targeting scenario for URLLC, as indicated in the specification 38.824, most of URLLC use cases (i.e. Rel-15 enabled use case, factory automation, transport industry and electrical power distribution), are evaluated under FR1, such as 700MHz. In that sense, FR1 should be the practical deployment for URLLC in current stage. Therefore, we prefer to perform the URLLC tests for requirements of high reliability with high BLER either FR1 or FR2 based on BS declaration to reduce the test effort if BS declares to support both FR1 and FR2.

Proposal 6: URLLC tests for requirement of high reliability with high BLER shall be done for either FR1 or FR2 based on BS declaration if BS declares to support both FR1 and FR2.

SCS and BW for FR2

With regarding SCS and BW, the following options are considered:

- Option 1: 50MHz for 60KHz, 100MHz for 120MHz - Option 2: 50MHz/100MHz for 60KHz, 50MHz/100MHz for 120KHz

In eMBB, RAN4 has defined the requirement with different SCS and CBW. From the test coverage aspects, it should be enough. Meanwhile, the requirements of minimum bandwidth for each SCS is specified. To reduce the test effort and fulfill the BS requirement with any supported BW, we prefer to only define requirement with 60 KHz SCS for 50MHz, and 120 KHz for 50MHz.

Proposal 7: Only define FR2 URLLC requirement with 60 KHz SCS for 50MHz, and 120 KHz SCS for 50MHz

2.3	Requirements for low latency

During the last RAN4 meeting, it is agreed to define the FR2 low latency requirement for URLLC with test applicability rule. The simulation assumption for FR1 is agreed, while there is no discussion about the test parameters for FR2. In this section, the view on the test parameters is presented.

Number of symbol for FR2

Regarding the number of OFDM symbol for low latency requirement, serval options for the number of OFDM symbol were proposed for low latency requirement with PUSCH mapping type B.  In the last meeting, based on the majority review, 2OS with MCS 10 is agreed to define the low latency requirement for FR1.

In terms of requirement, we think RAN4 should focus on the typical scenario with possible network scheduling.

Firstly, as indicated in the specification 38.824, most of URLLC use cases (i.e. Rel-15 enabled use case, factory automation, transport industry and electrical power distribution), 4 or 7 OS is assumption for baseline performance evaluation.

Observation 3:  4 or 7 OS is assumption for baseline performance evaluation for most of URLLC use cases.

Secondly, form the RAN1 physical design, in current Rel-16 URLLC feature with mini-slot repetition, different options based on 4 OS mini-slot repetition to investigate the scheme with supporting dynamic switch between mini-slot repetition and multi-segments as follow:

Figure 4. FDD structure with 8 HARQ process with slot aggregation n2 for non-full buffer HARQ RTT

Observation 4:  Mini-slot repetition with 4OS is the typical scenario in RAN1 discussion to supporting dynamic switch between mini-slot repetition and multi-segments

Thirdly, in last meeting, 2OS with MCS 10 was agreed to specify low latency requirement for FR1. In Rel-15, RAN1 has designed some features, such as slot-based repetition, low efficient MCS table, which aims to guarantee the high reliability transmission. To some extent, with configured low MCS index can be benefit considering achievable coding gain, also it can reduce the possibility for re-transmission, which will further reduce the transmission latency.

Meanwhile, the PT-RS can be configured in FR2.  It is not a typical scenario that network will configure less number of OFDM symbol for URLLC transmission. Under this scheduling, the number of available REs for data transmission is reduced, which will result in increasing coding rate, for given targeting information bits.

Again, from the test coverage perspective, 2 OS has already covered in the FR1 for low latency. As for 7OS symbol length, in current Rel-15 BS demodulation requirement for eMBB, RAN4 has already defined with 10 symbols requirement with type B in FR2. In terms for performance, we do not think there is too much different between 7OS and 10OS. Meanwhile, both 1 DMRS and 1+1 DMRS are configured for requirement for mapping type B in FR2. As indicated, the requirement between 1 DMRS and 2 DMRS configuration is minor.

Table 1. Minimum requirement of different DMRS configuration in FR2, 50MHz for 60 KHz and 100 MHz for 60 KHz

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-1 | pos0 | No | -2.0 |
|  |  |  |  |  | G-FR2-A3-13 | pos1 | No | -2.2 |

| Number of TX antennas | Number of demodulation branches | Cyclic prefix | Propagation conditions and correlation matrix (Annex G) | Fraction of maximum throughput | FRC (Annex A) | Additional DM-RS position | PT-RS | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLA30-300 Low | 70 % | G-FR2-A3-2 | pos0 | No | -2.1 |
|  |  |  |  |  | G-FR2-A3-14 | pos1 | No | -2.4 |

Observation 5:  Minor performance difference existed for DMRS configuration with 1 and 1+1 for eMBB in FR2

One issue raised by companies is about the channel estimation performance with only 1 DMRS configuration. For eMBB with FR2, 1 DMRS is also configured where the length of data symbol is 9, the proper performance can be also achieved. Since the length of data symbol is 3 for 4OS. Thus, we think there should be no limitation for DMRS estimation performance. As indicated in Figure 4, the initial evaluation is performed to compare the performance with different DMRS configuration.

Figure 5. Performance comparison with different length of OFDM symbols

Observation 6: From the target SNR value with 70% TP perspective, there is no significant difference with configured 2, 4 and 7 OS for PUSCH mini-slot transmission.

Based on above analysis, we prefer to align with RAN1 Rel-16 URLLC discussion with 4 OS.

Proposal 8:  Non-slot scheduling with 4 symbols can be considered for the lower latency requirement for FR2

Test applicability rule for FR1 and FR2

Similar with requirements of high reliability with high BLER, we prefer to perform the URLLC tests for low latency requirement either FR1 or FR2 based on BS declaration to reduce the test efforts.

Proposal 9: URLLC tests for low latency requirement shall be done for either FR1 or FR2 based on BS declaration if BS declares to support both FR1 and FR2.

SCS and BW for FR2

Similar with requirement for high reliability with high BLER, we prefer to only define the requirement of minimum bandwidth of each SCS with 60 KHz SCS for 50MHz, and 120 KHz for 50MHz.

Proposal 10: Only define FR2 URLLC requirement with 60 KHz SCS for 50MHz, and 120 KHz SCS for 50MHz

2.4	Requirements for URLLC based on Rel-16 URLLC feature.

Based on the WID of Rel-16 URLLC, the performance requirements, including UE/BS demodulation requirements, will be specified. Considering there are still some open issues existed based on Rel-15 URLLC feature, we prefer to deprioritize the requirement specified for Rel-16 as least in Q3.

Proposal 11: Prioritize the performance requirements specification based on Rel-15 URLLC feature.

Meanwhile, as agreed in previous meeting, there is no impact on RRM core requirement based on Rel-15 URLLC functionalities.  As indicated in the WID, although there is no objective to specify the URLLC core requirements based on Rel-16 URLLC functionalities,  whether there is impact on RRM core requirement should be further studied.

Compared with Rel-15, the high reliability requirement and lower latency requirement are further tighter, e.g., higher reliability (up to 10^-6) and short latency in the order of 0.5 to 1ms.

From the test methodology perspective, whether the existed methodology for 99.999% can be applied should be further studied.

Proposal 12: FFS on the test methodology for test metric of 99.9999% and low latency requirements (0.5ms~1ms), FFS on the performance requirements based on Rel-16 URLLC functionalities.

2.5	CR work split for BS

During the last meeting, the CR work split is proposed to facilitate specification of URLLC requirement. Considering it is agreed to introduce the FR2 requirement with test applicability rule, Samsung can voluntarily take some CR drafting work for FR2.

Proposal 13: Samsung can voluntarily take some CR drafting work for FR2 URLLC requirement

| Specifacation | Requirements title | CR Specifacation | Frequency range | CR work | CR Responsibility |
| --- | --- | --- | --- | --- | --- |
| BS demodulation (38.104/38.141-1/38.141-2) | FRC for all test cases | 38.104 | FR1 | FRC | Nokia |
|  |  |  | FR2 | FFS | Samsung |
|  |  | 38.141-1 | FR1 | FRC | Ericsson |
|  |  | 38.141-2 | FR1 | FRC | Huawei |
|  |  |  | FR2 | FFS | Samsung |
|  | Test methodology | 38.104? | N/A | Test methodology | Nokia |
|  |  | 38.141-1 | N/A | Test methodology | Nokia? |
|  |  | 38.141-2 | N/A | Test methodology |  |
|  | Test applicability for all test cases | 38.141-1 | FR1 | Test applicability | Huawei |
|  |  | 38.141-2 | FR1 | Test applicability | Ericsson |
|  |  |  | FR2 | FFS | FFS |
|  | Requirements for PUSCH with ultra-low BLER target (10-5) | 38.104 | FR1 | Requirements | Nokia |
|  |  |  | FR2 | FFS | Samsung |
|  |  | 38.141-1 | FR1 | Requirements/Measurement of Performance requirements Annex C.3 / Measurement system set-up Annex D | Ericsson |
|  |  | 38.141-2 | FR1 | Requirements / Measurement of Performance requirements Annex C.3 | Ericsson |
|  |  |  | FR2 | FFS | FFS |
|  | Requirements for PUSCH with aggregation factor configured | 38.104 | FR1 | Requirements | Huawei |
|  |  |  | FR2 | FFS | Samsung |
|  |  | 38.141-1 | FR1 | Requirements/Measurement of Performance requirements Annex C.3 / Measurement system set-up Annex D | Huawei |
|  |  | 38.141-2 | FR1 | Requirements / Measurement of Performance requirements Annex C.3 |  |
|  |  |  | FR2 | FFS | FFS |
|  | Requirements for PUSCH for mapping Type B with low number of symbols | 38.104 | FR1 | Requirements | Nokia |
|  |  |  | FR2 | FFS | Samsung |
|  |  | 38.141-1 | FR1 | Requirements/Measurement of Performance requirements Annex C.3 / Measurement system set-up Annex D | Huawei |
|  |  | 38.141-2 | FR1 | Requirements / Measurement of Performance requirements Annex C.3 |  |
|  |  |  | FR2 | FFS | FFS |

# 3	Simulation Results

In this section, the initial simulation results are provided for alignment purpose. The related simulation assumptions are indicated in Annex..

3.1	Results for ultra-lower BLER

Table 2. Minimum requirement with ultra-lower BLER

| SCS/BW | MCS | Tx/Rx | Number DMRS | Symbol length | Mapping type | Channel | Ideal SNR with BLER 10^-5 | Impairment SNR with BLER 10^-5 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15KHz 5MHz | MCS 5 from table 3 | 1T2R | 1+1 (2,11) | 14 | Type A | AWGN |  |  |
| 15KHz 5MHz | MCS 5 from table 3 | 1T2R | 1+1 (0,10) | 14 | Type B | AWGN |  |  |
| 30KHz 40MHz | MCS 5 from table 3 | 1T2R | 1+1 (2,11) | 14 | Type A | AWGN |  |  |
| 30KHz 40MHz | MCS 5 from table 3 | 1T2R | 1+1 (0,10) | 14 | Type B | AWGN |  |  |

3.2	Results for higher BLER

Table 3. Minimum requirement with high reliability and high BLER

| SCS/BW | MCS | Tx/Rx | Number DMRS | Symbol length | Mapping type | Channel | Ideal SNR with BLER 10^-2 | Impairment SNR with BLER 10^-2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15KHz 5MHz | MCS 5 from table 3 | 1T2R | 1+1 (2,11) | 14 | Type A | TDLB 100-400 |  |  |
| 15KHz 5MHz | MCS 5 from table 3 | 1T2R | 1+1 (0,10) | 14 | Type B | TDLB 100-400 |  |  |
| 15KHz 10MHz | MCS 5 from table 3 | 1T2R | 1+1 (2,11) | 14 | Type A | TDLB 100-400 |  |  |
| 15KHz 10MHz | MCS 5 from table 3 | 1T2R | 1+1 (0,10) | 14 | Type B | TDLB 100-400 |  |  |
| 30KHz 10MHz | MCS 5 from table 3 | 1T2R | 1+1 (2,11) | 14 | Type A | TDLB 100-400 |  |  |
| 30KHz 10MHz | MCS 5 from table 3 | 1T2R | 1+1 (0,10) | 14 | Type B | TDLB 100-400 |  |  |
| 30KHz 40MHz | MCS 5 from table 3 | 1T2R | 1+1 (2,11) | 14 | Type A | TDLB 100-400 |  |  |
| 30KHz 40MHz | MCS 5 from table 3 | 1T2R | 1+1 (0,10) | 14 | Type B | TDLB 100-400 |  |  |

3.3	Results for low latency

Table 4. Minimum requirement with low latency

| SCS/BW | MCS | Tx/Rx | Number DMRS | Symbol length | Mapping type | Channel | Ideal SNR with 70% TP | Impairment SNR with 70% TP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15KHz 5MHz | MCS 10 from table 3 | 1T2R | 1(0) | 2 | Type B | TDLC 300-100 | -1.26 | 0.74 |
| 15KHz 10MHz | MCS 10 from table 3 | 1T2R | 1(0) | 2 | Type B | TDLC 300-100 | -1.71 | 0.29 |
| 30KHz 10MHz | MCS 10 from table 3 | 1T2R | 1(0) | 2 | Type B | TDLC 300-100 | -1.27 | 0.73 |
| 30KHz 40MHz | MCS 10 from table 3 | 1T2R | 1(0) | 2 | Type B | TDLC 300-100 | -1.68 | 0.32 |

# 4 Conclusion

In this contribution, the view on the remaining issue of BS URLLC requirement are presented. Meanwhile, the initial simulation results are provided to facilitate the requirement alignment.

Proposal 1: Deprioritize FR2 URLLC requirement for ultra-lower BLER

Proposal 2: Only 15KHz SCS tested for ultra-lower BLER requirement if BS supports both 15 KHz SCS and 30 KHz SCS

Proposal 3: The RV sequence with 4 HARQ transmission is 0 3 0 3 under full buffer scenario

Observation 1: Configured PUSCH aggregation factor with n8 is not a typical scenario to meet the URLLC requirement with high reliability and low latency for 15 KHz SCS with TDD patter DDDSU.

Proposal 4: No PUSCH aggregation requirement for TDD with 15 KHz SCS

Observation 2: The RV value for effective UL transmission with PUSCH aggregation level n8 is same as TDD 30 KHz with PUSCH aggregation level n2.

Proposal 5:  If the slot aggregation requirement with n8 for TDD with 15KHz SCS is introduced, adding a note as

- The requirement with PUSCH aggregation level n8 for TDD with 15 KHz SCS can be applied with FDD 15KHz SCS or TDD 30 KHz SCS with PUSCH aggregation level n2.

Proposal 6: URLLC tests for requirement of high reliability with high BLER shall be done for either FR1 or FR2 based on BS declaration if BS declares to support both FR1 and FR2.

Proposal 7: Only define FR2 URLLC requirement with 60 KHz SCS for 50MHz, and 120 KHz SCS for 50MHz

Observation 3:  4 or 7 OS is assumption for baseline performance evaluation for most of URLLC use cases.

Observation 4:  Mini-slot repetition with 4OS is the typical scenario in RAN1 discussion to supporting dynamic switch between mini-slot repetition and multi-segments

Observation 5:  Minor performance difference existed for DMRS configuration with 1 and 1+1 for eMBB in FR2

Proposal 8:  Non-slot scheduling with 4 symbols can be considered for the lower latency requirement for FR2

Proposal 9: URLLC tests for low latency requirement shall be done for either FR1 or FR2 based on BS declaration if BS declares to support both FR1 and FR2.

Proposal 10: Only define FR2 URLLC requirement with 60 KHz SCS for 50MHz, and 120 KHz SCS for 50MHz

Proposal 11: Prioritize the performance requirements specification based on Rel-15 URLLC feature.

Proposal 12: FFS on the test methodology for test metric of 99.9999% and low latency requirements (0.5ms~1ms), FFS on the performance requirements based on Rel-16 URLLC functionalities.

Proposal 13: Samsung can voluntarily take some CR drafting work for FR2 URLLC requirement

# References

R4-2008805, Way Forward on ultra-low BLER requirements, Ericsson

R4-2008810, Way Forward on NR BS URLLC performance requirements, Huawei

# Annex

Table 5. Simulation assumption for ultra-low BLER requirement

| Parameters |  | Values |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Transform precoding |  | Disabled |
| Antenna configuration |  | 1x2 ULA Low |
| PUSCH configuration | PUSCH mapping type | Type A and Type B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | 1 |
| PUSCH DMRS configuration | DMRS type | Type 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | 1 |
| Number of HARQ transmission |  | 1 |
| PT-RS |  | Disabled |
| MCS table |  | Table 3, MCS 5 |
| SCS and BW |  | 15KHz for 10MHz, 30KHz for 40MHz |
| Propagation condition |  | AWGN |
| Maximum number of HARQ transmissions |  | 1 |
| TDD pattern |  | 15KHz SCS: 3D1S1U, S=10:2:2, 30KHz SCS: 7D1S2U: S=6:4:4 |
| Frequency domain resource assignment |  | Full bandwidth |
| Test metric |  | Target BLER : 10^-5 |

Table 6. Simulation assumption for high reliability with high BLER

| Parameters |  | Values |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Transform precoding |  | Disabled |
| Antenna configuration |  | 1x2 ULA Low |
| PUSCH configuration | PUSCH mapping type | Type A and Type B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n2 for 30Hz SCS FFS for 15KHz SCS |
| PUSCH DMRS configuration | DMRS type | Type 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | 1 |
| SCS & BW |  | 15KHz for 5/10MHz; 30KHz for 10/40MHz |
| TDD pattern |  | 15KHz SCS: 3D1S1U, S=10:2:2, 30KHz SCS: 7D1S2U; S=6:4:4 |
| MCS table |  | Table 3, MCS 5 |
| Propagation condition |  | TDLb100-400 |
| Maximum number of HARQ transmissions |  | 4 |
| Frequency domain resource assignment |  | Full bandwidth |
| Test metric |  | Target BLER : 10^-2 (Calculate the target BLER after all transmission) |

Table 7. Simulation assumption for low latency requirement

| Parameters |  | Values |
| --- | --- | --- |
| Frequency range |  | FR1 |
| Transform precoding |  | Disabled |
| Antenna configuration |  | 1x2 ULA Low |
| PUSCH configuration | PUSCH mapping type | Type B |
|  | Start symbol | 0 |
|  | Allocation length | 2 |
|  | PUSCH aggregation factor | 1 |
| PUSCH DMRS configuration | DMRS type | Type 1 |
|  | DM-RS duration | Single-symbol DM-RS |
|  | Number of additional DMRS | 0 |
| SCS & BW |  | 15KHz for 5/10MHz; 30KHz for 10/40MHz |
| TDD pattern |  | 15KHz SCS: 3D1S1U, S=10:2:2, 30KHz SCS: 7D1S2U; S=6:4:4 |
| MCS table |  | Table 3, MCS 10 |
| Propagation condition |  | TDLC 300-100 Low |
| Maximum number of HARQ transmissions |  | 1 |
| Frequency domain resource assignment |  | Full bandwidth for MCS 10 |
| Test metric |  | 70% TP |