3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2010356

# Electronic Meeting, 17th– 28th August, 2020

Source:	Verizon, Nokia, Samsung, Ericsson, Qualcomm

Title:	TP for TR 38.717-02-01: CA_n77-n260 and DC_n77-n260

Agenda item:	10.2.3

Document for:	Approval

# Introduction

In RAN#88-e meeting, the “New WID on NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2)” [1] was approved. This contribution is a text proposal for 38.717-02-01 for configuration of CA_n77-n260 and DC_n77-n260 band combination.

# Reference

[1] RP-200672: 	New WID on NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1, 2), ZTE

# Text Proposal

<Start of Text Proposal>

## 8.x			CA_n77-n260

### 8.x.1		Common for 1 band UL and 2 bands UL CA

#### 8.x.1.1		Operating bands for CA

Table 8.x.1.1-1: CA band combination of band n77+n260

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n77 | 3300 MHz | – | 4200 MHz | 3300 MHz | – | 4200 MHz | TDD |
| n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |

#### 8.x.1.2	Channel bandwidths per operating band for CA

Table 8.x.1.2-1: Supported bandwidths per CA band combination of band n77+n260

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | NR Uplink CA configuration | NR Band | SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz | 200 MHz | 400 MHz | BCS |
| CA_n77A-n260A | CA_n77A-n260A | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |  |  |
|  |  | n260 | 60 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes |  |  |
|  |  |  | 120 |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes | Yes | Yes |  |
| CA_n77A-n260G | CA_n77A-n260A CA_n77A-n260G | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |  |  |
|  |  | n260 | See CA_n260G in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n260H | CA_n77A-n260A CA_n77A-n260G CA_n77A-n260H | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |  |  |
|  |  | n260 | See CA_n260H in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n260I | CA_n77A-n260A CA_n77A-n260G CA_n77A-n260H CA_n77A-n260I | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |  |  |
|  |  | n260 | See CA_n260I in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n260J | CA_n77A-n260A CA_n77A-n260G CA_n77A-n260H CA_n77A-n260I | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |  |  |
|  |  | n260 | See CA_n260J in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n260K | CA_n77A-n260A CA_n77A-n260G CA_n77A-n260H CA_n77A-n260I | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |  |  |
|  |  | n260 | See CA_n260K in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n260L | CA_n77A-n260A CA_n77A-n260G CA_n77A-n260H CA_n77A-n260I | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |  |  |
|  |  | n260 | See CA_n260L in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_n77A-n260M | CA_n77A-n260A CA_n77A-n260G CA_n77A-n260H CA_n77A-n260I | n77 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes4 | Yes | Yes4 | Yes |  |  |  |
|  |  | n260 | See CA_n260M in Table 5.5A.1-1 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### 8.x.1.3		UE Co-existence studies

Table 8.x.1.3-1 lists up to 5th harmonics for CA_n77A-n260A.

Table 8.x.1.3-1: Band n77 and Band n260 UL harmonics products

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |
| n260 | 37000 | 40000 | 37000 | 40000 | 74000 | 80000 | 111000 | 120000 | 148000 | 160000 | 185000 | 200000 |

Based on Table 8.x.1.3-1, there is no harmonic skirt interference found to impact the receive frequency of each band. And, no further studies are needed for this combination.

#### 8.x.1.4		∆TIB and ∆RIB values

For CA_n77A-n260A, the TIB,c and RIB values are given in the tables below.

Table 8.x.4-1: ΔTIB,c

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n77-n260 | n77 | 0 |
|  | n260 | 0 |

Table 8.x.4-2: ΔRIB

| Inter-band CA Configuration | NR Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_n77-n260 | n77 | 0 |
|  | n260 | 0 |

#### 8.x.1.5		REFSENS requirements

There are no specific REFSENS requirements for this configuration.

### 8.x.2		Specific for 2 bands UL CA

#### 8.x.2.1		UE co-existence studies

Table 8.x.2.1-1 lists Band n77+Band n260 2UL bands CA  2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 3300 | 4200 | 37000 | 40000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 32800 | 36700 | 40300 | 44200 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 33400 | 28600 | 69800 | 76700 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 43600 | 48400 | 77300 | 84200 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 30100 | 24400 | 106800 | 116700 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 46900 | 52600 | 114300 | 124200 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 73400 | 65600 | 80600 | 88400 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 156700 | 143800 | 20200 | 26800 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 151300 | 164200 | 50200 | 56800 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 113400 | 102600 | 61400 | 70100 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 117600 | 128400 | 83900 | 92600 |

Based on Table 8.x.2.1-1, there is no IMD interference found to impact this TDD bands combination. And, no further studies are needed for this combination

Table 8.x.2.1-2 lists the protected bands required for the 2UL bands CA configuration.

Table 8.x.2.1-2: Protected bands for the 2UL bands CA configuration

| UL NR CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n77A-n260A | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 25, 26, 29, 30, 65, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

#### 8.x.2.2		REFSENS requirements

There are no need for additional REFSENS requirements for this configuration.

<Start of Text Proposal>

<Start of Text Proposal>

## 9.x	DC_n77-n260

### 9.x.1	Operating bands for DC_n77-n260

Table 9.x.1-1: Inter-band NR DC operating bands

| NR DC Band | NR Band |
| --- | --- |
| DC_n77-n260 | n77, n260 |

### 9.x.2   Configurations for DC_n77-n260

Table 9.x.2-1: Inter-band NR DC configurations

| NR DC configuration | Uplink NR DC configuration |
| --- | --- |
| DC_n77A-n260A | DC_n77A-n260A |
| DC_n77A-n260G | DC_n77A-n260A DC_n77A-n260G |
| DC_n77A-n260H | DC_n77A-n260A DC_n77A-n260G DC_n77A-n260H |
| DC_n77A-n260I | DC_n77A-n260A DC_n77A-n260G DC_n77A-n260H DC_n77A-n260I |
| DC_n77A-n260J | DC_n77A-n260A DC_n77A-n260G DC_n77A-n260H DC_n77A-n260I |
| DC_n77A-n260K | DC_n77A-n260A DC_n77A-n260G DC_n77A-n260H DC_n77A-n260I |
| DC_n77A-n260L | DC_n77A-n260A DC_n77A-n260G DC_n77A-n260H DC_n77A-n260I |
| DC_n77A-n260M | DC_n77A-n260A DC_n77A-n260G DC_n77A-n260H DC_n77A-n260I |

### 9.x.3   Maximum output power for NR-DC

| Uplink CA Configuration | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_n77A-n260A | 23 | +2/-3x |
| DC_n77A-n260G | 23 | +2/-3x |
| DC_n77A-n260H | 23 | +2/-3x |
| DC_n77A-n260I | 23 | +2/-3x |
| NOTE x: The power class and Pcmax for this FR1 and FR2 are defined independently in TS38.101-1 and TS38.101-2. This power tolerance is limited for FR1 and referred the Table 6.2.1-1 in 38.101-1 as the FR2 power classes are in a minimum peak EIRP (refer Table 6.2D.1.3-1 in 38.101-2) |  |  |

<Start of Text Proposal>