3GPP TSG-RAN WG4 Meeting #96-e	R4-2010471

Electronic Meeting, 17 – 28 August 2020

Agenda Item:	6.1.3.1

Source:	Ericsson

Title:	Simulation results of MPDCCH with DMRS+CRS

Document for:	Information

# 1	Introduction

RAN4#95-e agreed with the simulation assumption of MPDCCH demodulation requirements with DMRS+CRS [1]. This contribution provides our initial simulation results for both FDD and TDD according to the simulation assumption [2]

# 2	MPDCCH performance improvement

Table 1 and Table 2 show the simulation parameters for MPDCCH according to [1].

## 2.1	Simulation parameters

Table 1	Simulation parameters for MPDCCH performance improvements.

| Parameter |  | Unit | Test 1 (CE Mode A) | Test 2 (CE Mode B) |
| --- | --- | --- | --- | --- |
| OFDM starting symbol (startSymbolLC) |  | symbols | 2 | 2 |
| Unused RE-s and PRB-s |  |  | OCNG | OCNG |
| Cell ID |  |  | 0 | 0 |
| Downlink power allocation |  | dB | -3 | 0 |
|  |  | dB | -3 | 0 |
|  |  | dB | 0 | -3 |
|  | δ | dB | 3 | 0 |
| at antenna port |  | dBm/15kHz | -98 | -98 |
| Cyclic prefix |  |  | Normal | Normal |
| Subframe Configuration |  |  | Non-MBSFN | Non-MBSFN |
| Precoder Update Granularity |  | PRB | N/A | See TS36.211 6.8B.5 |
|  |  | ms | N/A | See TS36.211 6.8B.5 |
| Beamforming Pre-Coder |  |  | See TS36.211 6.8B.5 | See TS36.211 6.8B.5 |
| Cell Specific Reference Signal |  |  | Port 0 and 1 | Port 0 and 1 |
| Number of PRB per MPDCCH Set |  |  | 4 | 2+4 |
| Transmission type |  |  | Distributed | Localized |
| Frequency hopping |  |  | Disabled | Enabled |
| Number of frequency hopping narrowbands |  |  | N/A | 4 |
| Frequency hopping offset |  |  | N/A | 1 |
| Frequency hopping interval |  | ms | N/A | 16 |
| Value of G in MPDCCH start subframe (mpdcch-startSF-UESS) (Note 3) |  |  | 1.5 | 1.5 |
| Maximum number of repetitions (mPDCCH-NumRepetition) |  |  | 16 | 32 |
| MPDCCH repetition number |  |  | 16 | 32 |
| MPDCCH narrowband (mpdcch-Narrowband) |  |  | 1 | 7 |
| PDSCH TM |  |  | TM2 | TM2 |
| DCI Format |  |  | 6-1A | 6-1B |
| fdd-DownlinkOrTddSubframeBitmapBR |  |  | 1111111111 | 1111111111 |
| mpdcch-crs-config |  |  | Configured | Configured |
| Power offset between CRS and DMRS antenna ports of MPDCCH |  | dB | 0 | 0 |
| mpdcch-crs-localized-mapping-type |  |  | N/A | Not configured |

Table 2	Test case list for MPDCCH performance improvements.

| Test number | Bandwidth | Aggregation level | Reference Channel (TS 36.101) | Propagation condition | Antenna configuration | Pm-dsg (%) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 10MHz | 16 ECCE | R.82 FDD | EPA5 | 2x1 low | 1 |
| 2 | 10MHz | 24 ECCE | R.83 FDD | ETU1 | 2x1 low | 1 |

## 2.2	Simulation results

Table 3 shows our simulation results for enhanced MPDCCH. We propose to consider the simulation results to define the enhanced MPDCCH demodulation requirements.

Table 3	Simulation results of MPDCCH demodulation based on CRS+DMRS.

|  | CE Mode A |  | CE Mode B |  |
| --- | --- | --- | --- | --- |
|  | FDD/HD-FDD | TDD | FDD/HD-FDD | TDD |
| Alignment results | -5.9 | -7.4 | -12.3 | -12.3 |
| Impairment results | -3.9 | -5.4 | -10.3 | -10.3 |

# 3	References

- R4-2008758, “Way forward on UE/BS demodulation performance for additional MTC enhancements for LTE”, Ericsson.

- R4-2005513, “Way forward on UE/BS demodulation performance for additional MTC enhancements for LTE”, Ericsson.