3GPP TSG-RAN WG4 Meeting # 96-e	R4-2010490

# Electronic Meeting, 17-28 Aug., 2020

Source: 	Huawei, HiSilicon

Title:	A-MPR for n13

Agenda Item:	10.21.1

Document for:	Discussion

1	Introduction

In this contribution we provide simulation results on A-MPR for NR band n13.

2	Discussion

2.1 Simulation assumptions

  - CIM3: 60 dBc.

  - IQ image: 28 dB.

  - LO leakage: 28 dBc.

  - PA calibration point: 10 MHz, 15 kHz, QPSK, DFT-S-OFDM, 50 RB with 1 dB MPR

2.2 Additional emission requirements

It was approved in R4-2004464 to re-use the LTE network signaling parameter NS_07 using the supported bandwidths provided in the WID as shown in Table 1 and Table 2.

Table 1: Additional SEM requirements NS_07

| ΔfOOB (MHz) | 5 MHz | 10 MHz | Measurement bandwidth |
| --- | --- | --- | --- |
| ± 0 – 0.1 | -15 | -18 | 30 kHz |
| ± 0.1 – 1 | -13 | -13 | 100 kHz |
| ± 1 – 6 | -13 | -13 | 1 MHz |
| ± 6 – 10 | -25 |  |  |
| ± 10 – 15 |  | -25 |  |

Table 2: Additional requirements NS_07

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit- (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz |  |
| 769 ≤ f ≤ 775 | -57 | 6.25 kHz |
| NOTE 1: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

2.3 Simulation results

Simulation results are shown in the following figures.

Figure 2-1: simulation results for 5 MHz with Fc=779.5 MHz

Figure 2-2: simulation results for 5 MHz with Fc=782 MHz

Figure 2-3: simulation results for 5 MHz with Fc=784.5 MHz

Figure 2-4: simulation results for 10 MHz with Fc=782 MHz

2.4 A-MPR table

Table 3: A-MPR regions for NS_07

| Channel Bandwidth, MHz | Carrier Frequency, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBstart*12*SCS MHz | LCRB*12*SCS MHz |  |
| 5 MHz | 782 ≤ Fc ≤ 784.5 |  |  |  |
|  |  | >0 | ≥ 1.8 | A3 |
| 5 MHz | 779.5 ≤ Fc < 782 |  |  |  |
|  |  | ≤ 0.9 | ≥ 0 | A1 |
|  |  | > 0.9, ≤ 1.26 | ≥1.26 | A2 |
|  |  | > 1.26 | ≥ 1.8 | A3 |
| 10 MHz | Fc = 782 | ≤ 2.34 | ≥ 0 | A1 |
|  |  | >2.34, ≤ 3.24 | ≥ 1.44 | A2 |
|  |  | > 3.24, ≤ 7.56 | ≥ 3.24 | A3 |
|  |  | > 7.56 | ≤ 0.36 | A4 |

Table 4: A-MPR for modulation and waveform type

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | [12] | [8] | [6] | [3] |
| DFT-s-OFDM QPSK | [12] | [8] | [6] | [3] |
| DFT-s-OFDM 16 QAM | [12] | [8] | [6] | [3] |
| DFT-s-OFDM 64 QAM | [12] | [8] | [6] | [3] |
| DFT-s-OFDM 256 QAM | [12] | [8] | [6] | [3] |
| CP-OFDM QPSK | [14] | [9] | [7] | [3] |
| CP-OFDM 16 QAM | [14] | [9] | [7] | [3] |
| CP-OFDM 64 QAM | [14] | [9] | [7] | [3] |
| CP-OFDM 256 QAM | [14] | [9] | [7] | [3] |

3	Conclusions

In this contribution we provide simulation results and A-MPR proposal on NS_07 for NR band n13.