3GPP TSG-RAN WG4 Meeting #96-e								          R4-2010499

Online, 17th – 28th August 2020

Source: 	Huawei, HiSilicon

Title: 	Proposals on 100MHz CBW in NR-U

Agenda Item:	7.1.1.3

Document for:	Approval

Introduction

In this contribution, we discuss the remaining issues for 100MHz CBW in NR-U.

Discussion

In RAN4 #91, we have agreed that for type A multi LBT sub-band access, 60MHz CBW for band n46 might not be an issue. Similarly, if using type A multi LBT sub-band channel access, there is also no coexistence issue for 100MHz CBW in band n46. It is unreasonable not to define the channel raster for 100MHz CBW in band n46. The following channel raster for NRU with 100MHz bandwidth should be supported.

Table.1 channel raster for 100MHz CBW in NRU

| Channel frequency (MHz) | ARFCN | Reference frequency (MHz) |
| --- | --- | --- |
| 5200 | 746668 | 5200.02 |
| 5300 | 753332 | 5299.98 |
| 5520 | 768000 | 5520 |
| 5540 | 769332 | 5539.98 |
| 5560 | 770668 | 5560.02 |
| 5580 | 772000 | 5580 |
| 5620 | 774668 | 5620.02 |
| 5640 | 776000 | 5640 |
| 5660 | 777332 | 5659.98 |
| 5680 | 778668 | 5680.02 |
| 5785 | 785668 | 5785.02 |
| 5865 | 791000 | 5865 |

Observation 1: If using Type A multi LBT sub-band channel access, there is no issue for 100MHz CBW in band n46.

Proposal 1: Channel raster for 100MHz CBW in NRU as listed in Table.1 should be supported.

In RAN4 #93 meeting, the spectrum emission mask was agreed in [1]. The general mask in Figure.1 also applies for 100MHz CBW, for the punctured emission mask, similar requirement should be approved:

- for single punctured channel in the middle, the emission mask of the puncture center is limited at -23dBr

- for multiple punctured channels in the middle, the emission mask in the middle is floored at -25dBr

- for punctured channel(s) at the edge, the emission mask edge is floored at -28dBr.

Figure.1 general spectrum emission mask in NRU

Proposal 2: The spectrum emission mask for 100MHz channel bandwidths in NRU should be supported as below:

- for full bandwidth transmission, the general spectrum emission mask in NRU is applied.

- for single punctured channel in the middle, the emission mask of the puncture center is limited at -23dBr.

- for multiple punctured channels in the middle, the emission mask in the middle is floored at -25dBr.

- for punctured channel(s) at the edge, the emission mask edge is floored at -28dBr.

Conclusions

In this contribution, we discussed the remaining issues for channel arrangement in NR-U, and the following observation and proposal was made:

Observation 1: If using Type A multi LBT sub-band channel access, there is no issue for 100MHz CBW in band n46.

Proposal 1: Channel raster for 100MHz CBW in NRU as listed in Table.1 should be supported

Proposal 2: The spectrum emission mask for 100MHz channel bandwidths in NRU should be supported as below:

- for full bandwidth transmission, the general spectrum emission mask in NRU is applied.

- for single punctured channel in the middle, the emission mask of the puncture center is limited at -23dBr.

- for multiple punctured channels in the middle, the emission mask in the middle is floored at -25dBr.

- for punctured channel(s) at the edge, the emission mask edge is floored at -28dBr.

References

R4-1915979  WF on NR-U spectrum emission mask

38.101-1 V16.0.0  Base Station (BS) radio transmission and reception

TR 36.889 Study on Licensed-Assisted Access to Unlicensed Spectrum