3GPP TSG-RAN WG4 Meeting #96-e	R4-2010504

Electronic Meeting, 17-28 Aug., 2020

Source: 	Huawei, HiSilicon

Title:	Overview on introduction of channel bandwidths 35MHz and 45MHz for UE

Agenda Item:	10.18.3

Document for:	Discussion

1	Introduction

A new WI on introduction of channel bandwidths 35MHz and 45MHz for NR was approved in RAN#88-e [1]. This contribution provides an overview on UE RF requirements for 35MHz and 45MHz.

- 35MHz for NR band n3, n7, n8, n25, n66, n71

- 45MHz for NR band n3, n25, n66.

2	Discussion

The common part for both BS and UE is SU which is discussed in [2]. Based on the outcome of discussion on SU, maximum transmission bandwidth configuration and minimum guardband can be derived.

On UE RF aspect, the requirements can be divided into 2 parts:

- UE RF general requirements for TX and RX

- Band specific requirements

  - UE Channel bandwidth per operating band

  - UE reference sensitivity

  - When needed:

    - MPR (relative bandwidth criteria)

    - Additional Maximum Power Reduction (A-MPR)

On UE RF general requirements perspective, the following sub-clauses are expected to be updated for the addition of 35 and 45 MHz channel bandwidths, which should not be controversial and can be reviewed at the CR drafting phase.

- 6.3.2	Transmit OFF power

- 6.5.1	Occupied bandwidth

- 6.5.2.2	Spectrum emission mask

- 6.5.2.4	Adjacent channel leakage ratio

- 7.4	Maximum input level

- 7.5	Adjacent channel selectivity

- 7.6	Blocking characteristics

- 7.7	Spurious response

- 7.8	Intermodulation characteristics

On band specific requirements, we need look into the reference sensitivity, MPR and A-MPR case by case. An overview is summarized in Table 2-1.

Table 2-1 Overview on band specific requirements

| Band | Channel Bandwidth | REFSEMS | MPR(relative bandwidth criteria) | A-MPR |
| --- | --- | --- | --- | --- |
| n3 | 35 | UL configuration, MSD | Relative channel bandwidth=2 %, OK | No A-MPR requirement will be specified, same as the case for 40 MHz CBW |
|  | 45 | UL configuration, MSD | Relative channel bandwidth=2.6 %, OK |  |
| n7 | 35 | UL configuration | Relative channel bandwidth=1.4 %, OK | A-MPR for NS_46 need to be studied |
| n8 | 35 | UL configuration, MSD | Relative channel bandwidth=3.9 %, MPR for 35 MHz need to be studied | A-MPR for NS_43 and NS_43U need to be studied |
| n25 | 35 | UL configuration, MSD | Relative channel bandwidth=1.9 %, OK | A-MPR for NS_03 and NS_03U need to be studied |
|  | 45 | UL configuration, MSD | Relative channel bandwidth=2.4 %, OK |  |
| n66 | 35 | No MSD | Relative channel bandwidth=2 %, OK | A-MPR for NS_03 and NS_03U need to be studied |
|  | 45 | UL configuration | Relative channel bandwidth=2.6 %, OK |  |
| n71 | 35 | UL configuration, MSD | Relative channel bandwidth=5.1 %, MPR for 35 MHz need to be studied | A-MPR for NS_35 need to be studied |

3	Conclusions

This contribution provides an overview on UE RF requirements for 35MHz and 45MHz.

Reference

[1] RP-201321, “New WID on introduction of channel bandwidths 35MHz and 45MHz for NR”, Huawei, HiSilicon, Etisalat

[2] R4-2010503, Spectrum utilization for 35MHz and 45MHz, Huawei, HiSilicon,