3GPP TSG-RAN WG4 Meeting #96-e	R4-2010579

Electronic Meeting, August 17 – 28, 2020

Source: 	NTT DOCOMO, INC.

Title:	TP for CA 3DL1UL n1-n78-n79 for TR 38.717-03-01

Agenda item:	10.9.2

Document for:	Approval

- Introduction

This paper provides TP for CA_n1-n78-n79 for TR38.717-03-01. Co-existence studies and MSD analysis can be omitted because harmonic interference have been already studied in lower order 3DL/1ULCA combinations.

- Text proposal

< Start of Text Proposal>

6.x		CA_n1-n78-n79

6.x.1	Operating bands for CA

Table 6.x.1-1: 3DL Inter-band CA operating bands

| NR CA Band | NR Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_n1-n78-n79 | n1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n79 | 4400 MHz | – | 5000 MHz | 4400 MHz | – | 5000 MHz | TDD |

6.x.2	Channel bandwidths per operating band for CA

Table 6.x.2-1: Supported channel bandwidths per CA configuration for 3DL inter-band CA

| NR CA config | UL config | NR Band | SCS (kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 40 | 50 | 60 | 70 | 80 | 90 | 100 | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1A-n78A-n79A | - | n1 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | n78 | 15 |  | Yes | Yes | Yes |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |
|  |  | n79 | 15 |  |  |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  |  | 30 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |
|  |  |  | 60 |  |  |  |  |  |  | Yes | Yes | Yes |  | Yes |  | Yes |  |

NOTE: 	For the UE that signals support of any bandwidth combination set for carrier aggregation, the UE shall support all single carrier bandwidths for the constituent bands as defined in Table 5.3.5-1 of TS 38.101-1 and in Table 5.3.5-1 of TS 38.101-2 when operating in single carrier mode.

6.x.3	Co-existence studies

Co-existence studies can be omitted because harmonic interference from n1 to n78 and n79, and from n78 to n1 and n79, and from n79 to n1 and n78 have been already studied for 3DL/1UL fallback combinations CA n1-n78 and CA_n1-n79 and n78-n79.

6.x.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band n1, n78 and n79, the TIB,c and RIB,c values are shown in table 6.x.4-1 and table 6.x.4-2, respectively.

Table 6.x.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_n1-n78-n79 | n1 | 0.3 |
|  | n78 | 0.8 |
|  |  | 1.58 |
|  | n79 | 0.5 |
|  |  | 1.58 |
| NOTE 8: The requirements only apply for UE supporting inter-band carrier aggregation with simultaneous Rx/Tx capability, and NR UL carrier frequencies are confined to 3700 MHz-3800MHz for n78 and 4400 MHz-4500MHz for n79. Simultaneous Rx/Tx capability does not apply for UEs supporting band n78 with a n77 implementation. |  |  |

Table 6.x.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_n1-n78-n79 | n1 | 0 |
|  | n78 | 0.5 |
|  | n79 | 0 |

6.x.5	REFSENS requirements

MSD studies can be omitted because harmonic interference from n1 to n78 and n79, and from n78 to n1 and n79, and from n79 to n1 and n78 have been already studied for 3DL/1UL fallback combinations CA n1-n78 and CA_n1-n79 and n78-n79.

< End of Text Proposal>

- References