3GPP TSG-RAN WG4 Meeting #96-e	R4-2010969

Electronic Meeting, 17-28 Aug., 2020

Title: 	Simulation results for NPDSCH with multi-TB interleaved transmission

Source: 	Huawei, HiSilicon

Agenda item:	6.2.3.1

Document for:	Discussion

Background

At the last meeting, we agreed to introduce the performance requirements for NPDSCH with multi-TB interleaved transmission. In this paper, we provide our simulation results for NPDSCH with multi-TB interleaved transmission as per WF [1].

Simulation results

Table 1 is summary of the simulation assumptions for NPDSCH we have agreed as per WF [1]

Table 1: Simulation assumptions for NPDSCH

| Parameter Parameter | Value |
| --- | --- |
| System bandwidth | 200 kHz |
| Operation mode | Stand alone |
| Antenna configuration | 1T1R |
| Channel model | ETU 1Hz |
| Frequency error | 0 Hz |
| Timing error | 0μs |
| Performance target | SNR@70% of maximum throughput |
| NSF | 10 |
| Repetition number | 32 |
| Transmission mode | Interleaved |
| HARQ process | 2 |
| Carrier Type | Non-anchor |
| TB size | 1032 |
| Duplex mode | HD-FDD |

Table 2 is the summary of simulation results.

Table 2: Simulation results for NPDSCH

| TBS(bit) | NSF | NRep | Antenna | SNR@70% of maximum throughput(dB) |
| --- | --- | --- | --- | --- |
| 1032 | 10 | 32 | 1T1R | -8.7 |

Conclusion

In this contribution, we provide our simulation results for NPDSCH with multi-TB interleaved transmission

Reference

- R4-2008759	 WF on LTE UE and BS performance requirements for additional enhancements of NB-IoT