3GPP TSG-RAN4 Meeting #97e	R4-2014236

# Online, Nov. 02-13, 2020

Agenda item:	7.14.1

Source:	Apple

Title:   On remaining core issues of CSI-RS for L3 measurements

Document for:	Approval

# Introduction

In RAN4 #96e meeting, good progress has been made and core part completion is also confirmed in RAN#89e. However, there are still a few remaining issues to be resolved in Rel-16, including

- FFS on if both scenarios or one of them are considered in R16

  - Scenario 1: CSI-RS resources and SSB are fully or partially overlapped in time domain

  - Scenario 2: CSI-RS resources and SSB are non-overlapped in time domain

- Scaling factor N = [8] for RX beam sweeping in FR2 band

- When UE performs CSI-RS intra-frequency measurements in a TDD band

  - FFS: UE is not expected to transmit on data OFDM symbols overlapped by CSI-RS resource symbols to be measured, and [1] OFDM symbols before and after each consecutive CSI-RS symbols

In this contribution, our proposals on these issues are provided.

# CSI-RS and SSB overlapping in time domain

Two scenarios are considered

        - Scenario 1: CSI-RS resources and SSB are fully or partially overlapped in time domain

Scenario 2: CSI-RS resources and SSB are non-overlapped in time domain

Providing it has been agreed that CSI-RS and SSB are configured as separated MO, general measurement opportunity sharing mechanism can also apply to both Scenario 1 and 2.

Proposal 1: Specify requirements for both scenario 1 and 2.

Proposal 2: CSSF frame work can generally apply to both scenarios

Proposal 3: CSI-RS and SSB for L3 measurement, including gap based and non-gap based, equally share the measurement opportunities for both scenarios .

# Scaling factor for RX beam sweeping in FR2 band

Proposal 4: Scaling factor of 8 for Rx beam sweeping in FR2 bands

# When UE performs CSI-RS intra-frequency measurements in a TDD band

Proposal 5: When UE performs CSI-RS intra-frequency measurements in a TDD band

- UE is not expected to transmit on data OFDM symbols overlapped by CSI-RS resource symbols to be measured, and 1 OFDM symbols before and after each consecutive CSI-RS symbols

# Conclusion

In this contribution, the remaining issues on CSI-RS L3 measurement are discussed. It is proposed

Proposal 1: Specify requirements for both scenario 1 and 2.

Proposal 2: CSSF frame work can generally apply to both scenarios

Proposal 3: CSI-RS and SSB for L3 measurement, including gap based and non-gap based, equally share the measurement opportunities for both scenarios.

Proposal 4: Scaling factor of 8 for Rx beam sweeping in FR2 bands

Proposal 5: When UE performs CSI-RS intra-frequency measurements in a TDD band

- UE is not expected to transmit on data OFDM symbols overlapped by CSI-RS resource symbols to be measured, and 1 OFDM symbols before and after each consecutive CSI-RS symbols