3GPP TSG-RAN WG4 Meeting # 97-e	R4-2014476

Electronic Meeting, 2 – 13 November 2020

Agenda Item:	11.1.3.2

Source: 	Nokia, Nokia Shanghai Bell

Title: 	Downlink Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz

Document for:	Discussion

# 1.	Introduction

The study item on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz was approved at TSG RAN#87-e [1]. The purpose of this study item is to study the IMT parameters relevant for sharing and compatibility for the following frequencies:

-	6425-7025MHz and 7025-7125MHz.

-	10000-10500MHz.

This study item aims as answering requests from ITU-R WP5D regarding NR in these frequencies for IMT [2]. Two set of parameters requested by ITU-R WP5D are the ACLR and ACS of BS and UE in these frequencies. Currently, different sets of ACLR and ACS are specified in RAN4 specifications [3, 4, 5] for NR BS and UE in FR1 and FR2 based on coexistence studies as recorded in TR 38.803 [6]. Therefore, coexistence study will need to be carried out to provide answers to ITU-R WP5D on these parameters with sound technical justifications from coexistence perspective. The WF on the simulation assumptions for the coexistence study was agreed in RAN4#95-e [7] and RAN4#96-e [8].

This contribution provides the downlink coexistence simulation results according to the agreed assumptions in [7] and [8].

# 2.	Discussion

All simulation assumptions for the urban macro and indoor downlink scenario were agreed in [7] and [8], except the AAS BS antenna characteristics which point to the WF agreed in [9], these assumptions were used to obtain the simulation results in this contribution.

2.1	Urban Macro Scenario

The urban macro simulation results of the downlink SINR of the victim UE at 7GHz and 10GHz carrier frequency are provided in Figures 1 and 2 below. Here the downlink ACIR is obtained using 45dB BS ACLR and 33dB UE ACS (currently defined for BS and UE in FR1). It can be seen from the figures that in both cases enough downlink SINR (up to 65dB) can be provided using 43dBm BS conducted output power (as agreed in [7]) together with AAS BS beamforming, considering an additional 3dB gain is added to the total beamforming gain to account for the two polarization directions.

Figure 1: Urban Macro DL SINR of victim UE at 7GHz

Figure 2: Urban Macro DL SINR of victim UE at 10GHz

The urban macro simulation results of the downlink throughput loss of the victim UE with different ACIR offsets (compared to the ACIR using 45dB BS ACLR and 33dB UE ACS) at 7GHz and 10GHz carrier frequency are provided in Table 1 below. It can be seen from the table that the 5%-tile downlink throughput loss of the victim UE in the simulated urban macro scenario is higher than 5% using 45dB BS ACLR and 33dB UE ACS at 7GHz, while the downlink throughput loss of the victim UE in the simulated urban macro scenario can still be limited to 5% with downlink ACIR offsets of -2dB at 10GHz (i.e. 2dB less stringent ACIR).

| ACIR offset X [dB] | 0 | -1 | -2 | -3 | -4 | -5 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (7GHz) | 1.11% | 1.28% | 1.48% | 1.70% | 1.94% | 2.21% |
| 5%-tile throughput loss (7GHz) | 6.22% | 6.23% | 6.24% | 7.51% | 8.99% | 9.23% |
| Average throughput loss (10GHz) | 1.03% | 1.20% | 1.38% | 1.60% | 1.83% | 2.09% |
| 5%-tile throughput loss (10GHz) | 4.38% | 4.38% | 4.86% | 5.23% | 6.01% | 7.40% |

Table 1: Urban Macro DL throughput loss of victim UE at 7GHz and 10GHz

2.2	Indoor Hotspot Scenario

The indoor hotspot simulation results of the downlink SINR of the victim UE at 7GHz and 10GHz carrier frequency are provided in Figures 3 and 4 below. Here the downlink ACIR is obtained using 45dB BS ACLR and 33dB UE ACS (currently defined for BS and UE in FR1). It can be seen from the figures that in both cases enough downlink SINR (up to 40dB) can be provided using 24dBm BS conducted output power (as agreed in [7]) even without AAS BS beamforming.

Figure 3: Indoor Hotspot DL SINR of victim UE at 7GHz

Figure 4: Indoor Hotspot DL SINR of victim UE at 10GHz

The indoor hotspot simulation results of the downlink throughput loss of the victim UE with different ACIR offsets (compared to the ACIR using 45dB BS ACLR and 33dB UE ACS) at 7GHz and 10GHz carrier frequency are provided in Table 2 below. It can be seen from the table that the downlink throughput loss of the victim UE in the simulated indoor hotspot scenario can still be limited to 5% with a downlink ACIR offset of -1dB and -0.5dB, respectively, at 7GHz and 10GHz (i.e. 1dB and 0.5dB less stringent ACIR).

| ACIR offset X [dB] | 0 | -1 | -2 | -3 | -4 | -5 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (7GHz) | 0.56% | 0.66% | 0.78% | 0.91% | 1.06% | 1.24% |
| 5%-tile throughput loss (7GHz) | 4.23% | 4.80% | 5.92% | 6.63% | 8.50% | 9.24% |
| Average throughput loss (10GHz) | 0.56% | 0.66% | 0.78% | 0.91% | 1.07% | 1.25% |
| 5%-tile throughput loss (10GHz) | 4.42% | 5.13% | 6.10% | 7.10% | 8.59% | 9.13% |

Table 2: Indoor Hotspot DL throughput loss of victim UE at 7GHz and 10GHz

2.3	Dense Urban Scenario

It is agreed to down-prioritize the dense urban scenario in this coexistence study, as such the corresponding simulation assumptions and parameters are yet to be finalized. Here the simulation results are obtained using the simulation parameters proposed in [10] and the agreed antenna parameters in [11].

The dense urban simulation results of the downlink SINR of the victim UE at 7GHz and 10GHz carrier frequency are provided in Figures 5 and 6 below. Here the downlink ACIR is obtained using 45dB BS ACLR and 33dB UE ACS (currently defined for BS and UE in FR1). It can be seen from the figures that in both cases enough downlink SINR (up to 60dB) can be provided using 37dBm BS conducted output power (as agreed in [12]) together with AAS BS beamforming, considering an additional 3dB gain is added to the total beamforming gain to account for the two polarization directions.

Figure 5: Dense Urban DL SINR of victim UE at 7GHz

Figure 6: Dense Urban DL SINR of victim UE at 10GHz

The dense urban results of the downlink throughput loss of the victim UE with different ACIR offsets (compared to the ACIR using 45dB BS ACLR and 33dB UE ACS) at 7GHz and 10GHz carrier frequency are provided in Table 3 below. It can be seen from the table that the downlink throughput loss of the victim UE in the simulated dense urban scenario can still be limited to 5% with a downlink ACIR offset of -8dB and -12dB, respectively, at 7GHz and 10GHz (i.e. 8dB and 12dB less stringent ACIR).

| ACIR offset X [dB] | 0 | -2 | -4 | -6 | -8 | -10 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (7GHz) | 0.56% | 0.66% | 0.78% | 0.91% | 1.06% | 1.24% |
| 5%-tile throughput loss (7GHz) | 4.23% | 4.80% | 5.92% | 6.63% | 8.50% | 9.24% |
| Average throughput loss (10GHz) | 0.56% | 0.66% | 0.78% | 0.91% | 1.07% | 1.25% |
| 5%-tile throughput loss (10GHz) | 4.42% | 5.13% | 6.10% | 7.10% | 8.59% | 9.13% |

Table 3: Dense Urban DL throughput loss of victim UE at 7GHz and 10GHz

# 3.	Conclusion

This contribution has provided the downlink coexistence simulation results according to the agreed assumptions.

The simulation results have shown that:

1) The 5%-tile downlink throughput loss of the victim UE in the simulated urban macro scenario is higher than 5% using 45dB BS ACLR and 33dB UE ACS at 7GHz, while the downlink throughput loss of the victim UE in the simulated urban macro scenario can still be limited to 5% with downlink ACIR offsets of -2dB at 10GHz (i.e. 2dB less stringent ACIR).

2) The downlink throughput loss of the victim UE in the simulated indoor hotspot scenario can still be limited to 5% with a downlink ACIR offset of -1dB and -0.5dB, respectively, at 7GHz and 10GHz (i.e. 1dB and 0.5dB less stringent ACIR).

3) The downlink throughput loss of the victim UE in the simulated dense urban scenario can still be limited to 5% with a downlink ACIR offset of -8dB and -12dB, respectively, at 7GHz and 10GHz (i.e. 8dB and 12dB less stringent ACIR).

# References

[1]	RP-200513, “New SI proposal: Study on IMT parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5GHz”, Ericsson, Huawei, HiSilicon.

[2]	RP-200042, “LS on parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23”, ITU-R WP5D.

[3]	3GPP TS 38.101-1 v16.3.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4]	3GPP TS 38.101-2 v16.3.0, “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5]	3GPP TS 38.104 v16.3.0, “NR; Base Station (BS) radio transmission and reception”.

[6]	3GPP TR 38.803 v14.2.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”.

[7]	R4-2008928, “TP to TR 38.9xx: System level simulation methodology and assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.

[8]	R4-2011827, “WF on Simulation Assumptions for the SI on 6.425-7.125GHz and 10.0-10.5GHz”, Nokia.

[9]	R4-2008926, “WF on BS Antenna parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz”, Huawei, Nokia.

[10]	R4-2011195, “TP to TR 38.921: Dense urban system level simulation assumptions for study on IMT parameters for frequency ranges 6.425-7.125GHz and 10.0-10.5GHz”, Nokia, Nokia Shanghai Bell.

[11]	R4-2011932, “LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (6.425 to 10.5 GHz)”, Ericsson.

[12]	R4-2011829, “TP to TR 38.921: BS IMT technology related parameters”, Huawei, HiSilicon, Ericsson, Nokia, ZTE.