3GPP TSG RAN WG4 Meeting #97-e	R4-2014612

Electronic Meeting, 2-13 Nov., 2020

Agenda item:	10.4.2

Source:	SoftBank Corp.

Title:	TP for TR 37.717-21-11: EN-DC_1-42_n3

Document for:	Approval

1	Introduction

EN-DC of 2B LTE and 1B NR of DC_1-42_n3 was approved in RAN#88[1]. This TP is to capture the basic aspects for the EN-DC.

2	Combination proposed

Combinations proposed in [1] are as below.

Table 2-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-42A_n3A | DC_1A_n3A DC_42A_n3A |
| DC_1A-42C_n3A | DC_1A_n3A DC_42A_n3A DC_42C_n3A |

3.  Co-existence

When Uplink EN-DC configuration is DC_1A_n3A, IMD4 of (B1 - n3) will fall into Rx band of Band 42.

4.  Delta Tib/Rib

The delta Tib/Rib of subset EN-DC/CA are as follows. EN-DC values are picked.

Table 4-1: ΔTIB of constituent 2 band combo.

| ΔTIB | B1+B42 | B1+n3 | B42+n3 | Proposed Value |
| --- | --- | --- | --- | --- |
| B1 | 0.3 | 0.3 |  | 0.3 |
| B42 | 0.8 |  | 0.8 | 0.8 |
| n3 |  | 0.3 | 0.6 | 0.6 |

Table 4-2: ΔRIB of constituent 2 band combo.

| ΔRIB | B1+B42 | B1+n3 | B42+n3 | Proposed Value |
| --- | --- | --- | --- | --- |
| B1 | 0 | 0 |  | 0 |
| B42 | 0.5 |  | 0.5 | 0.5 |
| n3 |  | 0 | 0.2 | 0.2 |

Then the proposal for this 2LTE-1NR EN-DC is:

Table 4-3: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-42_n3 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n3 | 0.6 |

Table 4-4: ΔRIB,c

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-42_n3 | 1 | 0 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |

5.  Reference sensitivity exceptions

As mentioned in section 3, REFSENS exceptions due to IMD4 are expected. Based on CA_1-3-42, the following values are proposed.

Table 5-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_1A-42A_n3A | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n3 | 1782.5 | 5 | 25 | 1877.5 | N/A | FDD | N/A |
|  | 42 | 3425 | 5 | 25 | 3425 | 13.0 | TDD | IMD4 |

6.  Conclusion

This paper proposes study results of EN-DC 1-42_n3.

TP is shown in Annex.

7.  Reference

[1]  RP-201369  New WID on Dual Connectivity (DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL), Huawei

Annex.   Proposed text for TR37.717-21-11:

[Unchanged Parts Skipped]

5.x	DC_1-42_n3

### 5.x.1	Configurations for DC_1-42_n3

Table 5.x.1-1: Inter-band EN-DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-42A_n3A | DC_1A_n3A DC_42A_n3A |
| DC_1A-42C_n3A | DC_1A_n3A DC_42A_n3A DC_42C_n3A |

### 5.x.2	Co-existence studies

When Uplink EN-DC configuration is DC_1A_n3A, IMD4 of (B1 - n3) will fall into Rx band of Band 42.

### 5.x.3	∆TIB and ∆RIB values

The following relaxation values are proposed:

Table 5.x.3-1: ΔTIB,c

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-42_n3 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | n3 | 0.6 |

Table 5.x.3-2: ΔRIB

| Inter-band EN-DC configuration | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-42_n3 | 1 | 0 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |

### 5.x.4	Reference sensitivity exceptions

As mentioned above, IMD4 of B1 and n3 to Band42 Rx needs to be addressed for REFSENS relaxation. The following values are proposed:

Table 5.x.4-1: Reference sensitivity exceptions due to dual uplink operation for EN-DC in NR FR1 (three bands)

| NR or E-UTRA Band / Channel bandwidth / NRB / MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| DC_1A-42A_n3A | 1 | 1922.5 | 5 | 25 | 2112.5 | N/A | FDD | N/A |
|  | n3 | 1782.5 | 5 | 25 | 1877.5 | N/A | FDD | N/A |
|  | 42 | 3425 | 5 | 25 | 3425 | 13.0 | TDD | IMD4 |

[Unchanged Parts Skipped]