3GPP TSG-RAN WG4 Meeting # 97-e                	R4-2014938

Electronic Meeting, 2-13 Nov., 2020

Source: 	Nokia, Nokia Shanghai Bell

Title: 	2-step RACH BS demodulation simulation results

Agenda item: 	7.18.3

Document for: 	Discussion

Introduction

This paper presents simulation results for 2-step RACH BS demodulation considering the simulation assumptions agreed in [1].

Results

This section presents simulation results for MsgA PUSCH considering the simulation parameters on Table 1. The results are presented in Table 2 with and without impairment for Medium level cycling values.

Table 1 Simulation parameters for comparison between 1+1 and 1+1+1 DM-RS in FR1

| Parameter | Values for FR1 | Values for FR2 |
| --- | --- | --- |
| Waveform | CP-OFDM | CP-OFDM |
| Power offset between preamble and MsgA | Free choice | Free choice |
| Subcarrier spacing for PUSCH | 15kHz, 30kHz | 60 kHz, 120 kHz |
| PUSCH Mapping Type | Type A and Type B | Type B |
| MCS level | 1 | 2 |
| Number of symbols | 14 | 10 |
| TBS | 72 | 72 bits |
| DM-RS | 1+1+1 and 1+1 | 1+1 |
| Antenna configuration | 1T2R | 1T2R |
| Propagation channel | TDLC-300-100 | TDLA-30-300 |
| TO values | Medium level cycling values: 15k SCS: [0:0.4:2], 30k SCS: [0:0.2:1] | Medium level cycling values: 60k SCS: [0:0.1,0.5], 120 SCS: [0:0.05,0.25] |
| Test metric | SNR at BLER=0.01 and SNR at BLER=0.1 | SNR at BLER=0.01 and SNR at BLER=0.1 |
| Re-transmission | No retransmission considered | No retransmission considered |

Table 2 MsgA PUSCH simulation results

| SCS | DM-RS | Mapping Type | No impairment |  | With impairment |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BLER=0.1 | BLER=0.01 | BLER=0.1 | BLER=0.01 |
| 15 | 1+1 | A | -0.15 | 4.59 | 2.35 | 7.09 |
| 15 | 1+1 | B | -0.12 | 4.44 | 2.38 | 6.94 |
| 15 | 1+1+1 | A | -0.50 | 4.20 | 2 | 6.7 |
| 15 | 1+1+1 | B | -0.46 | 4.13 | 2.04 | 6.63 |
| 30 | 1+1 | A | -0.25 | 4.19 | 2.25 | 6.69 |
| 30 | 1+1 | B | -0.25 | 4.21 | 2.25 | 6.71 |
| 30 | 1+1+1 | A | -0.64 | 3.79 | 1.86 | 6.29 |
| 30 | 1+1+1 | B | -0.64 | 3.78 | 1.86 | 6.28 |
| 60 | 1+1 | B | 0.58 | 6.25 | 3.08 | 8.75 |
| 120 | 1+1 | B | -0.12 | 4.53 | 2.38 | 7.03 |

Conclusion

In this contribution, MsgA PUSCH simulation results are provided for scenarios in line with the simulation assumptions agreed in RAN4 #96-e [1].

References

- R4-2012705, “WF on BS demodulation requirements for 2-step RACH”, 3GPP TSG-RAN WG4 Meeting #96-e