3GPP TSG-RAN WG4 Meeting # 97-e                                                             R4-2015039

Electronic Meeting, 2-13 Nov., 2020

Source:	ZTE Corporation

Title:	On MSD for PC2 n41-n79 NR inter-band CA

Agenda Item:	10.18.1

Document for:	Approval

- 1	Introduction

- The PC2 NR inter-band CA n41-n79 is used as example PC2 TDD+TDD band combination in the revised WID[1]. For PC2 TDD-TDD NR inter-band CA, there are four cases for the power allocation for each bands, which is:

|  | UE power class | NR Carrier x power class | NR Carrier y power class |
| --- | --- | --- | --- |
| Case a | 26dBm | 23dBm | 23dBm |
| Case b | 26dBm | 23dBm | 26dBm |
| Case c | 26dBm | 26dBm | 23dBm |
| Case d | 26dBm | 26dBm | 26dBm |

- In this contribution, we give some proposals on the MSD for PC2 n41+n79 NR inter-band CA.

- 2	Discussion

According to PC3 n41-n79 inter-band CA analysis, there will be no harmonic and/or harmonic mixing issue as long as the frequency in Band 41 is above 2506. According to the spectrum situation in China and Japan, the lowest frequency to be used for this combination is 2515MHz. So there will be no harmonic and/or harmonic mixing issue. Also there are no MSD values caused by cross-band isolation defined for this combination.

However, comparing PC3 NR inter-band CA n41-n79, due to the output power for both band n41 and band n79 are increased up to 26dBm for PC2, and the band n41 filter rejection at band n79 Rx is about 25dB, therefore it needs to evaluate the cross band isolation MSD for PC2 NR inter-band CA n41-n79.

The evaluate parameters are assumed in table 1.

| PA output power for band B41/n79 (dBm) | 26 |
| --- | --- |
| Band n41 Tx PA output power at Bn79 Rx band(dBm/Hz) | -120 |
| Band n41 Tx Transceiver noise floor at PA output (dBm/Hz) | -130 |
| Band n79 Tx PA output power at Bn41 Rx band(dBm/Hz) | -160 |
| Band n79 Tx Transceiver noise floor at PA output (dBm/Hz) | -145 |
| Rx IIP2(dB) | 50 |
| Band n41 filter rejection @ band n79 Rx (dB) | 25 |
| Band n79 filter rejection @ band n41 Rx (dB) | 35 |
| Diplexer isolation (dB) | 15 |
| Antenna isolation (dB) | 10 |

With the assumption in table 1, the MSD values for 23dBm (n41)+26dBm(n79), 26dBm (n41)+23dBm(n79) and  26dBm (n41)+26dBm(n79) are proposed in table 2.

Table 2: Reference sensitivity exceptions (MSD) due to cross band isolation for PC2 NR CA n41-n79

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n41 | n79 |  |  |  |  |  |  | 3.4 | 3.4 | 3.4 |  | 3.4 |  | 3.4 |
| n79 | n41 |  | 3.5 | 3.3 | 3.2 |  | 3.0 | 2.9 | 2.8 | 2.7 |  | 2.6 | 2.5 | 2.5 |

- 3	Conclusion

- In this paper,  we give some discussion on the cross band isolation MSD for PC2 NR  inter-band CA n41-n79. For the three cases, i.e. 23dBm+26dBm, 26dBm+23dBm and 26dBm +26dBm, the MSD values are proposed:

Table 2: Reference sensitivity exceptions (MSD) due to cross band isolation for PC2 NR CA n41-n79

| NR Band / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n41 | n79 |  |  |  |  |  |  | 3.4 | 3.4 | 3.4 |  | 3.4 |  | 3.4 |
| n79 | n41 |  | 3.5 | 3.3 | 3.2 |  | 3.0 | 2.9 | 2.8 | 2.7 |  | 2.6 | 2.5 | 2.5 |

- 4	Reference

[1] RP-201584 Revised WID: SAR schemes for UE power class 2 (PC2) for NR inter-band Carrier Aggregation and supplemental uplink (SUL) configurations with 2 bands UL, China Telecom