3GPP TSG-RAN WG4 Meeting #97-e				  											R4-2015229 Online, 2nd Nov. 2020 – 13th Nov. 2020

Source: 	Nokia

Title: 		TP for 37.717-21-11 to introduce DC_2A-7A_n7A

Agenda Item: 			10.4.2 [DC_R17_2BLTE_1BNR_3DL2UL-Core]

Document for:	Approval

# 1	Introduction

This contribution is a TP for TR 37.717-21-11 to introduce DC_2A-7A_n7A.

# 2	Text Proposal

*********************** Start of the TP ***************************************************

## 5.x	DC_2-7_n7

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC Configuration | Uplink DC configuration |
| --- | --- |
| DC_2A-7A_n7A | DC_2A_n7A DC_7A_n7A2 |
| NOTE 2: Only single switched UL is supported |  |

### 5.x.2	Co-existence studies

Co-existence studies have been performed for lower order combinations. No further analysis is needed.

### 5.x.3	∆TIB and ∆RIB values

Table 5.x.3-1: ΔTIB,c

| Inter-band DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-7_n7 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | n7 | 0.5 |

Table 5.x.3-2: ΔRIB

| Inter-band DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-7_n7 | 2 | 0 |
|  | 7 | 0 |
|  | n7 | 0 |

### 5.x.4	Reference sensitivity exceptions

No further REFSENS exceptions needed.

*********************** End of the TP ***************************************************

# 3	References