3GPP TSG-RAN WG4 Meeting # 97-e	R4-2015381

# Electronic meeting, 2 – 13 November 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | 0237 | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.141-2 clarification on PN23 sequence generation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-10-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR introduces clarification to PN23 sequence generation in data content section for NR test models describes in [1], and clarify whether the same PN23 sequence is used for all PDCCH/PDSCH or individual PN23 sequence is used for each PDCCH/PDSCH in TMs with multi-users. Also clarification for TDD case is added. [1] R4-2015378 On PN23 sequence generation for data content for NR test models, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification added to PN23 sequence generation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification will be unclear and ambigous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.9.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.141-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

#### 4.9.2.3	Data content of physical channels and signals for NR-FR2-TM

Randomisation of the data content is obtained by utilizing a PN sequence generator and the length-31 Gold sequence scrambling of TS 38.211 [20], clause 5.2.1 which is invoked by all physical channels prior to modulation and mapping to the RE grid.

Initialization of the scrambler and RE-mappers as defined in TS 38.211 [20] use the following additional parameters:

-	NIDcell=1 for the lowest configured carrier, NIDcell=2 for the 2nd lowest configured carrier, …  NIDcell=n for the nth configured carrier

-	Antenna ports starting with 2000 for PDCCH

-	q = 0 (single code word)

-	For NR-FR2-TM1.1 when used for TAE requirement of two-layers MIMO transmission

-	Rank 2, two layers, no precoding

-	Antenna ports starting with 1000 and 1001 for PDSCH

Otherwise

-	Rank 1, single layer

##### 4.9.2.3.1	PDCCH

-	NsymbCORESET= 2

-	PDCCH modulation to be QPSK as described in TS 38.211 [20], clause 5.1.3.

-	For each slot the required amount of bits for all PDCCHs is as follows: 1(# of PDCCH) * 1(# of CCE per PDCCH) * 6(REG per CCE) * 9(data RE per REG) * 2(bits per RE) with these parameters according to the NR-FR2-TM definitions in clause 4.4.9.2.2.

-	Generate this amount of bits from the output of the PN23 sequence generator [28]. The PN sequence generator is initialized with a starting seed of “all ones” in the first allocated slot of each frame. The PN sequence is continuous over the slot boundaries.

-	1 CCE shall be according to TS 38.211 [20], clause 7.3.2. PDCCH using non-interleaved CCE-to-REG mapping. PDCCH occupies the first two symbols for 6 resource-element groups, where a resource element group equals one resource block during one OFDM symbol.

-	Perform PDCCH scrambling according to TS 38.211 [20], clause 7.3.2.3.

-	NID=NIDcell in DM-RS sequence generation in TS 38.211 [20], clause 7.4.1.3.

-	nRNTI=0 in scrambling sequence generation in TS 38.211 [20], clause 7.3.2.3.

-	Perform mapping to REs according to TS 38.211 [20], clause 7.3.2.5.

##### 4.9.2.3.2	PDSCH

-	Generate the required amount of bits from the output of the PN23 sequence generator [28]. The PN sequence generator is initialized with a starting seed of “all ones” in the first allocated slot of each frame. The PN sequence is continuous over the slot boundaries for slots where PDSCH or PDCCH is present. For PDSCH, the PN sequence is generated per user (nRNTI)..

-	NR-FR2-TMs utilize 1 or 2 user PDSCH transmissions distinguished by nRNTI. For each NR-FR2-TM, PRBs are mapped to user (nRNTI) as follows:

Table 4.9.2.3.2-1: Mapping of PRBs to  for NR-FR2-TM

| Test model |  | Number of users |
| --- | --- | --- |
| NR-FR2-TM1.1 | 2 for PRBs located in PRB#0-2 0 for remaining PRBs | 2 |
| NR-FR2-TM2 | 2 for all PRBs | 1 |
| NR-FR2-TM3.1 | 2 for PRBs located in PRB#0-2 0 for remaining PRBs | 2 |

-	Perform user specific scrambling according to TS 38.211 [20], clause 7.3.1.1.

-	nID=NIDcell

-	Perform modulation of the scrambled bits with the modulation scheme defined for each user according to TS 38.211 [20], clause7.3.1.2.

-	Perform mapping of the complex-valued symbols to layer according to TS 38.211 [20], clause 7.3.1.3.   Complex-valued modulation symbols  for codeword q shall be mapped onto the layers ,  where υ is equal to 1.

-	Perform PDSCH mapping type A according to TS 38.211 [20].

-	PDSCH resource allocation according to TS 38.214 [21] as following;

-	NR-FR2-TM1.1, NR-FR2-TM3.1: type 1 for PDSCH with nRNTI = 0, nRNTI = 2

-	NR-FR2-TM2: type 1 for PDSCH with nRNTI = 2

-	DM-RS sequence generation according to TS 38.211 [20], clause 7.4.1.1.1 where l is the OFDM symbol number within the slot with symbols indicated by table 4.9.2.2-3.

-	NIDnSCID=NIDcell

-	nSCID=0

-	DM-RS mapping according to TS 38.211 [20], clause 7.4.1.1.2 with parameters listed in table 4.9.2.2-3.

-	For NR-FR2-TM PT-RS sequence generation according to TS 38.211 [20], clause 7.4.1.2.1, with parameters listed in table 4.9.2.2-3.

-	For NR-FR2-TM PT-RS mapping according to TS 38.211 [20], clause 7.4.1.2.2, with parameters listed in table 4.9.2.2-3.

<Start of changes>