3GPP TSG-RAN WG4 Meeting #97-e								          R4-2015701

# Electronic Meeting, 2-13 Nov., 2020

Source: 	Huawei, HiSilicon

Title: 	Discussion on release independence

Agenda Item:	10.22.1

Document for:	Discussion

Introduction

In last meeting the release independence has been discussed and a WF was approved in [1]. 3 options are for further discussion.

  - Option 1: The support of 35 MHz and 45 MHz is from Rel-17 onwards.

  - Option 2: 35 MHz and 45 MHz is optional support from Rel-15.

  - Option 3: Release independence shall be discussed cases by case per band and bandwidths.

In the contribution, we provide considerations on the options.

Discussion

For the basket WID to add existing channel bandwidth, those channel bandwidth are automatically applied from Rel-15 onwards. If option 2 is adopted, current structure of 38.307 can be maintained. However, 35 MHz and 45 MHz are brand new channel bandwidths and the support of these new channel bandwidths need new analog and digital filters, hence there will be some additional complexity need to be taken into account. Option 1 cover such consideration, i.e. 35 MHz and 45 MHz is optional support from Rel-17.

Option 1: Table 5.1-3 of 38.307, NR UE channel bandwidth

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the UE channel bandwidth was introduced) |
| --- | --- | --- | --- |
| UE channel bandwidth {5 MHz, 10 MHz, 15 MHz,, 20 MHz, 25 MHz, 30 MHz, 40 MHz, 50 MHz, 60 MHz, 70 MHz, 80 MHz, 90 MHz, 100 MHz} | FDD, TDD, SUL, SDL | Rel-15 |  |
| UE channel bandwidth { 35 MHz, 45 MHz} | FDD, TDD, SUL, SDL | Rel-17 |  |

Option 3 propose to discuss the release independence case by case per band and bandwidths, considering the general rules that if the new channel bandwidth is smaller than the largest one already supported it can be introduced more easily. [3]

Option 3: Table 5.1-3 of 38.307, NR UE channel bandwidth

| Feature | Operating band | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the UE channel bandwidth was introduced) |
| --- | --- | --- | --- | --- |
| UE channel bandwidth {5 MHz, 10 MHz, 15 MHz,, 20 MHz, 25 MHz, 30 MHz, 40 MHz, 50 MHz, 60 MHz, 70 MHz, 80 MHz, 90 MHz, 100 MHz} | - | FDD, TDD, SUL, SDL | Rel-15 |  |
| UE channel bandwidth 35 MHz | n3, n7, n25, n66 | FDD | Rel-15 |  |
| UE channel bandwidth 35 MHz | n8, n71 | FDD | Rel-17 |  |
| UE channel bandwidth 45 MHz | n3, n25, n66 | FDD | Rel-17 |  |

Conclusions

In this contribution, we propose further consideration on the options in [1].

References

R4-2011791, WF on release independence and signalling for 35 MHz and 45 MHz, Huawei

R4-2011685, Draft CR to TS 38.307 Release independence support of new channel bandwidth from Rel-15, ZTE

R4-2010265, Spectrum Utilization for 35MHz and 45MHz Channels in FR1 and their Support, Skyworks Solutions, Inc.