3GPP TSG-RAN WG4 Meeting #97-e                                                                R4-2017211

Electronic Meeting, 2 Nov. – 13 Nov., 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR on multiple SCell activation with FR1+FR2 unknown cells in NR-DC Test Case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Enh-Perf |  |  |  |  |  | Date: |  |  | 2020-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The multiple SCell activation with FR1+FR2 unknown cells test case is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the new test case for multiple SCell activation with FR1+FR2 unknown cells. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.5.3.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is revision from R4-2014777. |  |  |  |  |  |  |  |  |

Start of Change

A.7.5.3.x	Multiple SCell Activation and deactivation of unknown SCell in both FR1 and FR2 in non-DRX

A.7.5.3.x.1	Test Purpose and Environment

The purpose of this test is to verify that the SCell activation and deactivation times are within the requirements stated in clause 8.3, when the SCell in FR1 and FR2 are both unknown by the UE at the time of activation. The test UE shall supports per-FR measurement gap capability.

The supported test configurations are as defined in clause A.7.5.3.x.1-1 below.

The general test parameters in MCG are the same as defined in Table A.6.5.3.1.1-2 except those described in the following clause.

The cell specific test parameters for MCG are described in Tables A.6.5.3.2.1-3.

The cell specific test parameters for SCG are described in Tables A.7.5.3.x.1-3. OTA related test parameters are the same as defined in Table A.7.5.3.x.1-4.

The test consists of three successive time periods, with duration of T1, T2 and T3, respectively. There are four NR carriers each with one cell. PCell(Cell 1) and SCell1 (Cell 2) are in FR1 inter-band. PSCell(Cell 3) and SCell2 (Cell 4) are in FR2 intra-band.

All cells have constant signal levels throughout the test. Before the test starts, the UE is connected to Cell 1 in MCG and Cell 3 in SCG, but is not aware of Cell2 and Cell 4. The UE is only monitoring the PCC(Cell 1) and PSCC(Cell 3). The UE shall be continuously scheduled in the PCell and PSCell throughout the whole test.

At the beginning of T1, the UE receives an RRC message by which the SCell1 (Cell 2) and SCell2 (Cell 4) become configured on radio channel 2. The test equipment sends two MAC messages for activation of the SCells in each CG.

The point in time at which the 1st MAC message is received at the UE antenna connector, in slot # denoted n1, defines the start of time period T2. The point in time at which the 2nd MAC message is received at the UE antenna connector, in slot # denoted n2 which is later than slot #n1.

The UE shall be able to report valid CSI in PCell for the activated SCell1 (Cell 2) at latest in slot THARQ+Tactivtion_time+TCSI_ReportingNR slot length, as defined in clause 8.3. The UE shall start reporting CSI in PCell in slot n1+1+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PCell interruption due to activation of SCell1 shall occur in the slot n1+1+THARQNR slot length to n1+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in section 8.2.

The UE shall be able to report valid CSI in PSCell for the activated SCell2 (Cell 4) at latest in slot THARQ+Tactivtion_time+TCSI_ReportingNR slot length, as defined in clause 8.3. The UE shall start reporting CSI in PSCell in slot n2+1+THARQ+3msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed. Any PSCell interruption due to activation of SCell2 shall occur in the slot n2+1+THARQNR slot length to n2+1+THARQ+3ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in section 8.2.

Time period T3 starts when one MAC message for deactivation of SCell1 in MCG, sent from the test equipment to the UE in a slot # denoted m1, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell1 in MCG in a slot m1+THARQ+3msNR slot length, as defined in clause 8.3, and the starting point of any PCell interruption due to the deactivation shall occur in the slot m1+1+THARQNR slot length to m1+1+THARQ+3msNR slot length, as defined in clause 8.3.

Another MAC message for deactivation of SCell2 in SCG, sent from the test equipment to the UE in a slot # denoted m2 later than slot #m1, is received at the UE antenna connector. The UE shall carry out deactivation of the SCell2 in SCG in a slot m2+THARQ+3msNR slot length, as defined in clause 8.3, and the starting point of any PSCell interruption due to the deactivation shall occur in the slot m2+1+THARQNR slot length to m2+1+THARQ+3msNR slot length, as defined in clause 8.3.

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell and PSCell during activation and deactivation of SCells, respectively.

The test equipment verifies the activation time by counting the slots from the time when the multiple SCells activation command are sent until a CSI report with other than CQI index 0 is received in both CGs.

The test equipment verifies the deactivation time by counting the slots from the time when the multiple SCells deactivation command are sent until CQI reporting for SCells are discontinued.

Table A.7.5.3.x.1-1: Supported test configurations for unknown FR1+FR2 SCell activation in non-DRX

| Configuration | Description |
| --- | --- |
| 1 | PCell and SCell1 in MCG: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode PSCell and SCell2 in SCG: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | PCell and SCell1 in MCG: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode PSCell and SCell2 in SCG: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 3 | PCell and SCell1 in MCG: NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode PSCell and SCell2 in SCG: 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations |  |

Table A.7.5.3.x.1-2: General test parameters for unknown FR1+FR2 SCell activation in non-DRX

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2,3,4 | Four NR radio channels are used for this test, cell 1, cell 2, cell 3 and cell 4 use RF channel 1,2,3 and 4, respectively. |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell1 |  | Cell 2 | Configured deactivated secondary cell in MCG on NR RF channel number 2 |
| Active PSCell |  | Cell 3 | Primary secondary cell on NR RF channel number 3. |
| Configured deactivated SCell2 |  | Cell 4 | Configured deactivated secondary cell in SCG on NR RF channel number 4 |
| Time alignment error between cell1 and cell2 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| Time alignment error between cell3 and cell4 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | ms | 100 | During this time the PCell and PSCell shall be known and the SCells configured, but not detected. |

Table A.7.5.3.x.1-3: FR2 serving cell specific test parameters for unknown FR1+FR2 SCell activation in non-DRX

| ParameterNote 5 |  |  | Unit | T1 |  | T2 |  | T3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 3 | Cell 4 | Cell 3 | Cell 4 | Cell 3 | Cell 4 |
| SSB ARFCN |  | Config 1,2,3 |  | Freq3 | Freq4 | Freq3 | Freq4 | Freq3 | Freq4 |
| Duplex mode |  |  |  | TDD |  | TDD |  | TDD |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| Downlink initial BWP Configuration |  |  |  | DLBWP.0.1 |  | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Downlink dedicated BWP Configuration |  |  |  | DLBWP.1.1 |  | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  | ULBWP.0.1 |  | ULBWP.0.1 |  |
| Uplink dedicated BWP configuration |  |  |  | ULBWP.1.1 |  | ULBWP.1.1 |  | ULBWP.1.1 |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  | TRS.2.1 TDD |  |
| TCI state |  |  |  | TCI.State.0 |  | TCI.State.0 |  | TCI.State.0 |  |
| BWchannel |  |  | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| PDSCH Reference measurement channel |  |  |  | SR.3.1 TDD | - | SR.3.1 TDD | - | SR.3.1 TDD | - |
| RMSI CORESET Parameters |  |  |  | CR.3.1 TDD | - | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Dedicated CORESET Parameters |  |  |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| OCNG Patterns |  |  |  | OP.1 | - | OP.1 |  |  |  |
| SSB Configuration |  |  |  | SSB.1 FR2 | - | SSB.1 FR2 |  |  |  |
| SMTC Configuration |  |  |  | SMTC.1 | - | SMTC.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | - | 0 |  |  |  |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |  |
| Propagation conditions |  |  | AWGN |  |  |  |  |  |  |
|  | Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: All parameters apply for configuration 1 and 2 |  |  |  |  |  |  |  |  |

Table A.7.5.3.x.1-4: OTA related test parameters for FR2 SCell

| Parameter |  | Unit | Cell 4 |  |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Angle of arrival configuration |  |  | NA | According to clause A.3.15.1 |  | NA |  |  |
| Note1 |  | dBm/15kHz | NA | -112 |  | NA Link only, see clause A.3.7A |  |  |
| Assumption for UE beams Note 7 |  |  |  | Rough |  |  |  |  |
| Note1 | Config 1,2 | dBm/SCS |  | -102.97 |  |  |  |  |
|  | Config 3, |  |  |  |  |  |  |  |
| SS-RSRPNote2 | Config 1,2 | dBm/SCS Note3 |  | -85.97 |  |  |  |  |
|  | Config 3 |  |  |  |  |  |  |  |
|  | Config 1,2,3 | dB |  | 17 |  |  |  |  |
|  |  | dB |  | 17 |  |  |  |  |
| IoNote2 | Config 1,2 | dBm/ChBWNote4,Note6 |  | -56.90 |  |  |  |  |
|  | Config 3 |  |  |  |  |  |  |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: ChBW is 94.04 MHz for Cell2, 9.36 MHz for Cell 3 in configurations 1,2,4,5, 38.1 MHz in configurations 3,6 Note 7: Information about types of UE beam is given in B.2.1.3 and does not imit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |

A.7.5.3.x.2	Test Requirements

The test requirements defined in clause A.6.5.3.1.2 shall apply to this test case, except Tactivation_time for SCell in MCG will be replaced with the value TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms and Tactivation_time for SCell in SCG will be replaced with the value max(Tuncertainty_MAC_multiple_scells + 5ms + TFineTiming, Tuncertainty_RRC_multiple_scells + TRRC_delay-THARQ) as defined in clause 8.3.7.

End of Change