3GPP TSG-RAN4 Meeting #98-e	R4-2100194

# Online, 25 Jan. – 5 Feb., 2021

Agenda item:	11.4.2.3

Source:	Apple

Title:	On PUCCH SCell activation and deactivation

Document for:	Discussion

# Introduction

In the WID of R17 RRM further enhancement [1], the working scope of PUCCH SCell activation/ deactivation is as below,

- PUCCH SCell activation/deactivation [RAN4]

  - Specify SCell Activation Delay Requirement for Deactivated PUCCH SCell (including valid TA and invalid TA)

  - Specify SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells (including valid TA and invalid TA)

  - Specify SCell Deactivation Delay Requirement for Activated PUCCH SCell

  - Specify SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells.

In last RAN4 meeting, the work plan has been agreed for this WID, as duplicated below,

  - 3GPP RAN4 #98e meeting (February, 2021, 1.5TU, Core part)

    - PUCCH SCell activation/deactivation [RAN4]

  - Initial discussion on SCell Activation/Deactivation Delay Requirement for single PUCCH SCell (including valid TA and invalid TA)

In this contribution we discuss the preliminary RRM requirements for PUCCH SCell activation/deactivation, including valid TA and invalid TA cases.

# Discussion on R17 working scope

As captured in the WID, the intention is to design the PUCCH SCell activation/deactivation requirements based on legacy R15 activation mechanism, and therefore we propose to preclude R16 direct SCell activation feature from this WI.

Proposal 1: in R17, RAN4 defines PUCCH SCell activation/deactivation requirements based on the “legacy R15 SCell activation mechanism” rather than “R16 direct SCell activation from DC/CA enhancement WI”.

# Activation requirement for PUCCH SCell

In R15 TS38.321, the UE behavior for PUCCH SCell activation was defined as:

The MAC entity shall for each configured SCell: 1> if an SCell Activation/Deactivation MAC CE is received activating the SCell: 2> activate the SCell according to the timing defined in TS 38.213 [6]; i.e. apply normal SCell operation including: 3> SRS transmissions on the SCell; 3> CSI reporting for the SCell; 3> PDCCH monitoring on the SCell; 3> PDCCH monitoring for the SCell; 3> PUCCH transmissions on the SCell, if configured. 2> if the SCell was deactivated prior to receiving this SCell Activation/Deactivation MAC CE: 3> activate the DL BWP and UL BWP indicated by firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id respectively;

In order for transmission PUCCH on the target SCell being activated, the TA information shall be applied for the SCell during the SCell activation, which is same as LTE PUCCH SCell activation.

Observation: both valid and invalid TA cases shall be specified for PUCCH SCell activation requirement.

The PUCCH requirement in LTE has been specified as below (TS36.133),

In NR MAC design, the definition/purpose of timeAlignmentTimer is same as LTE, which has been specified as,

RRC configures the following parameters for the maintenance of UL time alignment: - timeAlignmentTimer (per TAG) which controls how long the MAC entity considers the Serving Cells belonging to the associated TAG to be uplink time aligned.

Thus, same as in LTE, the condition to determine if the TA of target PUCCH SCell(sTAG) is valid can also reply on the whether timeAlignmentTimer is running or not. If the TA is valid for PUCCH SCell at UE, the TA acquisition time is not needed, and the PUCCH SCell activation delay could be as same as normal SCell activation delay in TS38.133 section 8.3.2.

Proposal 2: A TA is considered to be valid provided that the TimeAlignmentTimer associated with the TAG containing the PUCCH SCell is running.

Proposal 3: When the TA associated with target PUCCH SCell is valid, this PUCCH SCell activation delay is as same as the normal SCell activation delay in TS38.133 section 8.3.2.

When the TA is not valid for PUCCH SCell, UE needs to acquire the TA info during the PUCCH activation. In LTE, additional delay for TA acquisition consists of three parts:

- LTE T1 is the delay uncertainty in acquiring the first available PRACH occasion in the PUCCH SCell. T1 is up to 25 subframes and the actual value of T1 shall depend upon the PRACH configuration used in the PUCCH SCell.

- LTE T2 is the delay for obtaining a valid TA command for the sTAG to which the SCell configured with PUCCH belongs. T2 is up to 13 subframes.

- LTE T3 is the delay for applying the received TA for uplink transmission. T3 is 6 subframes.

In NR PUCCH SCell activation with invalid TA, those three additional delay parts are still needed for TA acquisition. But the value for each part shall be revisited based on NR UE capability/configuration.

Proposal 4: The three additional delay parts (T1/T2/T3) in LTE PUCCH SCell activation with invalid TA could be reused for NR PUCCH SCell activation with invalid TA. However, the values for T1/T2/T3 might be revisited for NR PUCCH SCell activation.

In NR handover requirement, the uncertainty delay of RACH occasion is defined as,

So, the T1 for NR PUCCH SCell activation could be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

Proposal 5: In NR PUCCH SCell activation delay requirement with invalid TA, T1 is the delay uncertainty in acquiring the first available PRACH occasion in the PUCCH SCell. T1 is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

T2 in LTE is up to the RAR window position on time domain, and it’s up to 13ms since RAR window starts 3ms after the PRACH msg1 in LTE and the RAR window length is up to 10ms. In NR direct SCell activation requirement at HO, the T2 is also used for the time uncertainty of RAR/TA-command reception. Since in NR the RAR window is configurable and there is no fixed upper bound for RAR window length, so we could reuse the T2 definition from direct SCell activation requirement at HO.

Proposal 6: In NR PUCCH SCell activation delay requirement with invalid TA, T2 is the delay from slot n + (Tactivate_basic +T1)/NR slot length until UE has obtained a valid TA command for the target PUCCH SCell being activated. Tactivate_basic is the normal SCell activation delay in TS38.133 section 8.3.2. slot n is the slot when UE received PUCCH SCell activation MAC CE.

T3 in NR PUCCH SCell activation requirement is the delay for applying the received TA for uplink transmission on target PUCCH SCell being activated. Similar as in NR direct SCell activation at HO, T3 is greater than or equal to k+1 slot, where k is defined in clause 4.2 in TS 38.213.

Proposal 7: In NR PUCCH SCell activation delay requirement with invalid TA, T3 is the delay for applying the received TA for uplink transmission on target PUCCH SCell being activated, and greater than or equal to k+1 slot, where k is defined in clause 4.2 in TS 38.213.

Regarding the applicability of PUCCH SCell activation requirement, the LTE conditions could also be used as baseline, as duplicated as below,

The above delay requirement (Tdelay_PUCCH SCell) shall apply provided that: - The UE has received a PDCCH order to initiate RA procedure on the PUCCH SCell within Tactivate_basic otherwise additional delay to activate the SCell is expected; and - The RA on PUCCH SCell is not interrupted by the RA on PCell otherwise additional delay to activate the SCell is expected; and - No SRS carrier based switching occurs during the SCell activation procedure otherwise the PUCCH SCell activation delay (Tdelay_PUCCH SCell) can be extended.

In NR PUCCH SCell activation requirement, it’s also necessary to assume that UE has received a PDCCH order to initiate RA procedure on the PUCCH SCell within Tactivate_basic otherwise additional delay to activate the SCell is expected.

In LTE applicability requirement, the RA colliding between PCell and target PUCCH SCell would also delay the completion of PUCCH SCell activation, however, dual RACH on PCell and SCell is not an issue to NR anymore with help of RAN1 power allocation design, and it’s also reflected in the NR PSCell addition requirement in TS38.133 section 8.9.2.

In LTE spec, only the possible interruption from SRS carrier based switching is considered to extend the PUCCH SCell activation delay, however, in NR there are many factors that can cause interruption to the PUCCH SCell activation, e.g. SRS carrier switching, BWP switching, CGI reading with autonomous gap, and etc. The target PUCCH SCell could be NR SCell in EN-DC, or in standalone NR carrier aggregation, or in NE-DC, or in NR-DC, and the interruption causes for different NR operation mode is summarized as,

- The causes of interruption in EN-DC is specified in TS38.133 section 8.2.1.1.

- The causes of interruption in NR SA is specified in TS38.133 section 8.2.2.1.

- The causes of interruption in NE-DC is specified in TS38.133 section 8.2.3.1.

- The causes of interruption in NR-DC is specified in TS38.133 section 8.2.4.1.

In addition, for a UE which does not support per-FR measurement gap, interruptions to the PCell and activated SCell may be caused by SCells on any frequency range. For a UE which supports per-FR gaps, interruptions to PCell and activated SCell may be caused by SCells on the same frequency range as the victim cell. Here, if UE can support per-FR MG, interruptions from the same FR may extend the PUCCH SCell activation delay; otherwise, if UE cannot support per-FR MG, all interruption may extend the PUCCH SCell activation delay. So, we propose that,

Proposal 8: The PUCCH SCell activation delay requirement shall apply provided that,

  - The UE has received a PDCCH order to initiate RA procedure on the PUCCH SCell within Tactivate_basic otherwise additional delay to activate the SCell is expected; and

  - No interruption occurs in same FR as the target PUCCH SCell during the SCell activation procedure if UE supports per-FR MG, otherwise the PUCCH SCell activation delay can be extended, and

  - No interruption occurs during the SCell activation procedure if UE does not support per-FR MG, otherwise the PUCCH SCell activation delay can be extended.

The above interruption is caused by factor defined in TS38.133 section 8.2.1.1 for EN-DC, in TS38.133 section 8.2.2.1 for NR SA, in TS38.133 section 8.2.3.1 for NE-DC and in TS38.133 section 8.2.4.1 for NR-DC.

# Deactivation requirement for PUCCH SCell

RAN2 has defined the followings in R15 TS38.321,

Since the SCell deactivation timer is not configured on the PUCCH SCell, so for PUCCH SCell we don’t need to define the timer based PUCCH SCell deactivation.

Proposal 9: only MAC CE based SCell deactivation requirement is specified for PUCCH activated SCell, i.e., no timer based PUCCH SCell deactivation is assumed.

For the PUCCH SCell deactivation, we did not see any difference from the normal SCell deactivation requirement.

Proposal 10: reuse MAC CE based normal SCell deactivation requirement to PUCCH SCell deactivation requirement.

# Interruption requirement for PUCCH SCell activation/deactivation

Like in LTE, the interruption caused by PUCCH SCell activation/deactivation is similar as that caused by normal SCell activation/deactivation, so we propose to reuse the interruption requirement of normal SCell activation/deactivation to the interruption requirement of PUCCH SCell activation/deactivation.

Proposal 11: reuse the interruption requirement of normal SCell activation/deactivation to the interruption requirement of PUCCH SCell activation/deactivation.

# TP for PUCCH SCell activation/deactivation delay requirement

Based on the previous analysis, the PUCCH SCell activation/deactivation delay requirement is proposed as below.

NR PUCCH SCell activation delay requirement: The requirements in this section shall apply for the UE configured with one downlink SCell and when PUCCH is configured for the SCell being activated. Upon receiving PUCCH SCell activation command in slot n, if the UE has a valid TA for transmitting on an SCell then the UE shall be able to transmit valid CSI report and apply actions related to the SCell activation command as specified in [TS38.321] for the SCell being activated on the PUCCH SCell no later than in slot n+Tactivate_basic: Where: - A TA is considered to be valid provided that the TimeAlignmentTimer [TS38.331] associated with the TAG containing the PUCCH SCell is running. - Tactivate_basic is the SCell activation delay defined in TS38.133 section 8.3.2. If the UE does not have a valid TA for transmitting on an SCell then the UE shall be capable to perform downlink actions related to the SCell activation command as specified in [TS38.321] for the SCell being activated on the PUCCH SCell no later than in slot n+Tactivate_basic and shall be capable to perform uplink actions related to the SCell activation command as specified in [TS38.321] for the SCell being activated on the PUCCH SCell no later than in slot n+Tdelay_PUCCH SCell and shall transmit valid CSI report for the SCell being activated on the PUCCH SCell no later than in slot n+Tdelay_PUCCH SCell, where: Tdelay_PUCCH SCell = Tactivate_basic + T1 + T2 + T3 Where: - T1 is the delay uncertainty in acquiring the first available PRACH occasion in the PUCCH SCell. T1 is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3]. - T2 is the delay from slot n + (Tactivate_basic +T1)/NR slot length until UE has obtained a valid TA command for the target PUCCH SCell being activated. - T3 is the delay for applying the received TA for uplink transmission on target PUCCH SCell being activated, and greater than or equal to k+1 slot, where k is defined in clause 4.2 in TS 38.213. The above delay requirement (Tdelay_PUCCH SCell) shall apply provided that: - The UE has received a PDCCH order to initiate RA procedure on the PUCCH SCell within Tactivate_basic otherwise additional delay to activate the SCell is expected; and - No interruption occurs in same FR as the target PUCCH SCell during the SCell activation procedure if UE supports per-FR MG, otherwise the PUCCH SCell activation delay can be extended, and - No interruption occurs during the SCell activation procedure if UE does not suppors per-FR MG, otherwise the PUCCH SCell activation delay can be extended. - The above interruption is caused by factor defined in TS38.133 section 8.2.1.1 for EN-DC, in TS38.133 section 8.2.2.1 for NR SA, in TS38.133 section 8.2.3.1 for NE-DC and in TS38.133 section 8.2.4.1 for NR-DC. NR PUCCH SCell deactivation delay requirement: The requirements in this section shall apply for the UE configured with one downlink SCell and when PUCCH is configured for the SCell being activated. The UE shall deactivate a SCell configured with PUCCH and meet the SCell deactivation delay requirements specified in TS38.133 section 8.3.3.

# Conclusion

In this contribution we discuss the preliminary RRM requirements for PUCCH SCell activation/deactivation, including valid TA and invalid TA cases.

Proposal 1: in R17, RAN4 defines PUCCH SCell activation/deactivation requirements based on the “legacy R15 SCell activation mechanism” rather than “R16 direct SCell activation from DC/CA enhancement WI”.

Observation: both valid and invalid TA cases shall be specified for PUCCH SCell activation requirement.

Proposal 2: A TA is considered to be valid provided that the TimeAlignmentTimer associated with the TAG containing the PUCCH SCell is running.

Proposal 3: When the TA associated with target PUCCH SCell is valid, this PUCCH SCell activation delay is as same as the normal SCell activation delay in TS38.133 section 8.3.2.

Proposal 4: The three additional delay parts (T1/T2/T3) in LTE PUCCH SCell activation with invalid TA could be reused for NR PUCCH SCell activation with invalid TA. However, the values for T1/T2/T3 might be revisited for NR PUCCH SCell activation.

Proposal 5: In NR PUCCH SCell activation delay requirement with invalid TA, T1 is the delay uncertainty in acquiring the first available PRACH occasion in the PUCCH SCell. T1 is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].

Proposal 6: In NR PUCCH SCell activation delay requirement with invalid TA, T2 is the delay from slot n + (Tactivate_basic +T1)/NR slot length until UE has obtained a valid TA command for the target PUCCH SCell being activated. Tactivate_basic is the normal SCell activation delay in TS38.133 section 8.3.2. slot n is the slot when UE received PUCCH SCell activation MAC CE.

Proposal 7: In NR PUCCH SCell activation delay requirement with invalid TA, T3 is the delay for applying the received TA for uplink transmission on target PUCCH SCell being activated, and greater than or equal to k+1 slot, where k is defined in clause 4.2 in TS 38.213.

Proposal 8: The PUCCH SCell activation delay requirement shall apply provided that,

  - The UE has received a PDCCH order to initiate RA procedure on the PUCCH SCell within Tactivate_basic otherwise additional delay to activate the SCell is expected; and

  - No interruption occurs in same FR as the target PUCCH SCell during the SCell activation procedure if UE supports per-FR MG, otherwise the PUCCH SCell activation delay can be extended, and

  - No interruption occurs during the SCell activation procedure if UE does not support per-FR MG, otherwise the PUCCH SCell activation delay can be extended.

The above interruption is caused by factor defined in TS38.133 section 8.2.1.1 for EN-DC, in TS38.133 section 8.2.2.1 for NR SA, in TS38.133 section 8.2.3.1 for NE-DC and in TS38.133 section 8.2.4.1 for NR-DC.

Proposal 9: only MAC CE based SCell deactivation requirement is specified for PUCCH activated SCell, i.e., no timer based PUCCH SCell deactivation is assumed.

Proposal 10: reuse MAC CE based normal SCell deactivation requirement to PUCCH SCell deactivation requirement.

Proposal 11: reuse the interruption requirement of normal SCell activation/deactivation to the interruption requirement of PUCCH SCell activation/deactivation.

# References

[1] RP-202874, Revised	WID of REL-17 NR RRM further enhancement, Apple, RAN #90