3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2100258

# Electronic Meeting, 17th– 28th August, 2020

Source:	Verizon

Title:	Requirement of band 46 reference sensitivity measurement for NR EN-DC band combinations

Agenda item:	9.3.2

Document for:	Discussion/Decision

# Introduction

In drafting TPs for DC_46_n2, DC_46_n5 and DC_46_n66 into TR 37.717-11-11 according to the request in [1], we realized a requirement of band 46 reference sensitivity measurement exclusion region is missing for the impact from licensed component carriers. This interrupts the related TPs works. However, a clear requirement for the LTE CA is defined in 36.101[2].

This contribution is to search for a RAN4 decision for how to utilize the band 46 in the EN-DC band combination in Rel-17.

# Reference

[1]	RP-202545, New WID on Rel-17 Dual Connectivity (DC) of 1 band LTE (1DL/1UL) and 1 NR band (1DL/1UL).

[2] TS36.101 (17.0.0): Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception

# Discussion

## Three EN-DC configurations are from Appendix below. The configurations of band 46 are restricted in the inter-band carrier aggregation and in downlink operation only. And its uplink will be paired with a licensed uplink operating band configured as Pcell carrier.

Table 3-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_46A_n2A | N/A | No |
| DC_46A_n5A | N/A | No |
| DC_46A_n66A | N/A | No |

### As indicated in the Appendix below, levels of harmonic products of the licensed operation bands will fall into the band 46 receiver frequency ranges although the band 46 is configured in downlink only. Demotion of Rx reference could be a sensitive impact for the individual RE falling in the band downlink operating bandwidth. Then, there is no such requirement defined for the band 46 in 38.101-3.

### Instead, RAN4 defined the requirement in term of ‘band 46 reference measurement exclusion regions’ for each licensed component carriers in the Table 7.3.1A-0eC of 36.101. Because of difference between LTE and NR definition, it is hard to re-claim the same requirement from 36.101 to the NR EN-DC band combination. For this, the three EN-DC carrier aggregations items listed in the table above are on pending.

For completion of three NE-DC requirements, a RAN4 decision is needed for whether or not a similar exclusion region should be required for the three NR EN-DC band combination as indicated in a table below (Table 3-2). And, what is the reference sensitivity measurement region in case if it is.

Proposal: RAN4 should make a decision if the band 46 reference measurement exclusion regions is needed for the licensed band n2, n5 and n66 in the EN-DC carrier aggregations.

Table 3-2: Band 46 Reference sensitivity measurement exclusion region in MHz.

| Licensed Component Carriers / E-UTRA Band / Harmonic order / Channel BW in UL |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Licensed Component Carriers | Harmonic order | 5MHz | 10MHz | 15MHz | 20MHz | 25MHz | 30MHz | 40MHz |
| n2 | 3 | ? | ? | ? | ? |  |  |  |
| n5 | 7 | ? | ? | ? | ? |  |  |  |
| n66 | 3 | ? | ? | ? | ? | ? | ? | ? |

# Conclusion

We discussed a possible missing requirement for the EN-DC carrier aggregation with restricted band 46 configuration. And, we have a proposal for the three EN-DC configurations.

Proposal: RAN4 should make a decision if the band 46 reference measurement exclusion regions is needed for the licensed band n2, n5 and n66 in the EN-DC carrier aggregations.

Apendex

3GPP TSG-RAN WG4 Meeting # xx-e 	R4-21xxxxx

# Electronic Meeting, dd/mm/2021

Source:

Title:	TP for TR 37.717-11-11 for DC_46_n2, DC_46_n5 and DC_46_n66

Agenda item:	x.x.x

Document for:

# Introduction

This contribution is a text proposal for TR 37.717-11-11 to include DC_46_n2, DC_46_n5 and DC_46_n66 according to the request in [1].

# Reference

[1]	RP-202545, New WID on Rel-17 Dual Connectivity (DC) of 1 band LTE (1DL/1UL) and 1 NR band (1DL/1UL).

# Text Proposal

<Start of Text Proposal>

## 6.1.x		DC_46_n2, DC_46_n5, DC_46_n66

### 6.1.x.1	Configuration for DC

Table 6.1.x.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_46A_n2A | N/A | No |
| DC_46A_n5A | N/A | No |
| DC_46A_n66A | N/A | No |

### 6.1.x.2	Maximum output power for DC

Table 6.1.x.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| EN-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_46A_n2A | 23 | +2/-3 |
| DC_46A_n5A | 23 | +2/-3 |
| DC_46A_n66A | 23 | +2/-3 |

### 6.1.x.3	Spurious emission band UE co-existence for DC

Table 6.1.x.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| EN-DC CA Configuration | Spurious emission |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_46A_n2A | N/A |  |  |  |  |
| DC_46A_n5A | N/A |  |  |  |  |
| DC_46A_n66A | N/A |  |  |  |  |

### 6.1.x.4	MSD analysis for DC

For study of 2UL/2DL, Table 6.1.x4-1 lists up to 7th order harmonics products for DC_46A-n2A

Table 6.1.x.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 5150 | 5925 | 1850 | 1910 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 10300 | 11850 | 3700 | 3820 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 15450 | 17775 | 5550 | 5730 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 20600 | 23700 | 7400 | 7640 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 25750 | 29625 | 9250 | 9550 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 36050 | 41475 | 12950 | 13370 |

For study of 2UL/2DL, Table 6.1.x4-2 lists up to 7th order harmonics products for DC_46A-n5A

Table 6.1.x.4-2: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 5150 | 5925 | 824 | 849 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 10300 | 11850 | 1648 | 1698 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 15450 | 17775 | 2472 | 2547 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 20600 | 23700 | 3296 | 3396 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 25750 | 29625 | 4120 | 4245 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 36050 | 41475 | 5768 | 5943 |

For study of 2UL/2DL, Table 6.1.x4-3 lists up to 7th order harmonics products for DC_46A-n66A

Table 6.1.x.4-3: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 5150 | 5925 | 1710 | 1780 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 10300 | 11850 | 3420 | 3560 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 15450 | 17775 | 5130 | 5340 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 20600 | 23700 | 6840 | 7120 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 25750 | 29625 | 8550 | 8900 |
| 7th harmonics frequency limits | 7*fx_low | 7*fx_high | 7* fy_low | 7* fy_high |
| 7th harmonics frequency limits (MHz) | 36050 | 41475 | 11970 | 12460 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |

In summary of studies, it could be seen,

- The 3rd order harmonics from band n2 and n66 UL may fall into band 46 DL frequency range

- The 7th order harmonics from band n5 may fall into band 46 DL frequency range

……

<End of Text Proposal>