# 3GPP TSG-RAN WG4 Meeting # 98-e 	R4-2100360

# Electronic Meeting, Jan. 25-Feb. 5, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.175 | CR | 0010 | rev |  | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.175 on IAB EMC performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Perf |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of performance requirements in IAB EMC specification is required to complete the EMC IAB standard. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of performance requirements in IAB EMC specification TS 38.175. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of performance requirements to proceed with IAB EMC testing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5; 6.1;6.2;9.2.3;9.3.3;9.4.3;9.5.2;9.5.3;9.7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Start of new section ------------------------------

## 4.5	IAB test configurations

The present clause defines the BS test configurations that shall be used for demonstrating conformance. A single NR carrier shall be used for testing of single-carrier capable IAB Node.

Single carrier configuration (SC) tests shall be performed using signal with narrowest supported NR IAB channel bandwidth with the smallest supported subcarrier spacing declared per operating band in [Test specification reference] clause X.

For other IAB Nodes, the test configurations in table 4.5-1 and table 4.5-2 shall be used. The NR IAB test configurations (X) are defined in [Test specification reference], clause X.

Table 4.5-1: Test configurations for IAB type 1-H

| IAB test case | IAB capable of multi-carrier and/or CA in a single band |  |  | BS capable of multi-band operation |  |
| --- | --- | --- | --- | --- | --- |
|  | Contiguous spectrum capable IAB | C and NC capable IAB with identical parameters | C and NC capable BS with different parameters | BS capable of multi-band operation with common connector | BS capable of multi-band operation with separate connector |
| Emission tests | TBD | TBD | TBD | TBD | TBD |
| Immunity tests | TBD | TBD | TBD | TBD | TBD |

Table 4.5-2: Test configurations for IAB type 1-O

| IAB test | Single band RIB |  |  | Multi-band RIB |
| --- | --- | --- | --- | --- |
|  | Contiguous spectrum capable IAB | C and NC capable IAB with identical parameters | C and NC capable IAB with different parameters |  |
| Emission tests | TBD | TBD | TBD | TBD |
| Immunity tests | TBD | TBD | TBD | TBD |

Table 4.5-3: Test configurations for IAB type 2-O

| IAB test case | Single band RIB |  |  |
| --- | --- | --- | --- |
|  | Contiguous spectrum capable IAB | C and NC capable IAB with identical parameters | C and NC capable IAB with different parameters |
| Emission tests | TBD | TBD | TBD |
| Immunity tests | TBD | TBD | TBD |

------------------------------ End of section ------------------------------

- ------------------------------ Start of new section ------------------------------

## 6.1	Performance criteria for continuous phenomena for IAB

The test should, where possible, be performed using a bearer with the characteristics of data rate and throughput defined in tables 6.1-1, 6.1-2, 6.1-3 and 6.1-4. If the test is not performed using one of these bearers (for example, of none of them are supported by the IAB node), the characteristics of the bearer used shall be recorded in the test report.

The throughput in tables 6.1-1, 6.1-2, 6.1-3 and 6.1-4 is stated relative to the maximum throughput of the FRC.

The IAB node uplink and downlink paths shall each meet the performance criteria defined in tables 6.1-1, 6.1-2, 6.1-3 and 6.1-4 during the test. If the uplink and downlink paths are evaluated as a one loop then the criteria is two times the throughput reduction shown in tables 6.1-1, 6.1-2, 6.1-3 and 6.1-4 (i.e. throughput > 90 % instead of throughput > 95 %). After each test case IAB Node shall operate as intended with no loss of user control function, stored data and the communication link shall be maintained.

Table 6.1-1: Performance criteria for continuous phenomena for IAB-DU Type 1-H

| NR IAB channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Bearer information data rate | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 in annex A.1 in TS 38.104 [22] | Throughput > 95 %, no loss of service |
| 10, 15 | 30 | G-FR1-A1-2 in annex A.1 in TS 38.104 [22] |  |
| 10, 15 | 60 | G-FR1-A1-3 in annex A.1 in TS 38.104 [22] |  |
| 20, 25, 30, 40, 50 | 15 | G-FR1-A1-4 in annex A.1 in TS 38.104 [22] |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 in annex A.1 in TS 38.104 [22] |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 in annex A.1 in TS 38.104 [22] |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

Table 6.1-2: Performance criteria for continuous phenomena for IAB-MT Type 1-H

| NR IAB channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Bearer information data rate | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 10, 15 | 30 | G-FR1-A1-22 in TS 38.174 [2] | Throughput > 95 %, no loss of service |
| 10, 15 | 60 | G-FR1-A1-23 in TS 38.174 [2] |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-25 in TS 38.174 [2] |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-26 in TS 38.174 [2] |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

Table 6.1-3: Performance criteria for continuous phenomena for IAB-DU Type 1-O

| NR IAB channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Bearer information data rate | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | G-FR2-A1-1 in annex A.1 in TS 38.104 [22] | Throughput > 95 %, no loss of service |
| 50 | 120 | G-FR2-A1-2 in annex A.1 in TS 38.104 [22] |  |
| 100, 200, 400 | 120 | G-FR2-A1-3 in annex A.1 in TS 38.104 [22] |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

Table 6.1-4: Performance criteria for continuous phenomena for IAB-MT Type 1-O

| NR IAB channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Bearer information data rate | Performance criteria (Note 1, Note 2) |
| --- | --- | --- | --- |
| 10, 15 | 30 | G-FR1-A1-22 in TS 38.174 [2] | Throughput > 95 %, no loss of service |
| 10, 15 | 60 | G-FR1-A1-23 in TS 38.174 [2] |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-25 in TS 38.174 [2] |  |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-26 in TS 38.174 [2] |  |
| NOTE 1: The performance criteria, throughput > 95 %, no loss of service, applies also if a bearer with another characteristics is used in the test. NOTE 2: The performance criteria, throughput > 90 %, no loss of service, applies instead if the uplink and downlink paths are evaluated as a one loop. |  |  |  |

## 6.2	Performance criteria for transient phenomena for IAB

At the conclusion of each exposure the EUT shall operate with no user noticeable loss of the communication link.

At the conclusion of the total test comprising the series of individual exposures the EUT shall operate as intended with no loss of user control functions or stored data, as declared by the manufacturer, and the communication link shall have been maintained. The channel bandwidth, sub-carrier spacing and bearer information data rate should be chosen based on manufacturers declaration and defined according to table 6.1-1, table 6.1-2, table 6.1-3 and table 6.1-4.

.

------------------------------ End of section ------------------------------

- ------------------------------ Start of new section ------------------------------

### 9.2.3	Performance criteria

IAB node:

The performance criteria of clause 6.1 shall apply.

Ancillary equipment:

The performance criteria of clause 6.3 shall apply.

------------------------------ End of section ------------------------------

- ------------------------------ Start of new section ------------------------------

### 9.3.3	Performance criteria

IAB node:

The performance criteria of clause 6.2 shall apply.

Ancillary equipment:

The performance criteria of clause 6.4 shall apply.

------------------------------ End of section ------------------------------

- ------------------------------ Start of new section ------------------------------

### 9.4.3	Performance criteria

IAB node:

The performance criteria of clause 6.2 shall apply.

Ancillary equipment:

The performance criteria of clause 6.4 shall apply.

------------------------------ End of section ------------------------------

- ------------------------------ Start of new section ------------------------------

### 9.5.2	Test method and level

The test method shall be in accordance with IEC 61000-4-6 [16]:

-	The test signal shall be amplitude modulated to a depth of 80 % by a sinusoidal audio signal of 1 kHz;

-	The stepped frequency increments shall be 50 kHz in the frequency range 150 kHz to 5 MHz and 1% frequency increment of the momentary frequency in the frequency range 5 MHz to 80 MHz;

-	The test level shall be severity level 2 as given in IEC 61000-4-6 [16] corresponding to 3 V rms, at a transfer impedance of 150 Ω;

-	The test shall be performed over the frequency range 150 kHz - 80 MHz;

-	The injection method to be used shall be selected according to the basic standard IEC 61000-4-6 [16];

-	Responses of stand-alone receivers or receivers which are part of transceivers occurring at discrete frequencies which are narrow band responses, shall be disregarded, see clause 4.3;

-	The frequencies of the immunity test signal selected and used during the test shall be recorded in the test report.

### 9.5.3	Performance criteria

IAB node:

The performance criteria of clause 6.1 shall apply.

Ancillary equipment:

The performance criteria of clause 6.3 shall apply.

------------------------------ End of section ------------------------------

- ------------------------------ Start of new section ------------------------------

### 9.7.3	Performance criteria

IAB node:

The performance criteria of clause 6.2 shall apply.

Ancillary equipment:

The performance criteria of clause 6.4 shall apply.

----------------------------- End of proposed text ------------------------------