3GPP TSG-RAN WG4 Meeting # 98-e                	R4-2100577

Electronic Meeting, Jan. 25-Feb. 5, 2021

Source: 	Nokia, Nokia Shanghai Bell

Title: 	NR-U PUCCH simulation results

Agenda item: 	7.1.7.4.3

Document for: 	Discussion

Introduction

This paper presents preliminary simulation results of PUCCH with enhanced formatted for operation on unlicensed bands [1].

Simulation assumptions

Table 1 Simulation assumptions for PUCCH format 0 analysis

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of symbol | 1 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 |
| Propagation conditions | TDLC300-100 low |
| Test metric | SNR at ProbPUCCH DTX→Ack bits ≤ 10-2 Prob(Ack missed detection) ≤ 10-2 |

Table 2 Simulation assumptions for PUCCH format 1 analysis

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of symbols | 14 |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | Neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| OCC index (timeDomainOCC) | 0 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz and 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 |
| Propagation conditions | TDLC300-100 low |
| Test metric | SNR at ProbPUCCH DTX→Ack bits ≤ 10-2 Prob(Ack missed detection) ≤ 10-2 SNR at ProbPUCCH DTX→Ack bits ≤ 10-2 ProbPUCCH NACK→ACK bits ≤ 10-3 |

Table 3 Simulation assumptions for PUCCH format 2 analysis

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Intra-slot frequency hopping | N/A |
| Number of symbols | 1 |
| The number of UCI information bits | 22 bits 4 bits |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Antenna configuration | 1T2R |
| Channel bandwidth | 20MHz |
| SCS | 15kHz 30kHz |
| Number of interlaces | 1 |
| Interlace index | 0 |
| Propagation conditions | TDLC300-100 low |
| OCC-Length-r16 | Not configured n2 |
| Test metric | SNR @ ProbUCI block BLER=10-2 |

Table 4 Simulation assumptions for PUCCH format 3 analysis

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| Intra-slot frequency hopping | N/A |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Additional DM-RS configuration | No additional DM-RS |
| Number of symbols and UCI information bits | 16 bits and 4 symbols 4 bits and 14 symbols |
| Channel bandwidth | 20MHz |
| SCS | 15kHz 30kHz |
| Antenna configuration | 1T2R |
| Number of interlaces | 1 |
| Interlace index | 0 |
| Propagation conditions | TDLC300-100 low |
| Index of OCC | n0 |
| Length of OCC | n1 (first priority) n2 (second priority) |
| Cyclic shift index for DMRS | 0 |
| Test metric | SNR @ UCI block BLER=10-2 |

Simulation results

Table 5 Simulation results for interlaced PUCCH format 0

| SCS | SNR (dB) at Prob(PUCCH DTX→Ack bits) ≤ 10^(−2) Prob(Ack missed detection) ≤ 10^(−2) | SNR@Prob(NACK→ACK) ≤ 10^(−3) |
| --- | --- | --- |
| 15kHz | -5.8 | -7.3 |
| 30kHz | -5.8 | -7.3 |

Table 6 Simulation results for interlaced PUCCH format 1

| SCS | SNR (dB) at Prob(PUCCH DTX→Ack bits) ≤ 10^(−2) Prob(Ack missed detection) ≤ 10^(−2) | SNR (dB) at Prob(PUCCH DTX→Ack bits) ≤ 10^(−2) Prob(PUCCH NACK→ACK bits) ≤ 10^(−3) |
| --- | --- | --- |
| 15kHz | -17.6 | -16.5 |
| 30kHz | -17.6 | -16.5 |

Table 7 Simulation results for interlaced PUCCH format 2

| SCS | Number of UCI information bits | OCC length | SNR (dB) @ BLER=10^(−2) |
| --- | --- | --- | --- |
| 15kHz | 22bit | 1 | -0.1 |
|  |  | 2 | 0.6 |
|  | 4bit | 1 | -5.5 |
|  |  | 2 | -5.1 |
| 30kHz | 22bit | 1 | 0 |
|  |  | 2 | 1 |
|  | 4bit | 1 | -5.6 |
|  |  | 2 | -5.1 |

Table 8 Simulation results for interlaced PUCCH format 3

| SCS | Number of symbols and UCI information bits | OCC length | SNR (dB) @ BLER=10^(−2) |
| --- | --- | --- | --- |
| 15kHz | 16bit, 4symbol | 1 | -7.8 |
|  |  | 2 | -7.6 |
|  | 4bit, 14symbol | 1 | -14.5 |
|  |  | 2 | -14.5 |
| 30kHz | 16bit, 4symbol | 1 | -7.7 |
|  |  | 2 | -7.5 |
|  | 4bit, 14symbol | 1 | -14.5 |
|  |  | 2 | -14.3 |

References

- R4-2017467, Way forward on PUCCH demodulation requirements, 3GPP TSG-RAN WG4 Meeting #97-e

- R4-2014942, PUCCH Demodulation performance requirements for operation in unlicensed bands, 3GPP TSG-RAN WG4 Meeting #97-e, Nokia

- R4-2100576, PUCCH Demodulation performance requirements for operation in unlicensed bands, 3GPP TSG-RAN WG4 Meeting #98-e, Nokia