3GPP TSG-RAN WG4 Meeting #98-e	R4-2100798

# Electronic Meeting, Jan. 25-Feb. 5, 2021

Agenda Item:	7.5.1

Source: 	MediaTek Inc.,

Title: 	MSD due to wider BW evaluation for DC_28_n5

Document for:	Approval

- 1. Introduction

In [1] [2], MSD due to introducing wider CBW was further discussed. DC_3_n1, DC_1_n40 had been characterized in [3] [4]. In this contribution, we provide further discussion for DC_28_n5.

- 2. Discussion

DC_28_n5

The B28 operation frequency range has been restricted in existing spec. Even with the restriction, it is 36MHz separation from B28A RX upper edge to n5 TX lower edge. Thus the CIM5 of n5 UL may still fall in the B28 DL channel. And since there’s no existing LB-LB quadplexer, we consider separate antenna for the LB-LB combination analysis.

Calculation assumptions are listed in Table 1:

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| B28 filter rejection at n5 UL range | 35 | dB |
| CIM5 of n5 UL relative to wanted level | -70 | dBc |
| Front-end loss | 4 | dB |
| PCB isolation | 67 | dB |
| PA RXBN noise | -125 | dBm/Hz |
| Thermal noise at n79 RX ANT port | -165.5 | dBm/Hz |
| Transceiver effective phase noise | -144 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 1 Receiver performance parameters for MSD analysis

| Direct Signal Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity |  |
| n5 TX power at antenna port (Primary path) | 20 | 20 | dBm |
| n5PA output noise power at B28 LNA input port | -101.2 | -101.2 | dBm/5MHz |
| n5 reciprocal mixing noise at B28 LNA input port | -102.4 | -102.4 | dBm/5MHz |
| Thermal noise at RX ant port | -162.5 |  | dBm/Hz |
| CIM5 noise at B28 LNA input port | -94 | -94 | dBm |
| Total noise level refer to B28 LNA input port | -90.6 | -90.6 |  |
| Combined |  |  |  |
| MSD (5MHz BW) | 7.9 |  | dB |

Table 2 Link analysis for Band 28 5MHz REFSENS calculation at fc = 785.5 MHz

With above analysis, we have MSD proposal for DC_28_n5:

Table 6.1.45.7-1: Reference sensitivity exceptions (MSD) due to cross band isolation for EN-DC in NR FR1

|  | E-UTRA or NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band |  | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| n5 |  | 28 | 4.5 | 3 | 2.2 | 0.3 |  |  |  |  |  |  |  |  |
| n5x |  | 28 | 7.9 | 7.3 | 7.3 | 7.3 |  |  |  |  |  |  |  |  |
| Note x: Applicable only for n5 UL BW = 20MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.45.7-2: Uplink configuration for reference sensitivity exceptions due to close proximity of bands for EN-DC in NR FR1

| E-UTRA or NR Band / SCS / Channel bandwidth of the affected DL band |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | SCS of UL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n5 | 28 |  | 25 | 25 | 20 | 20 |  |  |  |  |  |  |  |
| n5 | 28 | 15 | 6 | 6 | 6 | 6 |  |  |  |  |  |  |  |

Reference:

- R4-2011777 WF on handling new channel BW’s for EN-DC and NR CA band combinations with MSD, RAN4#96-e

- R4-2016839 WF on EN-DC large BW Cross Band Noise, RAN4#97-e

- R4-2009623, “CR for missing DC_1A_n40A Cross Band Noise MSD for large NR UL BW in 38.101-3”, RAN4#96-e, Qualcomm, August 2020.

- R4-2009620, “CR for missing DC_3A_n1A Cross Band Noise MSD for large NR UL BW in 38.101-3”, RAN4#96-e, Qualcomm, August 2020.