3GPP TSG-RAN WG4 Meeting #98-e                                            	R4-2100901

Electronic Meeting, Jan.25-Feb.5, 2021

Source:	Samsung

Title:	Discussion and simulation results for Rel-16 eType II codebook test cases

Agenda item:		7.9.4.3

Document for:	Discussion

- Introduction

In last RAN4 meeting, Rel-16 eType II codebook requirements with SU-MIMO set up was agreed with proper test parameters, test metric or test requirements, and test procedure to ensure enough performance gain over than Type I. Meanwhile, MU-MIMO scenario should postpone to Rel-17 WI for further study if necessary.

Interested companies are encouraged to provide the simulation results with multi test metrics, including following eType II PMI, random eType II and following Type I PMI, and random Type I PMI under the same condition.

In this contribution, we investigate the above test metrics with Rel-16 eType II codebook, and Type I codebook respectively and give our simulation results based on the agreed configuration parameters. Moreover, we also provide our views on Rel-16 e-Type according to the simulation results.

- Simulation results and discussion

  - Simulation results for Rel-16 eType II codebook

In following figures, we evaluated performance under following PMI and random PMI with Type I and Type II codebook constructions under XP Medium channel correlation and XP Custom Low correlation with 64QAM rank2 case.

- Simulation results for FDD

Fig.1 Throughput performance of FDD 16x2 XP Medium

Fig.2 Throughput performance of FDD 16x4 XP Medium

Fig.3 Throughput performance of FDD 16x2 XP Custom Low

Fig.4 Throughput performance of FDD 16x4 Custom Low

The SNR point at 70%,90%,and 95% of full throughput with different test metrics are summarized in the following tables respectively, in which the TP ratio with marked yellow is newly added in WF [1] ：

Table 2.1: SNR point and TP ratio at 70% full throughput

| SNR point(dB)/TP ratio | XP Med 16x2 | XP Med 16x4 | XP Custom Low 16x2 | XP Custom Low 16x4 |
| --- | --- | --- | --- | --- |
| Type I (following Type I PMI / random Type I PMI) | 9.4 dB/2.55 | 6.8 dB/1.93 | 7.9 dB/2.62 | 4.3 dB/2.30 |
| Enhanced Type II (following Enhanced Type II PMI / random Type I PMI) | 3.5 dB/18 | 0.9 dB/8.45 | 4.6 dB/8.74 | 1.8dB/3.19 |

Table 2.2: SNR point and TP ratio under 90% full throughput

| SNR point(dB)/TP ratio | XP Med 16x2 | XP Med 16x4 | XP Custom Low 16x2 | XP Custom Low 16x4 |
| --- | --- | --- | --- | --- |
| Type I (following Type I PMI / random Type I PMI) | 13.2dB/2.10 | 8.8dB/2.01 | 10.8dB/2.37 | 6.0dB/2.32 |
| Enhanced Type II (following Enhanced Type II PMI /random Type I PMI) | 6.3dB/6.5 | 2.6dB/6.05 | 7.8dB/3.46 | 3.8dB/3.19 |

Table 2.3: SNR point and TP ratio under 95% full throughput

| SNR point(dB)/TP ratio | XP Med 16x2 | XP Med 16x4 | XP Custom Low 16x2 | XP Custom Low 16x4 |
| --- | --- | --- | --- | --- |
| Type I (following Type I PMI / random Type I PMI) | 14.9dB/1.9 | 9.6dB/1.98 | 12.0dB/2.16 | 7.1dB/2.19 |
| Enhanced Type II (following Enhanced Type II PMI /random Type I PMI) | 7.7dB/4.41 | 3.4dB/5.76 | 9.6dB/2.73 | 4.8dB/2.89 |

- Simulation results for TDD

Fig.5 Throughput performance of TDD 16x2 XP Medium

Fig.6 Throughput performance of TDD 16x4 XP Medium

Fig.7 Throughput performance of TDD 16x2 Custom Low

Fig.8 Throughput performance of TDD 16x4 Custom Low

The SNR point at 70%,90%,and 95% of full throughput with different test metrics are summarized in the following tables respectively, in which the TP ratio with marked yellow is newly added in WF [1] ：

Table 2.4: SNR point and TP ratio under 70% full throughput

| SNR point(dB)/TP ratio | XP Med 16x2 | XP Med 16x4 | XP Custom Low 16x2 | XP Custom Low 16x4 |
| --- | --- | --- | --- | --- |
| Type I (following Type I PMI / random Type I PMI) | 10.0dB/1.86 | 6.9/1.70 | 8.0/1.98 | 5.4/1.74 |
| Enhanced Type II (following Enhanced Type II PMI /random Type I PMI) | 3.3dB/10.25 | 0.1dB/17.98 | 5.2dB/3.20 | 1.7dB/3.42 |

Table 2.5: SNR point and TP ratio under 90% full throughput

| SNR point(dB)/TP ratio | XP Med 16x2 | XP Med 16x4 | XP Custom Low 16x2 | XP Custom Low 16x4 |
| --- | --- | --- | --- | --- |
| Type I (following Type I PMI / random Type I PMI) | 13.2dB/1.83 | 9.1dB/1.76 | 11.1dB/1.83 | 7.6dB/1.78 |
| Enhanced Type II (following Enhanced Type II PMI /random Type I PMI) | 6.7dB/4.05 | 3.0dB/4.15 | 8.9dB/2.35 | 3.8dB/2.74 |

Table 2.6: SNR point and TP ratio under 95% full throughput

| SNR point(dB)/TP ratio | XP Med 16x2 | XP Med 16x4 | XP Custom Low 16x2 | XP Custom Low 16x4 |
| --- | --- | --- | --- | --- |
| Type I (following Type I PMI / random Type I PMI) | 14.7dB/1.68 | 9.9dB/1.73 | 12.0dB/1.79 | 8.6dB/1.71 |
| Enhanced Type II (following Enhanced Type II PMI /random Type I PMI) | 8.4dB/3.12 | 4.2dB/3.35 | 10.1dB/2.13 | 5.1dB/2.37 |

  - Simulation results summary

Observation 1-MIMO Correlation Selection: The performance gap between following eTypeII and Type I is more obvious under MIMO-Medium correlation compared with MIMO-Custom Low correlation.

Observation 2-Test point: The performance gap between following eTypeII and Type I is larger under 70% relative TP points compared to 90%, and 95% relative TP points.

Observation 3-Performance gap between eType II and Type I (XP Medium): There is obvious performance gap between eTypeII and Type I to discriminate UE behaviour under current test set-up under XP-Medium correlation

- The SNR gap is 7dB around between following PMI with eType II and following PMI with Type I at 70%, 90% and 95% relative TP points

- The performance under 90% relative TP point is more stable to introduce requirements.

- The relative TP ratio at 90% point:

- Type I (following Type I PMI /random Type I PMI): For FDD mode, TP gain is 2.0 around; TDD mode TP gain is 1.8 around;

- eType II (following eType II PMI /random Type I PMI): For FDD mode, TP gain is 6.0 around; TDD mode TP gain is 4.0 around

->Conclusion: there is enough performance gap between eType II and Type I with current agreed test set-up, MIMO correlation (XP Medium) and test metric (relative TP ratio with following eTypeII /random Type I); and it’s feasible to introducing proper test requirements to ensure UE reporting eTypeII properly i.e. UE reporting Type I codebook only will fail the test cases.

Proposal-1: Introduce eType II codebook PMI test cases with following test set-up

- SU-MIMO Set-up

- MIMO correlation: XP-Medium

- Test metric: following eType II PMI/random PMI with Type I codebook

- Test point: 90% Relative TP ratio with following PMI (slight preference), 70% acceptable

Proposal-2: Introducing following test requirements (with 90% relative TP point assumption):

- FDD mode: 2Rx, 3.0; 4Rx, 3.0

- TDD mode: 2Rx, 2.5; 4Rx, 2.5

- Conclusion

In this contribution, we provide simulation results for Rel-16 eType II, and Type I codebook based on the agreements in RAN4 #97e meeting. We compare different test metrics under various types of codebook. According to the above results, we observe that:

there is enough performance gap between eType II and Type I with current agreed test set-up, MIMO correlation (XP Medium) and test metric (relative TP ratio with following eTypeII /random Type I); and it’s feasible to introducing proper test requirements to ensure UE reporting eTypeII properly i.e. UE reporting Type I codebook only will fail the test cases.

Proposal-1: Introduce eType II codebook PMI test cases with following test set-up

- SU-MIMO Set-up

- MIMO correlation: XP-Medium

- Test metric: following eType II PMI/random PMI with Type I codebook

- Test point: 90% Relative TP ratio with following PMI (slight preference), 70% acceptable

Proposal-2: Introducing following test requirements (with 90% relative TP point assumption):

- FDD mode: 2Rx, 3.0; 4Rx, 3.0

- TDD mode: 2Rx, 2.5; 4Rx, 2.5

Reference

[1] R4-2017531,	“Way forward on PMI reporting requirements for NR eMIMO”, Ericsson, Samsung, 3GPP RAN4 #97-e meeting

[2] R4-2017563,	“Simulation assumptions for NR PMI reporting requirements for more than 8 Tx ports”, Ericsson, 3GPP RAN4 #97-e meeting

[3] R4-2014740, “Views and simulation results for Rel-16 Type II PMI test cases”, Samsung, 3GPP RAN4 #97-e meeting

Annex: Simulation configurations

The simulation configurations are proposed in [1], the main test parameters for SU-MIMO are set as below:

- Test metric: Further check results based on following candidate options during 2nd round - Option 1: Following PMI/Random PMI - Option 2: Following with eType II codebook / following PMI with Type I codebook - Option 3: Following eTypeII codebook / random PMI with Type I - Number of CSI-RS ports: - 16 ports with (N1, N2) = (4,2) and (O1,O2)=(4,4) - Number of PMI Sub-bands per CQI Sub-band - R = 1 - Codebook parameter configuration - paramCombination-r16: 6, with L =4, pν =1/2, β=1/2 - Sub-band Size: - 4 for FDD with 15kHz SCS, 10MHz CBW - 8 for TDD with 30kHz SCS, 40MHz CBW - Channel Model - TDLA30-5 - MIMO Correlation - XP Medium as Baseline - XP (custom) Low only can be considered if XP medium not workable - MCS and Rank - MCS 20 (64QAM Table), Rank 2 - Beam-Steering Approach - Enable two independent beam directions applying the beam for comparison both eType II and Type I cookbook simulation - Random PMI generation for W2 with eType II codebook - Select the main beam index (i18,l) from 2L beams randomly, l is the layer index - Fix the main beam with following amplitude (Table 5.2.2.2.5-2, Table 5.2.2.2.5-3 in spec) and phase coefficient indicators. Amplitude=1, Phase =0 - Select the remaining NZ coefficients from the table of amplitude and phase coefficient indicators over the (Beam x FD basis) grid until reaching K0 per layer - Note: The number of chosen coefficients per layer should be chosen randomly (number of ones in i17 bitmap); - Loop by the layer, and repeat the step1-3. Stop if 2K0 is reached across all layers - Precoder is constructed according to Table 5.2.2.2.5-5

In addition, the remaining parameters will be remained as Rel-15 Type II codebook simulation assumptions in [2].